# Mitigation

- INTRODUCTION

The preceding chapters of this Environmental Impact Statement (EIS) discuss the potential for significant adverse impacts to result from the proposed Memorial Sloan-Kettering Cancer Center (MSK)/The City University of New York (CUNY)-Hunter project. In accordance with the 2012 City Environmental Quality Review (CEQR) Technical Manual, where such potential impacts have been identified—in the areas transportation (i.e., traffic) and construction-period transportation—measures are examined to minimize or eliminate the anticipated impacts to the fullest extent practicable. These mitigation measures are discussed below.

principal conclusions

As discussed in Chapter 9, “Transportation,” traffic conditions were evaluated at 19 intersections for the weekday AM, midday, and PM peak hours, and the proposed project would result in significant adverse impacts at 11 different intersections, 8 intersections each during the weekday AM, midday, and PM peak hours. As summarized in Table 17-1, with the implementation of standard mitigation measures (including primarily signal timing changes and daylighting), the significant adverse traffic impacts identified above could be fully mitigated during all three analysis peak hours, with the exception of those at the York Avenue and East 79th Street intersection.

| Table 17-1 Summary of Significant Adverse Traffic Impacts |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Intersection |  | AM Peak Hour |  | Midday Peak Hour |  | PM Peak Hour |  |
| EB/WB Street | NB/SB Street | Significant Impacts | Mit | Significant Impacts | Mit | Significant Impacts | Mit |
| East 79th St | York Ave | EB-LTR | No | EB-LTR | No | EB-LTR | No |
|  |  | NB-LTR | No | NB-LTR | No |  |  |
|  |  |  |  |  |  | NB-TR | No |
| East 75th St | York Ave |  |  | NB-LTR | Yes |  |  |
| East 74th St | York Ave | EB-LTR | Yes | EB-LTR | Yes | EB-LTR | Yes |
|  |  |  |  | WB-LR | Yes | WB-LR | Yes |
| East 73rd St | York Ave |  |  |  |  | WB-LTR | Yes |
|  |  | NB-LTR | Yes | NB-LTR | Yes | NB-LTR | Yes |
|  |  | SB-DefL | Yes |  |  | SB-DefL | Yes |
|  |  |  |  | SB-LTR | Yes |  |  |
|  |  | SB-TR | Yes |  |  | SB-TR | Yes |
| East 72nd St | York Ave | EB-DefL | Yes | EB-DefL | Yes | EB-DefL | Yes |
|  |  | NB-LTR | Yes | NB-LTR | Yes | NB-LTR | Yes |
| East 71st St | York Ave | NB-LTR | Yes |  |  |  |  |
| East 66th St | York Ave |  |  | NB-LTR | Yes |  |  |
|  |  |  |  |  |  | SB-LTR | Yes |
| East 65th St | York Ave | EB-LR | Yes | EB-LR | Yes | EB-LR | Yes |
| East 61st St | York Ave | WB-R | Yes |  |  |  |  |
| East 72nd St | First Ave |  |  |  |  | EB-DefL | Yes |
| East 65th St | First Ave | EB-LT | Yes | EB-LT | Yes | EB-LT | Yes |
| Notes: EB = Eastbound; WB = Westbound; NB = Northbound; SB = Southbound; L = Left Turn; T = Through; R = Right Turn; and MIT = Mitigation Provided |  |  |  |  |  |  |  |

With respect to construction, the proposed project could would result in significant adverse construction traffic impacts during the PM construction peak hour. These impacts could be mitigated using the same similar measures identified for the operational significant adverse traffic impacts, and likewise, traffic impacts during construction at the York Avenue and East 79th Street intersection would be unmitigated. Between the Draft and Final EIS, in coordination with NYCDOT, additional analysis of construction traffic will be prepared.

- transportation

TRAFFIC

As discussed in Chapter 9, “Transportation,” traffic conditions were evaluated at 19 intersections for the weekday AM, midday, and PM peak hours, and the proposed project, would result in significant adverse impacts at 11 different intersections, 8 intersections each during the weekday AM, midday, and PM peak hours. Table 17-2 summarizes the recommended mitigation measures that are subject to review and approval by the New York City Department of Transportation (NYCDOT).

Tables 17-3 to 17-5 compare the level of service (LOS) conditions for the 2019 No Build, Build, and Mitigation conditions for the weekday AM, midday, and PM peak hours, respectively. Provided below is a discussion of each intersection with significant adverse traffic impacts and its recommended mitigation.

York avenue and east 79th street

The significant adverse impacts at this intersection during the weekday AM, midday, and PM peak hours could be mitigated by prohibiting parking and signal timing adjustments. However, based on NYCDOT’s review, the proposed parking prohibition measures have been deemed infeasible and, therefore, the significant adverse impacts could not be mitigated. Additional mitigation measures will be were explored between the Draft and Final EIS to potentially mitigate the significant adverse impacts at this intersection. However, no other standard mitigation measures were determined to be feasible.

York avenue and east 75th street

The significant adverse impact at the northbound approach of this intersection during the weekday midday peak hour could be fully mitigated by shifting 1 second of green time from the westbound phase to the northbound/southbound phase.

York avenue and east 74th street

The significant adverse impact at the eastbound approach of this intersection during the weekday AM peak hour could be fully mitigated by shifting 2 second of green time from the northbound/southbound phase to the eastbound/westbound phase.

The significant adverse impacts at the eastbound and westbound approaches of this intersection during the weekday midday and PM peak hours could be fully mitigated by installing a No Standing 10AM – 7PM Monday through Friday sign on the north side of East 74th Street on the westbound approach for approximately 100 feet from the intersection and shifting 4 seconds of green time from the northbound/southbound phase to the eastbound/westbound phase.

The daylighting of the north curb of the westbound approach would prohibit curbside loading/unloading activities during the weekday midday and PM peak hours.

| Table 17-2 Recommended Mitigation Measures |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intersection | Weekday AM |  |  |  | Weekday Midday |  |  |  | Weekday PM |  |  |  |
| York Avenue and East 79th Street | Unmitigated. |  |  |  | Unmitigated. |  |  |  | Unmitigated. |  |  |  |
| York Avenue and East 75th Street | No significant adverse impact |  |  |  | Shift 1 second of green time from the WB phase to the NB/SB phase. |  |  |  | No significant adverse impact |  |  |  |
| York Avenue and East 74th Street | Shift 2 seconds of green time from the NB/SB phase to the EB/WB phase. |  |  |  | 1) Install No Standing 10AM-7PM Monday-Friday sign on the north side of the WB approach for approximately 100 feet from the intersection; 2) Shift 4 seconds of green time from the NB/SB phase to the EB/WB phase. |  |  |  | 1) Install No Standing 10AM-7PM Monday-Friday sign on the north side of the WB approach for approximately 100 feet from the intersection; 2) Shift 4 seconds of green time from the NB/SB phase to the EB/WB phase. |  |  |  |
| York Avenue and East 73rd Street | 1) Prohibit NB left-turns from 7AM-7PM Monday-Friday; 2) Install No Standing 7AM-7PM Monday-Friday sign on the west side of the SB approach from East 73rd Street to East 74th Street to provide a SB right-turn lane; 3) Shift 2 seconds of green time from the LPI phase to the SB phase; 4) Shift 1 second of green time from the LPI phase to the NB/SB phase. |  |  |  | 1) Prohibit NB left-turns from 7AM-7PM Monday-Friday; 2) Install No Standing 7AM-7PM Monday-Friday sign on the west side of the SB approach from East 73rd Street to East 74th Street to provide a SB right-turn lane; 3) Shift 1 second of green time from the LPI phase to the NB/SB phase. |  |  |  | 1) Prohibit NB left-turns from 7AM-7PM Monday-Friday; 2) Install No Standing 7AM-7PM Monday-Friday sign on the west side of the SB approach from East 73rd Street to East 74th Street to provide a SB right-turn lane; 3) Shift 1 second of green time from the LPI phase to the WB phase; 4) Shift 3 seconds of green time from the LPI phase to the SB phase. |  |  |  |
| York Avenue and East 72nd Street | 1) Install No Standing 7AM-7PM Monday-Friday sign on the east side of the NB approach for approximately 100 feet from the intersection to provide a NB right-turn lane; 2) Shift the centerline on the EB approach 5 feet to the north to provide two (2) 11-foot moving lanes and one (1) 10-foot parking lane. |  |  |  | 1) Install No Standing 7AM-7PM Monday-Friday sign on the east side of the NB approach for approximately 100 feet from the intersection to provide a NB right-turn lane; 2) Shift the centerline on the EB approach 5 feet to the north to provide two (2) 11-foot moving lanes and one (1) 10-foot parking lane. |  |  |  | 1) Install No Standing 7AM-7PM Monday-Friday sign on the east side of the NB approach for approximately 100 feet from the intersection to provide a NB right-turn lane; 2) Shift the centerline on the EB approach 5 feet to the north to provide two (2) 11-foot moving lanes and one (1) 10-foot parking lane. |  |  |  |
| York Avenue and East 71st Street (1) | Modify signal phasing to provide an additional exclusive NB phase. |  |  |  | Modify signal phasing to provide an additional exclusive NB phase. |  |  |  | Modify signal phasing to provide an additional exclusive NB phase. |  |  |  |
|  | Phase | Green | Amber | Red | Phase | Green | Amber | Red | Phase | Green | Amber | Red |
|  | WB | 48 | 3 | 2 | WB | 48 | 3 | 2 | WB | 48 | 3 | 2 |
|  | NB/SB | 49 | 3 | 2 | NB/SB | 49 | 3 | 2 | NB/SB | 49 | 3 | 2 |
|  | NB | 8 | 3 | 2 | NB | 8 | 3 | 2 | NB | 8 | 3 | 2 |
|  | Cycle length = 120 seconds |  |  |  | Cycle length = 120 seconds |  |  |  | Cycle length = 120 seconds |  |  |  |
|  | Install No Standing Anytime sign on the west side of the SB approach for approximately 60 feet from the intersection to provide a SB right-turn lane. |  |  |  | Install No Standing Anytime sign on the west side of the SB approach for approximately 60 feet from the intersection to provide a SB right-turn lane. |  |  |  | Install No Standing Anytime sign on the west side of the SB approach for approximately 60 feet from the intersection to provide a SB right-turn lane. |  |  |  |
| York Avenue and East 66th Street | No significant adverse impact |  |  |  | Shift 3 seconds of green time from the WB phase to the NB phase. |  |  |  | Shift 1 second of green time from the WB phase to the NB/SB phase. |  |  |  |
| York Avenue and East 65th Street | Shift 2 seconds of green time from the NB/SB phase to the EB phase. |  |  |  | Shift 2 seconds of green time from the NB/SB phase to the EB phase. |  |  |  | Shift 1 second of green time from the NB/SB phase to the EB phase. |  |  |  |
| York Avenue and East 61st Street | Shift 1 second of green time from the NB/SB phase to the WB phase. |  |  |  | No significant adverse impact |  |  |  | No significant adverse impact |  |  |  |
| First Avenue and East 72nd Street (2) | Shift 1 second of green time from the NB phase to the EB/WB phase. |  |  |  | No significant adverse impact |  |  |  | Shift 2 seconds of green time from the NB phase to the EB/WB phase. |  |  |  |
| First Avenue and East 65th Street | Shift 1 second of green time from the NB phase to the EB phase. |  |  |  | Shift 1 second of green time from the NB phase to the EB phase. |  |  |  | Shift 1 second of green time from the NB phase to the EB phase. |  |  |  |
| Notes: EB = Eastbound; WB = Westbound; NB = Northbound; SB = Southbound; LPI = Lead Pedestrian Interval (1) Intersection of York Avenue and East 71st Street not impacted during the weekday midday and PM peak hours and was analyzed under mitigation conditions for verification purposes only. (2) Intersection of First Avenue and East 72nd Street not impacted during the weekday AM peak hour. However, signal timing adjusted to accommodate the proposed mitigation measures at the intersection of York Avenue and East 73rd Street. |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 17-3 2019 No Build, Build, and Mitigation Conditions Level of Service Analysis Weekday AM Peak Hour |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2019 No Build |  |  |  | 2019 Build |  |  |  |  | 2019 Mitigation |  |  |  |  |
|  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |  | Lane | v/c | Delay |  |  |
| Intersection | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |  | Group | Ratio | (sec) | LOS |  |
| York Avenue & East 79th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | - | - | - | - | - | - | - | - |  | Unmitigated |  |  |  |  |
|  | LTR | 1.06 | 95.5 | F | LTR | 1.12 | 114.3 | F | + |  |  |  |  |  |
|  | - | - | - | - | - | - | - | - |  |  |  |  |  |  |
| Westbound | LTR | 0.23 | 32.4 | C | LTR | 0.23 | 32.5 | C |  |  |  |  |  |  |
| Northbound | LTR | 1.13 | 107.2 | F | LTR | 1.17 | 121.1 | F | + |  |  |  |  |  |
| Southbound | TR | 0.93 | 51.7 | D | TR | 0.96 | 56.2 | E |  |  |  |  |  |  |
|  | Intersection |  | 80.8 | F | Intersection |  | 92.0 | F |  |  |  |  |  |  |
| York Avenue & East 74th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 0.56 | 36.0 | D | LTR | 0.76 | 46.6 | D | + | LTR | 0.75 | 44.3 | D |  |
| Westbound | LR | 0.06 | 25.6 | C | LR | 0.33 | 31.5 | C |  | LR | 0.31 | 29.5 | C |  |
| Northbound | TR | 0.57 | 18.8 | B | TR | 0.63 | 20.1 | C |  | TR | 0.65 | 21.7 | C |  |
| Southbound | LT | 0.74 | 23.4 | C | LT | 0.81 | 27.1 | C |  | LT | 0.84 | 29.9 | C |  |
|  | Intersection |  | 23.1 | C | Intersection |  | 27.1 | C |  | Intersection |  | 28.7 | C |  |
| York Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.20 | 45.9 | D | LTR | 0.24 | 46.9 | D |  | LTR | 0.24 | 46.9 | D |  |
| Northbound | LTR | 1.04 | 71.4 | E | LTR | 1.17 | 119.9 | F | + | TR | 1.05 | 72.0 | E |  |
| Southbound | DefL | 1.00 | 81.9 | F | DefL | 1.09 | 112.8 | F | + | DefL | 0.98 | 76.6 | E |  |
|  | - | - | - | - | - | - | - | - |  | T | 0.73 | 20.7 | C |  |
|  | TR | 1.06 | 75.4 | E | TR | 1.16 | 113.2 | F | + | - | - | - | - |  |
|  | - | - | - | - | - | - | - | - |  | R | 0.37 | 13.7 | B |  |
|  | Intersection |  | 73.7 | E | Intersection |  | 115.1 | F |  | Intersection |  | 53.1 | D |  |
| York Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | DefL | 1.05 | 109.1 | F | DefL | 1.13 | 136.1 | F | + | DefL | 1.05 | 109.9 | F |  |
|  | TR | 0.54 | 36.3 | D | TR | 0.56 | 37.1 | D |  | TR | 0.52 | 35.4 | C |  |
|  | R | 0.47 | 36.7 | D | R | 0.50 | 38.2 | D |  | R | 0.48 | 37.3 | C |  |
| Westbound | LTR | 0.46 | 33.1 | C | LTR | 0.47 | 33.3 | C |  | LTR | 0.47 | 33.3 | C |  |
| Northbound | - | - | - | - | - | - | - | - |  | LT | 1.00 | 55.1 | E |  |
|  | LTR | 1.09 | 83.5 | F | LTR | 1.22 | 137.0 | F | + | - | - | - | - |  |
|  | - | - | - | - | - | - | - | - |  | R | 0.26 | 15.0 | B |  |
| Southbound | LTR | 0.64 | 20.9 | C | LTR | 0.71 | 23.4 | C |  | LTR | 0.69 | 22.7 | C |  |
|  | Intersection |  | 59.1 | E | Intersection |  | 86.9 | F |  | Intersection |  | 46.1 | D |  |
| York Avenue & East 71st Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.83 | 39.3 | D | LTR | 0.85 | 40.6 | D |  | LTR | 0.857 | 40.62.8 | D |  |
| Northbound | LTR | 0.89 | 38.2 | D | LTR | 0.98 | 52.4 | D | + | LTR | 0.935 | 42.04.4 | D |  |
| Southbound | LTR | 0.61 | 23.4 | C | LTR | 0.62 | 23.6 | C |  | LTR | 0.78 | 37.0 | D |  |
|  |  |  |  |  |  |  |  |  |  | LT | 0.43 | 19.5 | B |  |
|  |  |  |  |  |  |  |  |  |  | R | 0.46 | 24.8 | C |  |
|  | Intersection |  | 34.3 | C | Intersection |  | 40.4 | D |  | Intersection |  | 35.541.8 | D |  |
| York Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LR | 1.03 | 97.6 | F | LR | 1.10 | 118.1 | F | + | LR | 1.04 | 97.8 | F |  |
| Northbound | T | 0.53 | 14.4 | B | T | 0.56 | 14.9 | B |  | T | 0.58 | 16.2 | B |  |
| Southbound | T | 0.45 | 13.6 | B | T | 0.45 | 13.6 | B |  | T | 0.47 | 14.8 | B |  |
|  | Intersection |  | 26.8 | C | Intersection |  | 30.5 | C |  | Intersection |  | 28.4 | C |  |
| York Avenue & East 61st Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | L | 0.23 | 24.1 | C | L | 0.23 | 24.1 | C |  | L | 0.23 | 23.4 | C |  |
|  | LTR | 0.75 | 34.6 | C | LTR | 0.77 | 35.4 | D |  | LTR | 0.75 | 34.1 | C |  |
|  | R | 0.80 | 43.1 | D | R | 0.88 | 50.5 | D | + | R | 0.86 | 47.6 | D |  |
| Northbound | LT | 0.84 | 29.7 | C | LT | 0.85 | 30.2 | C |  | LT | 0.86 | 31.7 | C |  |
| Southbound | TR | 0.47 | 19.8 | B | TR | 0.47 | 19.9 | B |  | TR | 0.48 | 20.6 | C |  |
|  | Intersection |  | 29.7 | C | Intersection |  | 31.1 | C |  | Intersection |  | 31.2 | C |  |
| First Avenue & East 72nd Street (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 0.98 | 57.1 | E | LT | 0.99 | 59.6 | E |  | LT | 0.97 | 54.8 | D |  |
| Westbound | TR | 0.45 | 21.9 | C | TR | 0.46 | 22.0 | C |  | TR | 0.47 | 21.6 | C |  |
| Northbound | L | 0.60 | 45.4 | D | L | 0.60 | 45.4 | D |  | L | 0.60 | 45.4 | D |  |
|  | TR | 0.76 | 20.3 | C | TR | 0.79 | 21.1 | C |  | R | 0.81 | 22.3 | C |  |
|  | Intersection |  | 29.1 | C | Intersection |  | 30.1 | C |  | Intersection |  | 29.7 | C |  |
| First Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 1.01 | 76.4 | E | LT | 1.07 | 92.0 | F | + | LT | 1.03 | 79.8 | E |  |
| Northbound | TR | 0.91 | 24.2 | C | TR | 0.93 | 26.3 | C |  | TR | 0.95 | 29.2 | C |  |
|  | Intersection |  | 31.8 | C | Intersection |  | 36.1 | D |  | Intersection |  | 36.8 | D |  |
| Notes: L = Left Turn, T = Through, R = Right Turn, DefL = Defacto Left Turn, LOS = Level of Service |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| + Denotes a significant adverse traffic impact |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) Intersection not impacted during the weekday AM peak hour. However, signal timing adjusted to accommodate the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| proposed mitigation measures at the intersection of York Avenue and East 73rd Street. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 17-4 2019 No Build, Build, and Mitigation Conditions Level of Service Analysis Weekday Midday Peak Hour |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2019 No Build |  |  |  | 2019 Build |  |  |  |  | 2019 Mitigation |  |  |  |  |
|  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |  | Lane | v/c | Delay |  |  |
| Intersection | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |  | Group | Ratio | (sec) | LOS |  |
| York Avenue & East 79th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | - | - | - | - | - | - | - | - |  | Unmitigated. |  |  |  |  |
|  | LTR | 0.86 | 54.1 | D | LTR | 0.91 | 60.1 | E | + |  |  |  |  |  |
|  | - | - | - | - | - | - | - | - |  |  |  |  |  |  |
| Westbound | LTR | 0.34 | 34.5 | C | LTR | 0.34 | 34.6 | C |  |  |  |  |  |  |
| Northbound | LTR | 0.97 | 54.7 | D | LTR | 1.07 | 81.8 | F | + |  |  |  |  |  |
| Southbound | TR | 0.79 | 39.5 | D | TR | 0.82 | 40.7 | D |  |  |  |  |  |  |
|  | Intersection |  | 47.9 | D | Intersection |  | 59.7 | E |  |  |  |  |  |  |
| York Avenue & East 75th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.10 | 30.2 | C | LTR | 0.10 | 30.2 | C |  | LTR | 0.11 | 31.0 | C |  |
| Northbound | LTR | 0.87 | 29.5 | C | LTR | 0.98 | 47.0 | D | + | LTR | 0.96 | 42.2 | D |  |
| Southbound | LTR | 0.67 | 18.0 | B | LTR | 0.71 | 19.3 | B |  | LTR | 0.70 | 18.4 | B |  |
|  | Intersection |  | 23.8 | C | Intersection |  | 33.0 | C |  | Intersection |  | 30.2 | C |  |
| York Avenue & East 74th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 0.59 | 36.7 | D | LTR | 0.84 | 52.9 | D | + | LTR | 0.77 | 43.2 | D |  |
| Westbound | LR | 0.09 | 26.0 | C | LR | 1.03 | 111.2 | F | + | LR | 0.65 | 39.5 | D |  |
| Northbound | TR | 0.56 | 18.6 | B | TR | 0.65 | 20.6 | C |  | TR | 0.69 | 24.2 | C |  |
| Southbound | LT | 0.57 | 18.9 | B | LT | 0.74 | 24.1 | C |  | LT | 0.80 | 29.7 | C |  |
|  | Intersection |  | 21.4 | C | Intersection |  | 35.0- | C |  | Intersection |  | 30.5 | C |  |
| York Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.40 | 51.3 | D | LTR | 0.50 | 55.5 | E |  | LTR | 0.50 | 55.5 | E |  |
| Northbound | LTR | 0.89 | 40.3 | D | LTR | 1.01 | 61.5 | E | + | TR | 0.92 | 43.2 | D |  |
| Southbound | - | - | - | - | - | - | - | - |  | DefL | 0.59 | 21.8 | C |  |
|  | LTR | 1.01 | 56.7 | E | LTR | 1.23 | 138.3 | F | + | - | - | - | - |  |
|  | - | - | - | - | - | - | - | - |  | T | 0.78 | 24.3 | C |  |
|  | - | - | - | - | - | - | - | - |  | R | 0.45 | 16.5 | B |  |
|  | Intersection |  | 48.7 | D | Intersection |  | 99.2 | F |  | Intersection |  | 33.5 | C |  |
| York Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | DefL | 0.73 | 50.7 | D | DefL | 0.81 | 59.9 | E | + | DefL | 0.75 | 52.6 | D |  |
|  | TR | 0.51 | 35.6 | D | TR | 0.53 | 36.4 | D |  | TR | 0.49 | 34.8 | C |  |
|  | R | 0.48 | 37.3 | D | R | 0.52 | 39.7 | D |  | R | 0.50 | 38.6 | D |  |
| Westbound | LTR | 0.48 | 34.1 | C | LTR | 0.54 | 35.9 | D |  | LTR | 0.54 | 35.9 | D |  |
| Northbound | - | - | - | - | - | - | - | - |  | LT | 0.90 | 36.2 | D |  |
|  | LTR | 0.96 | 46.3 | D | LTR | 1.06 | 72.2 | E | + | - | - | - | - |  |
|  | - | - | - | - | - | - | - | - |  | R | 0.15 | 13.4 | B |  |
| Southbound | LTR | 0.57 | 19.1 | B | LTR | 0.65 | 21.3 | C |  | LTR | 0.64 | 21.1 | C |  |
|  | Intersection |  | 36.2 | D | Intersection |  | 48.9 | D |  | Intersection |  | 32.3 | C |  |
| York Avenue & East 71st Street (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.68 | 32.7 | C | LTR | 0.68 | 32.9 | C |  | LTR | 0.6870 | 32.94.0 | C |  |
| Northbound | LTR | 0.64 | 24.2 | C | LTR | 0.70 | 26.1 | C |  | LTR | 0.689 | 25.3 | C |  |
| Southbound | LTR | 0.61 | 23.3 | C | LTR | 0.63 | 23.7 | C |  | LTR | 0.78 | 37.2 | D |  |
|  |  |  |  |  |  |  |  |  |  | LT | 0.49 | 20.5 | C |  |
|  |  |  |  |  |  |  |  |  |  | R | 0.30 | 19.8 | B |  |
|  | Intersection |  | 26.6 | C | Intersection |  | 27.4 | C |  | Intersection |  | 26.032.0 | C |  |
| York Avenue & East 66th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.04 | 29.2 | C | LTR | 0.04 | 29.2 | C |  | LTR | 0.04 | 31.4 | C |  |
| Northbound | LTR | 1.07 | 74.2 | E | LTR | 1.15 | 103.8 | F | + | LTR | 1.08 | 73.3 | E |  |
| Southbound | LTR | 0.82 | 31.6 | C | LTR | 0.86 | 33.9 | C |  | LTR | 0.86 | 33.9 | C |  |
|  | Intersection |  | 52.7 | D | Intersection |  | 69.2 | E |  | Intersection |  | 53.8 | D |  |
| York Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LR | 0.92 | 71.0 | E | LR | 0.98 | 83.4 | F | + | LR | 0.93 | 70.0 | E |  |
| Northbound | T | 0.61 | 16.2 | B | T | 0.64 | 17.0 | B |  | T | 0.66 | 18.5 | B |  |
| Southbound | T | 0.52 | 14.6 | B | T | 0.53 | 14.7 | B |  | T | 0.54 | 16.0 | B |  |
|  | Intersection |  | 23.7 | C | Intersection |  | 26.2 | C |  | Intersection |  | 25.3 | C |  |
| First Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 0.99 | 69.4 | E | LT | 1.03 | 80.2 | F | + | LT | 1.00 | 70.5 | E |  |
| Northbound | TR | 0.82 | 19.8 | B | TR | 0.84 | 20.5 | C |  | TR | 0.85 | 21.8 | C |  |
|  | Intersection |  | 27.9 | C | Intersection |  | 30.4 | C |  | Intersection |  | 29.9 | C |  |
| Notes: L = Left Turn, T = Through, R = Right Turn, DefL = Defacto Left Turn, LOS = Level of Service |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| + Denotes a significant adverse traffic impact (1) Intersection not impacted but analysis was conducted to incorporate signal phasing parking regulation changes proposed as mitigation measures in the weekday AM peak hour. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 17-5 2019 No Build, Build, and Mitigation Conditions Level of Service Analysis Weekday PM Peak Hour |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2019 No Build |  |  |  | 2019 Build |  |  |  |  | 2019 Mitigation |  |  |  |  |
|  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |  | Lane | v/c | Delay |  |  |
| Intersection | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |  | Group | Ratio | (sec) | LOS |  |
| York Avenue & East 79th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | - | - | - | - | - | - | - | - |  | Unmitigated. |  |  |  |  |
|  | LTR | 1.02 | 82.5 | F | LTR | 1.05 | 90.2 | F | + |  |  |  |  |  |
|  | - | - | - | - | - | - | - | - |  |  |  |  |  |  |
| Westbound | LTR | 0.42 | 36.1 | D | LTR | 0.42 | 36.2 | D |  |  |  |  |  |  |
| Northbound | - | - | - | - | DefL | 0.89 | 81.3 | F |  |  |  |  |  |  |
|  | LTR | 1.21 | 137.2 | F | - | - | - | - |  |  |  |  |  |  |
|  | - | - | - | - | - | - | - | - |  |  |  |  |  |  |
|  | - | - | - | - | TR | 1.39 | 217.5 | F | + |  |  |  |  |  |
|  | - | - | - | - | - | - | - | - |  |  |  |  |  |  |
| Southbound | TR | 0.92 | 50.5 | D | TR | 0.93 | 52.7 | D |  |  |  |  |  |  |
|  | Intersection |  | 87.5 | F | Intersection |  | 110.0 | F |  |  |  |  |  |  |
| York Avenue & East 74th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 0.59 | 36.9 | D | LTR | 0.81 | 51.1 | D | + | LTR | 0.76 | 43.7 | D |  |
| Westbound | LR | 0.09 | 26.1 | C | LR | 0.85 | 68.6 | E | + | LR | 0.53 | 34.0 | C |  |
| Northbound | TR | 0.55 | 18.3 | B | TR | 0.60 | 19.6 | B |  | TR | 0.64 | 22.8 | C |  |
| Southbound | LT | 0.71 | 22.3 | C | LT | 0.82 | 27.4 | C |  | LT | 0.89 | 35.2 | C |  |
|  | Intersection |  | 22.7 | C | Intersection |  | 30.7 | C |  | Intersection |  | 31.8 | C |  |
| York Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.43 | 52.3 | D | LTR | 0.56 | 58.4 | E | + | LTR | 0.53 | 55.7 | E |  |
| Northbound | LTR | 1.15 | 111.0 | F | LTR | 1.28 | 164.9 | F | + | TR | 1.12 | 100.5 | E |  |
| Southbound | DefL | 1.17 | 137.4 | F | DefL | 1.35 | 211.1 | F | + | DefL | 1.17 | 136.0 | F |  |
|  | - | - | - | - | - | - | - | - |  | T | 0.72 | 20.0- | B |  |
|  | TR | 1.00 | 58.1 | E | TR | 1.17 | 113.6 | F | + | - | - | - | - |  |
|  | - | - | - | - | - | - | - | - |  | R | 0.41 | 14.5 | B |  |
|  | Intersection |  | 95.1 | F | Intersection |  | 149.8 | F |  | Intersection |  | 75.6 | E |  |
| York Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | DefL | 0.72 | 49.0 | D | DefL | 0.82 | 60.5 | E | + | DefL | 0.76 | 52.6 | D |  |
|  | TR | 0.47 | 34.8 | C | TR | 0.50 | 35.6 | D |  | TR | 0.46 | 34.2 | C |  |
|  | R | 0.47 | 36.8 | D | R | 0.50 | 38.8 | D |  | R | 0.49 | 37.7 | D |  |
| Westbound | LTR | 0.31 | 29.7 | C | LTR | 0.42 | 32.3 | C |  | LTR | 0.42 | 32.3 | C |  |
| Northbound | - | - | - | - | - | - | - | - |  | LT | 1.05 | 67.4 | E |  |
|  | LTR | 1.10 | 86.0 | F | LTR | 1.17 | 113.8 | F | + | - | - | - | - |  |
|  | - | - | - | - | - | - | - | - |  | R | 0.13 | 13.1 | B |  |
| Southbound | LTR | 0.54 | 18.4 | B | LTR | 0.57 | 19.2 | B |  | LTR | 0.57 | 19.1 | B |  |
|  | Intersection |  | 55.5 | E | Intersection |  | 69.8 | E |  | Intersection |  | 45.8 | D |  |
| York Avenue & East 71st Street (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.72 | 34.1 | C | LTR | 0.73 | 34.6 | C |  | LTR | 0.735 | 34.65.9 | CD |  |
| Northbound | LTR | 0.77 | 28.9 | C | LTR | 0.82 | 31.7 | C |  | LTR | 0.801 | 30.25 | C |  |
| Southbound | LTR | 0.61 | 23.3 | C | LTR | 0.64 | 24.0 | C |  | LTR | 0.80 | 37.9 | D |  |
|  |  |  |  |  |  |  |  |  |  | LT | 0.51 | 20.9 | C |  |
|  |  |  |  |  |  |  |  |  |  | R | 0.26 | 19.0 | B |  |
|  | Intersection |  | 28.8 | C | Intersection |  | 30.1 | C |  | Intersection |  | 28.534.5 | C |  |
| York Avenue & East 66th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.03 | 29.1 | C | LTR | 0.03 | 29.1 | C |  | LTR | 0.03 | 29.8 | C |  |
| Northbound | DefL | 0.54 | 29.7 | C | DefL | 0.57 | 32.3 | C |  | DefL | 0.55 | 30.6 | C |  |
|  | TR | 0.43 | 13.3 | B | TR | 0.46 | 13.7 | B |  | TR | 0.46 | 13.1 | B |  |
| Southbound | LTR | 0.96 | 46.2 | D | LTR | 1.01 | 56.4 | E | + | LTR | 0.99 | 51.1 | D |  |
|  | Intersection |  | 33.6 | C | Intersection |  | 39.5 | D |  | Intersection |  | 36.2 | D |  |
| York Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LR | 1.10 | 118.3 | F | LR | 1.14 | 131.3 | F | + | LR | 1.11 | 119.7 | F |  |
| Northbound | T | 0.31 | 11.7 | B | T | 0.33 | 11.9 | B |  | T | 0.33 | 12.4 | B |  |
| Southbound | T | 0.60 | 16.1 | B | T | 0.61 | 16.3 | B |  | T | 0.62 | 17.0 | B |  |
|  | Intersection |  | 33.6 | C | Intersection |  | 36.2 | D |  | Intersection |  | 34.5 | C |  |

| Table 17-5 (cont’d) 2019 No Build, Build, and Mitigation Conditions Level of Service Analysis Weekday PM Peak Hour |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2019 No Build |  |  |  | 2019 Build |  |  |  |  | 2019 Mitigation |  |  |  |  |
|  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |  | Lane | v/c | Delay |  |  |
| Intersection | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |  | Group | Ratio | (sec) | LOS |  |
| First Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | - | - | - | - | DefL | 0.77 | 46.8 | D | + | DefL | 0.75 | 43.1 | D |  |
|  | LT | 0.73 | 30.5 | C | - | - | - | - |  | - | - | - | - |  |
|  | - | - | - | - | T | 0.62 | 27.8 | C |  | T | 0.59 | 25.3 | C |  |
| Westbound | TR | 0.42 | 21.4 | C | TR | 0.45 | 21.9 | C |  | TR | 0.46 | 20.8 | C |  |
| Northbound | L | 0.48 | 40.1 | D | L | 0.48 | 40.1 | D |  | L | 0.48 | 40.1 | D |  |
|  | TR | 0.76 | 20.1 | C | TR | 0.77 | 20.5 | C |  | TR | 0.81 | 22.9 | C |  |
|  | Intersection |  | 22.7 | C | Intersection |  | 23.6 | C |  | Intersection |  | 24.5 | C |  |
| First Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 1.10 | 101.8 | F | LT | 1.13 | 113.2 | F | + | LT | 1.09 | 98.6 | F |  |
| Northbound | TR | 0.82 | 19.8 | B | TR | 0.84 | 20.2 | C |  | TR | 0.85 | 21.6 | C |  |
|  | Intersection |  | 33.4 | C | Intersection |  | 35.9 | D |  | Intersection |  | 34.6 | C |  |
| Notes: L = Left Turn, T = Through, R = Right Turn, DefL = Defacto Left Turn, LOS = Level of Service |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| + Denotes a significant adverse traffic impact (1) Intersection not impacted but analysis was conducted to incorporate signal phasing parking regulation changes proposed as mitigation measures in the weekday AM peak hour. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

York avenue and east 73rd street

The significant adverse impacts at the northbound approach, southbound defacto left-turn, and southbound through/right-turn of this intersection during the weekday AM peak hour could be fully mitigated by prohibiting northbound left-turns on York Avenue (installing a No Left-Turns 7AM – 7PM Monday through Friday sign); prohibiting parking (installing a No Standing 7AM – 7PM Monday through Friday sign) on the west side of York Avenue on the southbound approach from East 73rd Street to East 74th Street; and shifting 2 seconds of green time from the lead pedestrian interval (LPI) phase to the southbound phase; and shifting 1 second of green time from the LPI to the northbound/southbound phase.

The significant adverse impacts at the northbound and southbound approaches of this intersection during the weekday midday peak hour could be fully mitigated by prohibiting northbound left-turns on York Avenue (installing a No Left-Turns 7AM – 7PM Monday through Friday sign); and prohibiting parking (installing a No Standing 7AM – 7PM Monday through Friday sign) on the west side of York Avenue on the southbound approach from East 73rd Street to East 74th Street; and shifting 1 second of green time from the LPI to the northbound/southbound phase.

The significant adverse impacts at the westbound approach, northbound approach, southbound defacto left-turn, and southbound through/right-turn of this intersection during the weekday PM peak hour could be fully mitigated by prohibiting northbound left-turns on York Avenue (installing a No Left-Turns 7AM – 7PM Monday through Friday sign); prohibiting parking (installing a No Standing 7AM – 7PM Monday through Friday sign) on the west side of York Avenue on the southbound approach from East 73rd Street to East 74th Street; shifting 1 second of green time from the LPI phase to the westbound phase; and shifting 3 seconds of green time from the LPI phase to the southbound phase.

The daylighting of the west curb of the southbound approach would prohibit parking at approximately 2 on-street parking spaces during the weekday AM, midday, and PM peak hours.

York avenue and east 72nd street

The significant adverse impacts at the eastbound defacto left-turn and the northbound approach of this intersection during the weekday AM, midday, and PM peak hours could be fully mitigated by prohibiting parking (installing a No Standing 7AM – 7PM Monday through Friday sign) on the east side of York Avenue on the northbound approach for approximately 100 feet from the intersection to provide a northbound right-turn lane and by shifting the centerline on the eastbound approach 5 feet to the north to provide two (2) 11-foot moving lanes and one (1) 10-foot parking lane.

The daylighting of the east curb of the northbound approach would prohibit parking at approximately 4 metered parking spaces during the weekday AM, midday, and PM peak hours.

York avenue and east 71st street

The significant adverse impact at the northbound approach of this intersection during the weekday AM peak hour could be fully mitigated by modifying the signal phasing to provide an additional exclusive northbound phase (see Table 17-2) prohibiting parking (installing a No Standing Anytime sign) on the west side of York Avenue on the southbound approach for approximately 60 feet from the intersection to provide a 10-foot southbound right-turn lane.

York avenue and east 66th street

The significant adverse impact at the northbound approach of this intersection during the weekday midday peak hour could be fully mitigated by shifting 3 seconds of green time from the westbound phase to the northbound phase.

The significant adverse impact at the southbound approach of this intersection during the weekday PM peak hour could be fully mitigated by shifting 1 second of green time from the westbound phase to the northbound/southbound phase.

York avenue and east 65th street

The significant adverse impact at the eastbound approach of this intersection during the weekday AM and midday peak hours could be fully mitigated by shifting 2 seconds of green time from the northbound/southbound phase to the eastbound phase.

The significant adverse impact at the eastbound approach of this intersection during the PM peak hour could be fully mitigated by shifting 1 second of green time from the northbound/southbound phase to the eastbound phase.

York avenue and east 61st street

The significant adverse impact at the westbound right-turn of this intersection during the weekday AM peak hour could be fully mitigated by shifting 1 second of green time from the northbound/southbound phase to the westbound phase.

first avenue and east 72nd street

This intersection is not impacted during the weekday AM peak hour. However, the signal timing was adjusted under the mitigation conditions (shifting 1 second of green time from the northbound phase to the eastbound/westbound phase) in order to accommodate the proposed mitigation measures at the intersection of York Avenue and East 73rd Street as described above.

The significant adverse impact at the eastbound defacto left-turn of this intersection during the weekday PM peak hour could be fully mitigated by shifting 2 seconds of green time from the northbound phase to the eastbound/westbound phase.

first avenue and east 65th street

The significant adverse impact at the eastbound approach of this intersection during the weekday AM, midday, and PM peak hours could be fully mitigated by shifting 1 second of green time from the northbound phase to the eastbound phase.

Effects of Traffic Mitigations on Pedestrian Operations

As described above, intersection operations would change with the implementation of the recommended traffic mitigation measures. These measures would include changes to existing signal timings and lane utilizations. A review of the effects of these changes on pedestrian circulation and service levels at intersection corners and crosswalks showed that they would not alter the conclusions made for the pedestrian impact analyses, nor would they result in the potential for any additional significant adverse pedestrian impacts.

- construction

transportation

As discussed in Chapter 15, “Construction,” the peak construction traffic increments would be lower than the full operational traffic increments associated with the proposed project in 2019. Therefore, the potential traffic impacts during peak construction would be within the envelope of significant adverse traffic impacts identified for the Build condition in Chapter 9, “Transportation.” Nonetheless, because existing and No Build traffic conditions at some of the study area intersections through which construction-related traffic would also travel were determined to operate at unacceptable levels during commuter peak hours, it is possible that significant adverse traffic impacts could occur at some or many of these locations during construction. In order to alleviate construction traffic impacts, measures recommended to mitigate impacts associated with the operational traffic of the proposed project could be implemented during construction before full build-out of the proposed project. As detailed above, measures to mitigate the operational traffic impacts in 2019 were recommended for implementation at 10 out of the 11 different impacted intersections during weekday peak hours. These measures would encompass primarily signal timing adjustments and other operational measures, all of which could be implemented earlier at the discretion of NYCDOT to address actual conditions experienced at that time. However, traffic impacts during construction at the York Avenue and East 79th Street intersection would likewise be unmitigated. Between the Draft and Final EIS, In coordination with NYCDOT, additional analysis of construction traffic will be was prepared as presented in Chapter 15, “Construction,” of the FEIS. The proposed project would result in significant adverse traffic impacts at three different intersections during the PM construction peak hour. The significant adverse impacts at the intersections of York Avenue and East 73rd Street and First Avenue and East 72nd Street could be fully mitigated by applying mitigation measures similar to those proposed for mitigation under the operational conditions. Similar to the operational conditions, significant adverse impacts at the intersection of York Avenue and East 79th Street could not be fully mitigated during the PM construction peak hour.	