# Transportation

- Introduction

As described in detail in Chapter 1, “Project Description,” Memorial Sloan-Kettering Cancer Center (MSK) and The City University of New York (CUNY) are partnering to build a new ambulatory care center (MSK ACC) and the Hunter College Science and Health Professions Building (CUNY-Hunter Building) adjacent to the Franklin Delano Roosevelt (FDR) Drive between East 73rd and East 74th Streets. In total, the project would introduce 1,134,159 square feet (sf) with 749,357 gsf in the MSK ACC and 402,990 gsf in the CUNY-Hunter Building. The MSK ACC would contain ambulatory care facilities, including clinics for dermatological, breast, and prostate cancers; consultation rooms; infusion rooms; a medical/surgical clinic; an interventional radiology clinic; a bone marrow transplant clinic; academic offices; a pharmacy; and conference rooms, as well as up to 250 accessory parking spaces for patients and visitors. The CUNY-Hunter Building would house teaching and research laboratories, class rooms, a learning center, a single 350-seat lecture hall, faculty offices, and a vivarium to house research animals.

This chapter examines the potential effects of the proposed MSK-CUNY project on the study area transportation systems, and compares the probable impacts of the proposed project (the “Build” condition) with the future without the proposed project (the “No Build” condition). The analyses consider the 2019 analysis year to identify potential impacts, and if warranted, determine feasible mitigation measures that would be appropriate to address those impacts (Chapter 17, “Mitigation,” presents details on the proposed mitigation measures). The travel demand projections, trip assignments, and capacity analysis contained in this chapter were conducted pursuant to the methodologies outlined in the June 2012 City Environmental Quality Review (CEQR) Technical Manual.

Principal Conclusions

Traffic

Traffic conditions were evaluated at 19 intersections for the weekday AM, midday, and PM peak hours. Under the future with the proposed project, there would be the potential for significant adverse impacts at 11 different intersections, 8 intersections each during the weekday AM, midday, and PM peak hours, as follows:

Weekday AM Peak Hour

- York Avenue and East 79th Street – eastbound and northbound approaches;

- York Avenue and East 74th Street – eastbound approach;

- York Avenue and East 73rd Street – northbound approach, southbound defacto left-turn, and southbound through/right-turn;

- York Avenue and East 72nd Street – eastbound defacto left-turn and northbound approach;

- York Avenue and East 71st Street – northbound approach;

- York Avenue and East 65th Street – eastbound approach;

- York Avenue and East 61st Street – westbound right-turn; and

- First Avenue and East 65th Street – eastbound approach.

Weekday Midday Peak Hour

- York Avenue and East 79th Street – eastbound and northbound approaches;

- York Avenue and East 75th Street – northbound approach;

- York Avenue and East 74th Street – eastbound and westbound approaches;

- York Avenue and East 73rd Street – northbound and southbound approaches;

- York Avenue and East 72nd Street – eastbound defacto left-turn and northbound approach;

- York Avenue and East 66th Street – northbound approach;

- York Avenue and East 65th Street – eastbound approach; and

- First Avenue and East 65th Street – eastbound approach.

Weekday PM Peak Hour

- York Avenue and East 79th Street – eastbound approach and northbound through/right-turn;

- York Avenue and East 74th Street – eastbound and westbound approaches;

- York Avenue and East 73rd Street – westbound approach, northbound approach, southbound defacto left-turn, and southbound through/right-turn;

- York Avenue and East 72nd Street – eastbound defacto left-turn and northbound approach;

- York Avenue and East 66th Street – southbound approach;

- York Avenue and East 65th Street – eastbound approach;

- First Avenue and 72nd Street – eastbound defacto left-turn; and

- First Avenue and East 65th Street – eastbound approach.

Table 9-1 provides a summary of the above impacted locations by analysis time periods. Traffic capacity improvements that would be needed to mitigate these significant adverse impacts are addressed in Chapter 17, “Mitigation.”

Transit

The preliminary screening assessment summarized below concluded that a bus line-haul analysis of the M66 and M72 bus routes, a line-haul analysis of the future Second Avenue Q subway line, and a detailed analysis of station elements at the 72nd Street/Second Avenue subway station (future Second Avenue Q line), which is currently under Phase 1 construction and planned to open in 2016, were warranted. Based on the results of the transit analyses, the proposed project would not result in any significant adverse impacts on subway line-haul or circulation and control area elements at the future Second Avenue Subway station. In addition, a detailed allocation of incremental bus riders onto specific segments of the M66 and M72 bus routes was performed. This analysis concluded that the proposed project would not have the potential to incur a significant adverse line-haul impact on either of these bus routes.

| Table 9-1 Summary of Significant Adverse Traffic Impacts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Intersection |  | AM Peak Hour | Midday Peak Hour | PM Peak Hour |
| EB/WB Street | NB/SB Street |  |  |  |
| East 79th Street | York Avenue | EB-LTR | EB-LTR | EB-LTR |
|  |  | NB-LTR | NB-LTR | NB-TR |
| East 75th Street | York Avenue |  | NB-LTR |  |
| East 74th Street | York Avenue | EB-LTR | EB-LTR | EB-LTR |
|  |  |  | WB-LR | WB-LR |
| East 73rd Street | York Avenue |  |  | WB-LTR |
|  |  | NB-LTR | NB-LTR | NB-LTR |
|  |  | SB-DefL |  | SB-DefL |
|  |  |  | SB-LTR |  |
|  |  | SB-TR |  | SB-TR |
| East 72nd Street | York Avenue | EB-DefL | EB-DefL | EB-DefL |
|  |  | NB-LTR | NB-LTR | NB-LTR |
| East 71st Street | York Avenue | NB-LTR |  |  |
| East 66th Street | York Avenue |  | NB-LTR |  |
|  |  |  |  | SB-LTR |
| East 65th Street | York Avenue | EB-LR | EB-LR | EB-LR |
| East 61st Street | York Avenue | WB-R |  |  |
| East 72nd Street | First Avenue |  |  | EB-DefL |
| East 65th Street | First Avenue | EB-LT | EB-LT | EB-LT |
| Notes: EB = Eastbound; WB = Westbound; NB = Northbound; SB = Southbound; L = Left Turn; T = Through; R = Right Turn; DefL = Defacto Left Turn |  |  |  |  |

Pedestrians

Weekday peak period pedestrian conditions were evaluated at key sidewalk, corner reservoir, and crosswalk elements at seven area intersections. It was concluded that the proposed project would not result in any significant adverse pedestrian impacts at any of the analysis locations.

Vehicular and Pedestrian Safety

Accident data for the study area intersections were obtained from the New York State Department of Transportation (NYSDOT) for the time period between January 1, 2009 and December 31, 2011. During this period, a total of 280 reportable and non-reportable accidents, zero fatalities, 209 injuries, and 68 pedestrian/bicyclist-related accidents occurred at the study area intersections. A rolling total of accident data identifies two study area intersections as high accident locations in the 2009 to 2011 period. These locations are First Avenue at East 72nd Street and York Avenue at East 72nd Street.

With the proposed project, the intersection of First Avenue and East 72nd Street would experience moderate increases in vehicular and pedestrian traffic. The incremental vehicular and pedestrian levels at this intersection would be above the CEQR analysis threshold of 50 peak hour vehicle trips while the incremental pedestrian levels would be below the CEQR analysis threshold of 200 peak hour pedestrian trips. The intersection of First Avenue and East 72nd Street would be impacted during the weekday PM peak hour. However, as described in Chapter 17, “Mitigation,” the predicted impact at this intersection could be fully mitigated with standard traffic engineering measures. Therefore, the proposed project is not anticipated to exacerbate any of the current causes of pedestrian-related accidents. Nonetheless, additional safety measures, such as the installation of countdown timers on all pedestrian crosswalks, the installation of pedestrian safety signs warning turning vehicles to yield to pedestrians in the crosswalk, and restriping both the faded north and south crosswalks, can be implemented to improve pedestrian safety at this intersection.

With the proposed project, the intersection of York Avenue and East 72nd Street would experience noticeable increases in vehicular and pedestrian traffic. The incremental vehicular and pedestrian levels at this intersection would be above the CEQR analysis threshold of 50 peak hour vehicle trips and 200 peak hour pedestrian trips. The intersection of York Avenue and East 72nd Street would be impacted during all three analysis peak hours. However, as described in Chapter 17, “Mitigation,” the predicted impacts at this intersection could be fully mitigated with standard traffic engineering measures. Therefore, the proposed project is not anticipated to exacerbate any of the current causes of pedestrian-related accidents. Nonetheless, additional safety measures, such as the installation of countdown timers on all pedestrian crosswalks and the installation of pedestrian safety signs warning turning vehicles to yield to pedestrians in the crosswalk, can be implemented to improve pedestrian safety at this intersection.

Parking

The proposed project would displace existing public parking spaces and include new off-street accessory parking spaces. In the Build condition, anticipated future development projects (including No Build projects and the proposed project) are expected to displace the surface public parking lot on the western portion of the project site, for a total displacement of 128 parking spaces. The proposed project would include a total of up to 250 off-street accessory parking spaces. Accounting for the displacement of the public parking spaces, the addition of the accessory parking spaces, and the parking demand generated from background growth, No Build projects, and the proposed project, the Build public parking supply and utilization analysis shows that there would be a parking shortfall during the weekday midday period within the ¼-mile off-street parking study area. It is anticipated that the excess demand could be accommodated with a slightly longer walking distance beyond the ¼-mile radius. Furthermore, as stated in the CEQR Technical Manual, a parking shortfall resulting from a project located in Manhattan does not constitute a significant adverse parking impact, due to the magnitude of available alternative modes of transportation.

Summary of mitigation Analysis

Out of the 11 different impacted traffic intersections summarized above, all projected significant adverse impacts, except for those at one study area intersection, could be fully mitigated with readily implementable measures, such as signal retiming, changes to parking regulations, lane restriping, and prohibition of left-turns. The specific measures that would be feasible to mitigate the significant adverse impacts summarized above are further discussed in Chapter 17, “Mitigation.” These measures would be subject to the review and approval by the New York City Department of Transportation (NYCDOT).

- Preliminary Analysis Methodology and Screening Assessment

The CEQR Technical Manual recommends a two-tier screening procedure for the preparation of a “preliminary analysis” to determine if quantified analyses of transportation conditions are warranted. As discussed below, the preliminary analysis begins with a trip generation analysis (Level 1) to estimate the volume of person and vehicle trips attributable to the proposed project. If the proposed project is expected to result in fewer than 50 peak hour vehicle trips and fewer than 200 peak hour transit or pedestrian trips, further quantified analyses are not warranted. When these thresholds are exceeded, detailed trip assignments (Level 2) are performed to estimate the incremental trips at specific transportation elements and to identify potential locations for further analyses. If the trip assignments show that the proposed project would result in 50 or more peak hour vehicle trips at an intersection, 200 or more peak hour subway trips at a station, 200 or more peak hour subway trips in one direction on a subway line, 50 or more peak hour bus trips in one direction along a bus route, or 200 or more peak hour pedestrian trips traversing a pedestrian element, then further quantified analyses may be warranted to assess the potential for significant adverse impacts on traffic, transit, pedestrians, parking, and vehicular and pedestrian safety.

Level 1 Screening Assessment

A Level 1 trip generation screening assessment was conducted to estimate the numbers of person and vehicle trips by mode expected to be generated by the proposed project during the weekday AM, midday, and PM peak hours. These estimates were then compared to the CEQR Technical Manual thresholds to determine if a Level 2 screening and/or quantified operational analyses would be warranted. As demonstrated in this section, the Level 1 Screening Assessment indicated the need to undertake a Level 2 screening assessment.

Background

As described above, the proposed project would be developed jointly by MSK and CUNY and would result in the development of the MSK ACC and the CUNY-Hunter Building. Tables 9-2 and 9-3 provide summaries of the projected MSK ACC and CUNY-Hunter Building daily populations.

MSK anticipates that the proposed ACC would primarily be in operation during weekdays, with minimal patient visits during weekends (up to 20 patients per day). In addition, CUNY has indicated that no scheduled classes are currently planned for the CUNY-Hunter Building. Therefore, a weekend analysis for the proposed project is not warranted and the travel demand analyses presented below are for representative weekday conditions.

Transportation Planning Assumptions

Trip generation factors for the proposed project were developed based on information provided by MSK and CUNY-Hunter, other approved Environmental Assessment Statements (EASs) and Environmental Impact Statements (EISs), and assumptions developed in coordination with MSK, CUNY, and NYCDOT. These factors, which have been signed-off by NYCDOT, are detailed below.

MSK ACC Population

For the daily staff person trip rate, it was assumed that each staff member would generate one commuting in trip at the beginning of their shift and one commuting out trip at the end of their shift. Also, approximately 50 percent of the staff during each shift is expected to generate one discretionary roundtrip (one out and one in trip) during the middle of their shift. This resulted in a daily staff person trip rate of three trips per staff and a temporal distribution profile of 33 percent traveling during the commuting in period, 34 percent during the middle of the shift, and 33 percent during the commuting out period. Based on MSK ACC’s anticipated shift allocations, the various staff shifts were overlaid over the 24-hour period to develop the overall staff weekday AM, midday, and PM peak hour temporal and directional distributions presented in Table 9-4.

| Table 9-2 MSK ACC Population |  |
| --- | --- |
| Component | Increment |
| Staff (1) | 1,620 |
| Patients | 1,335 |
| Visitors and Family | 2,670 |
| Building (gsf) | 749,357 |
| Accessory Parking Spaces(2) | Up to 250 |
| Notes: (1) MSK estimates that 95 percent of the staff would work in MSK ACC on a daily basis. Therefore, for trip generation purposes, 1,539 staff (95 percent of 1,620) was utilized. (2) Access and egress to the accessory garage, which would be valet parking, would be provided on East 74th Street. Source: MSK, 2012 |  |

| Table 9-3 CUNY-Hunter Building Population |  |
| --- | --- |
| Component | Increment |
| Undergraduate Students | 1,130 |
| Graduate Students | 1,219 |
| Research Staff | 209 |
| Faculty | 153 |
| Adjunct Faculty | 114 |
| Support Staff | 71 |
| Visitors | 48 |
| Lecture Hall Students (1) | 1,784 |
| Building (gsf) | 402,990 |
| Notes: (1) Lecture hall students consist of existing Hunter College students from the 68th Street main campus who are anticipated to attend classes at the new 350-seat lecture hall at the 74th Street campus. Source: CUNY-Hunter, 2012 |  |

For the daily patients and visitors trip rate, it was assumed that the patients and visitors would generate one trip arriving for their scheduled appointments and one trip departing at the end of their scheduled appointments. Based on MSK’s expectation of appointment scheduling patterns and average duration of each appointment at the new ACC facility, the trip-making of the projected patients and visitors were allocated to a 24-hour period to develop the overall patients and visitors weekday AM, midday, and PM peak hour temporal and directional distributions presented in Table 9-4.

The modal splits for both staff and patients and visitors were provided by MSK based on transportation surveys conducted at existing facilities. Applicable trip-making characteristics from the 2001 MSKCC Rezoning FEIS were also used in the trip estimates. However, because medical treatment has substantially evolved over the past 10 years, the projected population and travel to/from the proposed building rely primarily on discussions with and information provided by MSK’s administrators and facilities personnel. The projected travel profile and trip estimates were then reviewed with MSK to arrive at consensus on their reasonableness and to ensure their consistency with MSK’s current practice and expectations for the new ACC facility.

| Table 9-4 Travel Demand Assumptions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| USE | ACC Staff |  |  | ACC Patients/Visitors |  |  | Undergraduate Students |  |  | Graduate Students |  |  |
| Daily | (1) |  |  | (1) |  |  | (4) |  |  | (4) |  |  |
| Person Trip | 3.0 |  |  | 2.0 |  |  | 3.5 |  |  | 3.5 |  |  |
| Generation Rate | Trips / Person |  |  | Trips / Person |  |  | Trips / Person |  |  | Trips / Person |  |  |
| Absentee Rate | N/A |  |  | N/A |  |  | 5% |  |  | 5% |  |  |
| Link Credit | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| Final Trip Rate | 3.0 |  |  | 2.0 |  |  | 3.3 |  |  | 3.3 |  |  |
| Person Trip | (1) | (1) | (1) | (1) | (1) | (1) | (7) | (7) | (7) | (7) | (7) | (7) |
| Temporal | AM | MD | PM | AM | MD | PM | AM | MD | PM | AM | MD | PM |
| Distribution | 12.1% | 8.1% | 12.2% | 4.0% | 10.0% | 7.5% | 16.4% | 8.0% | 7.1% | 7.5% | 5.7% | 12.2% |
| Directional Distribution | (1) | (1) | (1) | (1) | (1) | (1) | (7) | (7) | (7) | (7) | (7) | (7) |
| In | 97% | 52% | 12% | 100% | 60% | 20% | 99% | 49% | 13% | 99% | 82% | 60% |
| Out | 3% | 48% | 88% | 0% | 40% | 80% | 1% | 51% | 87% | 1% | 18% | 40% |
| Total | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Modal Split | (1) | (1) | (1) | (1) | (1) | (1) | (2,4) | (2,4) | (2,4) | (4) | (4) | (4) |
| Auto | 12.0% | 12.0% | 12.0% | 53.0% | 53.0% | 53.0% | 1.0% | 1.0% | 1.0% | 6.0% | 5.0% | 6.0% |
| Taxi | 4.0% | 4.0% | 4.0% | 14.0% | 14.0% | 14.0% | 2.5% | 2.0% | 2.5% | 2.5% | 2.0% | 2.5% |
| Ambulette | 0.0% | 0.0% | 0.0% | 4.0% | 4.0% | 4.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Subway | 47.0% | 47.0% | 47.0% | 16.0% | 16.0% | 16.0% | 45.0% | 14.0% | 45.0% | 40.0% | 10.0% | 40.0% |
| Bus | 16.0% | 16.0% | 16.0% | 6.0% | 6.0% | 6.0% | 4.5% | 8.0% | 4.5% | 4.5% | 8.0% | 4.5% |
| Walk | 21.0% | 21.0% | 21.0% | 7.0% | 7.0% | 7.0% | 47.0% | 75.0% | 47.0% | 47.0% | 75.0% | 47.0% |
| Total | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Vehicle Occupancy | (3) |  |  | (1) |  |  | (4) |  |  | (4) |  |  |
| Auto | 1.27 |  |  | 3.00 |  |  | 1.20 |  |  | 1.20 |  |  |
| Taxi | 1.35 |  |  | 3.00 |  |  | 1.30 |  |  | 1.30 |  |  |
| Ambulette | N/A |  |  | 3.00 |  |  | N/A |  |  | N/A |  |  |
| Daily | (3) |  |  |  |  |  |  |  |  |  |  |  |
| Delivery Trip | 0.20 |  |  |  |  |  |  |  |  |  |  |  |
| Generation Rate | Delivery Trips / 1,000 SF |  |  |  |  |  |  |  |  |  |  |  |
| Delivery Trip | (3) | (3) | (3) |  |  |  |  |  |  |  |  |  |
| Temporal | AM | MD | PM |  |  |  |  |  |  |  |  |  |
| Distribution | 10.0% | 9.0% | 5.0% |  |  |  |  |  |  |  |  |  |
| Directional Distribution | (3) | (3) | (3) |  |  |  |  |  |  |  |  |  |
| In | 50% | 50% | 50% |  |  |  |  |  |  |  |  |  |
| Out | 50% | 50% | 50% |  |  |  |  |  |  |  |  |  |
| Total | 100% | 100% | 100% |  |  |  |  |  |  |  |  |  |
| USE | Research Staff/Faculty/ Adjunct Faculty |  |  | Support Staff |  |  | Visitors |  |  | Lecture Hall Students |  |  |
| Daily | (4) |  |  | (4) |  |  | (8) |  |  | (1) |  |  |
| Person Trip | 3.5 |  |  | 3.5 |  |  | 0.26 |  |  | 2.0 |  |  |
| Generation Rate | Trips / Person |  |  | Trips / Person |  |  | Trips / 1,000 SF |  |  | Trips / Person |  |  |
| Absentee Rate | 20% |  |  | 5% |  |  | N/A |  |  | N/A |  |  |
| Link Credit | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| Final Trip Rate | 2.8 |  |  | 3.3 |  |  | 0.29 |  |  | 2.0 |  |  |
| Person Trip | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (1) | (1) | (1) |
| Temporal | AM | MD | PM | AM | MD | PM | AM | MD | PM | AM | MD | PM |
| Distribution | 11.0% | 9.0% | 11.0% | 21.0% | 13.0% | 5.5% | 15.0% | 15.0% | 1.5% | 0.0% | 18.2% | 8.4% |
| Directional Distribution | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (1) | (1) | (1) |
| In | 95% | 50% | 30% | 95% | 50% | 30% | 90% | 60% | 0% | 50% | 49% | 100% |
| Out | 5% | 50% | 70% | 5% | 50% | 70% | 10% | 40% | 100% | 50% | 51% | 0% |
| Total | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Modal Split | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4,6) | (4) | (5) | (5) | (5) |
| Auto | 35.0% | 5.0% | 35.0% | 20.5% | 5.0% | 20.5% | 25.0% | 25.0% | 25.0% | 0.0% | 0.0% | 0.0% |
| Taxi | 3.5% | 2.0% | 3.5% | 1.5% | 2.0% | 1.5% | 2.0% | 2.0% | 2.0% | 0.0% | 0.0% | 0.0% |
| Ambulette | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Subway | 31.0% | 10.0% | 31.0% | 49.5% | 10.0% | 49.5% | 50.0% | 50.0% | 50.0% | 0.0% | 0.0% | 0.0% |
| Bus | 6.5% | 8.0% | 6.5% | 12.5% | 8.0% | 12.5% | 13.0% | 13.0% | 13.0% | 25.0% | 25.0% | 25.0% |
| Walk | 24.0% | 75.0% | 24.0% | 16.0% | 75.0% | 16.0% | 10.0% | 10.0% | 10.0% | 75.0% | 75.0% | 75.0% |
| Total | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Vehicle Occupancy | (4) |  |  | (4) |  |  | (4) |  |  | (4) |  |  |
| Auto | 1.20 |  |  | 1.20 |  |  | 1.20 |  |  | 1.20 |  |  |
| Taxi | 1.30 |  |  | 1.30 |  |  | 1.30 |  |  | 1.30 |  |  |
| Ambulette | N/A |  |  | N/A |  |  | N/A |  |  | N/A |  |  |

| Table 9-4 (cont’d) Travel Demand Assumptions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| USE | Research Staff/Faculty/ Adjunct Faculty |  |  |  | Support Staff | Visitors |  |  | Lecture Hall Students |  |  |
| Daily |  |  |  |  |  | (4) |  |  |  |  |  |
| Delivery Trip |  |  |  |  |  | 0.03 |  |  |  |  |  |
| Generation Rate |  |  |  |  |  | Delivery Trips / 1,000 SF |  |  |  |  |  |
| Delivery Trip |  |  |  |  |  | (4) | (4) | (4) |  |  |  |
| Temporal |  |  |  |  |  | AM | MD | PM |  |  |  |
| Distribution |  |  |  |  |  | 9.7% | 9.1% | 5.1% |  |  |  |
| Directional Distribution |  |  |  |  |  | (4) | (4) | (4) |  |  |  |
| In |  |  |  |  |  | 50% | 50% | 50% |  |  |  |
| Out |  |  |  |  |  | 50% | 50% | 50% |  |  |  |
| Total |  |  |  |  |  | 100% | 100% | 100% |  |  |  |
| Sources: (1) Based on information provided by MSK or CUNY. (2) The undergraduate student modal splits were further adjusted based on the ratio of the undergraduate and graduate student auto mode shares presented in the NYU Core FEIS (2012). (3) MSKCC Rezoning FEIS (2001). (4) CUNY Hunter School of Social Work Project EAF (2008). (5) Based on the undergraduate student midday modal splits and adjusted by reducing all other modes besides walk and added to the bus share to reflect students traveling from the 68th Street campus. (6) Adjusted to reflect AM and PM modal splits. (7) NYU Core FEIS (2012). Adjusted based on the anticipated daily peaking characteristics for each of the school’s population groups. (8) Based on information provided by CUNY and adjusted based on the anticipated visitor population and square footage of the proposed CUNY-Hunter Building. |  |  |  |  |  |  |  |  |  |  |  |

CUNY-Hunter Building Population

Trip generation factors, including the daily person trip rates for the undergraduate and graduate students, research staff, faculty, adjunct faculty, and support staff for the proposed school populations are primarily based on the CUNY Hunter School of Social Work Project EAF (2008). The daily visitor trip rate is based on the anticipated total daily visitor trips (assumed two trips per visitor) divided by the proposed square footage of the proposed CUNY-Hunter Building. The daily person trip rate for the lecture hall students is based on the assumption that each student would generate one arriving trip and one departing trip.

The temporal and directional distributions for the CUNY undergraduate and graduate student populations are based on the NYU Core FEIS (2012) and adjusted for the anticipated daily peaking characteristics for each of the school’s population groups. The temporal and directional distributions for the other CUNY populations (except for the lecture hall students) are based on the CUNY Hunter School of Social Work Project EAF and adjusted for the anticipated daily peaking characteristics for each of the school’s population groups. The temporal and directional distributions for the lecture hall students are based on CUNY-Hunter’s anticipated class scheduling patterns and durations.

The modal splits for the CUNY populations are also primarily based on the CUNY Hunter School of Social Work Project EAF. The modal splits for the undergraduate students were further adjusted based on the ratio of the undergraduate and graduate student auto mode shares presented in the NYU Core FEIS. The adjusted undergraduate student midday modal splits were also applied to the lecture hall students and further adjusted by reducing all other modes besides walk and adding it to the bus share to reflect students traveling from the 68th Street campus.

Similar to the projected travel profile and trip estimates developed for the proposed MSK ACC, the process undertaken for estimating future trip-making to/from the proposed CUNY-Hunter Building involved a careful review of findings made in several previously approved studies and consultation with CUNY-Hunter’s administrators and facilities personnel. The resulting estimates are in line with current practice and expectations for the new proposed building.

Travel Demand Projection Summary

As summarized in Table 9-5, the proposed project would generate a total of 2,000, 2,516, and 2,381 person trips during the weekday AM, midday, and PM peak hours, respectively. Approximately 316, 329, and 375 vehicle trips would be generated during the same respective time periods.

| Table 9-5 Trip Generation Summary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSK ACC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Person Trips |  |  |  |  |  |  |  |  |  |  | Vehicle Trips |  |  |  |  |  |
| Peak Hour | In/Out | Auto | Taxi | Ambulette | Subway |  | Bus |  | Walk | Total | In/Out | Auto | Taxi | Ambulette | Delivery | Total |
| AM | In | 235 | 67 | 13 | 306 |  | 106 |  | 136 | 863 | In | 108 | 31 | 4 | 7 | 150 |
|  | Out | 2 | 1 | 0 | 8 |  | 3 |  | 4 | 18 | Out | 2 | 31 | 4 | 7 | 44 |
|  | Total | 237 | 68 | 13 | 314 |  | 109 |  | 140 | 881 | Total | 110 | 62 | 8 | 14 | 194 |
| Midday | In | 278 | 75 | 19 | 168 |  | 60 |  | 75 | 675 | In | 103 | 41 | 8 | 7 | 159 |
|  | Out | 192 | 52 | 13 | 135 |  | 48 |  | 60 | 500 | Out | 74 | 41 | 8 | 7 | 130 |
|  | Total | 470 | 127 | 32 | 303 |  | 108 |  | 135 | 1,175 | Total | 177 | 82 | 16 | 14 | 289 |
| PM | In | 72 | 20 | 5 | 51 |  | 18 |  | 22 | 188 | In | 27 | 43 | 7 | 4 | 81 |
|  | Out | 314 | 87 | 19 | 310 |  | 108 |  | 138 | 976 | Out | 131 | 43 | 7 | 4 | 185 |
|  | Total | 386 | 107 | 24 | 361 |  | 126 |  | 160 | 1,164 | Total | 158 | 86 | 14 | 8 | 266 |
| CUNY-Hunter Building |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Person Trips |  |  |  |  |  |  |  |  |  |  | Vehicle Trips |  |  |  |  |  |
| Peak Hour | In/Out | Auto | Taxi | Ambulette | Subway | Bus |  | Walk |  | Total | In/Out | Auto | Taxi | Ambulette | Delivery | Total |
| AM | In | 86 | 28 | 0 | 464 | 57 |  | 466 |  | 1,101 | In | 72 | 22 | 0 | 1 | 95 |
|  | Out | 4 | 0 | 0 | 8 | 0 |  | 6 |  | 18 | Out | 4 | 22 | 0 | 1 | 27 |
|  | Total | 90 | 28 | 0 | 472 | 57 |  | 472 |  | 1,119 | Total | 76 | 44 | 0 | 2 | 122 |
| Midday | In | 16 | 8 | 0 | 51 | 114 |  | 547 |  | 736 | In | 15 | 8 | 0 | 0 | 23 |
|  | Out | 9 | 5 | 0 | 36 | 105 |  | 450 |  | 605 | Out | 9 | 8 | 0 | 0 | 17 |
|  | Total | 25 | 13 | 0 | 87 | 219 |  | 997 |  | 1,341 | Total | 24 | 16 | 0 | 0 | 40 |
| PM | In | 34 | 10 | 0 | 149 | 93 |  | 391 |  | 677 | In | 29 | 18 | 0 | 0 | 47 |
|  | Out | 52 | 15 | 0 | 220 | 27 |  | 226 |  | 540 | Out | 44 | 18 | 0 | 0 | 62 |
|  | Total | 86 | 25 | 0 | 369 | 120 |  | 617 |  | 1,217 | Total | 73 | 36 | 0 | 0 | 109 |
| Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Person Trips |  |  |  |  |  |  |  |  |  |  | Vehicle Trips |  |  |  |  |  |
| Peak Hour | In/Out | Auto | Taxi | Ambulette | Subway | Bus |  | Walk |  | Total | In/Out | Auto | Taxi | Ambulette | Delivery | Total |
| AM | In | 321 | 95 | 13 | 770 | 163 |  | 602 |  | 1,964 | In | 180 | 53 | 4 | 8 | 245 |
|  | Out | 6 | 1 | 0 | 16 | 3 |  | 10 |  | 36 | Out | 6 | 53 | 4 | 8 | 71 |
|  | Total | 327 | 96 | 13 | 786 | 166 |  | 612 |  | 2,000 | Total | 186 | 106 | 8 | 16 | 316 |
| Midday | In | 294 | 83 | 19 | 219 | 174 |  | 622 |  | 1,411 | In | 118 | 49 | 8 | 7 | 182 |
|  | Out | 201 | 57 | 13 | 171 | 153 |  | 510 |  | 1,105 | Out | 83 | 49 | 8 | 7 | 147 |
|  | Total | 495 | 140 | 32 | 390 | 327 |  | 1,132 |  | 2,516 | Total | 201 | 98 | 16 | 14 | 329 |
| PM | In | 106 | 30 | 5 | 200 | 111 |  | 413 |  | 865 | In | 56 | 61 | 7 | 4 | 128 |
|  | Out | 366 | 102 | 19 | 530 | 135 |  | 364 |  | 1,516 | Out | 175 | 61 | 7 | 4 | 247 |
|  | Total | 472 | 132 | 24 | 730 | 246 |  | 777 |  | 2,381 | Total | 231 | 122 | 14 | 8 | 375 |

As per the criteria established in the CEQR Technical Manual, a quantified transportation analysis may be warranted if a proposed project is expected to result in 50 or more vehicle trips, 200 or more transit trips (200 or more peak hour transit riders at any given subway station, 200 or more peak hour transit riders on a particular subway line in one direction, or 50 or more peak hour bus trips on a particular route in one direction), and/or 200 or more pedestrian trips during a given peak hour.

Traffic

Since the projected incremental vehicle trips would be greater than the CEQR Technical Manual analysis threshold of 50 peak hour vehicle trips, a Level 2 screening assessment is warranted to determine if there is a need for additional quantified traffic analyses and identify the potential intersections warranting analysis. The Level 2 screening assessment for traffic is provided in the next section, “Level 2 Screening Assessment.”

Transit

As shown in Table 9-5, the proposed project would generate a total of approximately 786, 390, and 730 person trips by subway and 166, 327, and 246 person trips by bus during the weekday AM, midday, and PM peak hours, respectively. Since the net incremental bus trips would be at or greater than 200 during the midday and PM peak hours, and the net incremental subway trips would be greater than 200 during all three peak hours, a Level 2 screening assessment is warranted to determine if there is a need for additional quantified transit analyses. The Level 2 screening assessment for transit is provided in the next section, “Level 2 Screening Assessment.”

Pedestrian

As shown in Table 9-5, the proposed project would result in more than 200 pedestrian trips in all three peak hours. Therefore, a Level 2 screening assessment is warranted to determine if there is a need for additional quantified pedestrian analyses. The Level 2 screening assessment for pedestrians is provided in the next section, “Level 2 Screening Assessment.”

Level 2 Screening Assessment

As detailed in the previous section, the Level 1 Screening Assessment indicated that a Level 2 Screening Assessment is warranted for traffic, transit, and pedestrians. A Level 2 screening assessment involves the distribution and assignment of projected trips to the transportation network and the determination of whether specific locations are expected to experience incremental trips exceeding CEQR Technical Manual thresholds. If the results of this analysis show that the proposed project would result in 50 or more peak hour vehicle trips through an intersection, 50 or more peak hour bus riders on a bus route in a single direction, 200 or more peak hour subway passengers per station, or 200 or more peak hour pedestrian trips per pedestrian element, further quantified analyses may be warranted to evaluate the potential for significant adverse traffic, transit, pedestrian, and parking impacts.

For the proposed MSK/CUNY-Hunter project, trips projected for the 2019 analysis year, representing the maximum number of project-generated trips, were allocated to the area’s roadways, transit facilities, and pedestrian elements and the proposed site plan was considered. The project site is bounded by East 74th Street to the north, East 73rd Street to the south, the FDR Drive to the east, and York Avenue to the west. Access and egress to the proposed MSK ACC accessory parking garage for the patients and visitors would be provided on the south side of East 74th Street. The primary entrances for both MSK ACC and CUNY-Hunter Building would be provided on East 74th Street. There would be a secondary entrance on East 73rd Street in the MSK ACC only accessible by its staff. Enclosed loading docks for the MSK ACC and CUNY-Hunter Building would be provided on East 73rd Street.

Traffic

Vehicle trips were assigned to the area intersections based on the most likely routes to and from the project site (primarily FDR Drive, York Avenue, First Avenue, Second Avenue, East 79th Street, East 72nd Street, and East 66th Street), the configuration and direction of the roadway network, prevailing travel patterns, commuter origin-destination summary from the census data, zip-code statistics from student enrollment data provided by CUNY-Hunter, and the expected locations of the site access and egress points. Project-generated vehicle trips to and from the project site were assigned to the area’s street network. Auto trips were assigned to and from the proposed MSK ACC accessory garage and to other public parking facilities in the area (Section G, “Parking Conditions Assessment,” provides details on the inventory of off-street parking facilities in the study area). Taxi trips were assigned to various project block fronts (including East 74th Street, East 73rd Street, and York Avenue), ambulette trips were assigned to the MSK ACC front entrance on East 74th Street, and delivery vehicles were assigned to the site via NYCDOT-designated truck routes.

MSK ACC Staff

The distribution of vehicle trips generated by MSK ACC staff was based on the origin-destination patterns from the Reverse-Journey-to-Work (RJTW) statistics for the study area obtained from the 2000 U.S. Census. The vehicle trips were distributed to the study area roadways in the following manner: approximately 30 percent of project-generated vehicle trips were assume to approach the project site from the north, 40 percent from the south, 25 percent from the east, and 5 percent from the west.

MSK ACC Patients and Visitors

The distribution of vehicle trips generated by MSK ACC patients and visitors was developed based on a review of the origin-destination patterns from the RJTW statistics for the study area obtained from the 2000 U.S. Census, most likely access routes to and from the project site, population densities surrounding the project site, and adjusted to reflect a more even geographical draw. The vehicle trips were distributed to the study area roadways in the following manner: approximately 30 percent of project-generated vehicle trips were assumed to approach the project site from the north, 40 percent from the south, 15 percent from the east, and 15 percent from the west. The on-site accessory garage would provide a limited amount of parking spaces (up to 250 spaces) to accommodate some of the parking demand generated by the patients and visitors. Vehicle trips related to parking demand that could not be accommodated on-site would result in additional recirculation as drivers would drop off the patients at the MSK ACC first before circulating to find off-site parking. This is further discussed in Section C, “Detailed Traffic Analysis.”

CUNY-Hunter Building Research Staff/Faculty/Adjunct Faculty, Support Staff, Visitors

The distribution of vehicle trips generated by the CUNY-Hunter Building research staff, faculty, adjunct faculty, support staff, and visitors was based on the origin-destination patterns from the RJTW statistics for the study area obtained from the 2000 U.S. Census. The vehicle trips were distributed to the study area roadways in the following manner: approximately 30 percent of project-generated vehicle trips were assume to approach the project site from the north, 40 percent from the south, 25 percent from the east, and 5 percent from the west.

CUNY-Hunter Building Students

The distribution of vehicle trips generated by CUNY-Hunter Building undergraduate and graduate students was based on the zip-code statistics from the existing student enrollment data provided by CUNY-Hunter. The vehicle trips were distributed to the study area roadways in the following manner: approximately 25 percent of project-generated vehicle trips were assumed to approach the project site from the north, 35 percent from the south, 35 percent from the east, and 5 percent from the west.

Summary

The project-generated vehicle trip assignments are shown in Figures 9-1 to 9-3. As shown in Figure 9-4, 19 traffic intersections were identified for detailed analysis. They include:

- York Avenue and East 79th Street

- York Avenue and East 76th Street

- York Avenue and East 75th Street

- York Avenue and East 74th Street

- York Avenue and East 73rd Street

- York Avenue and East 72nd Street

- York Avenue and East 71st Street

- York Avenue and East 66th Street

- York Avenue and East 65th Street

- York Avenue and East 61st Street

- First Avenue and East 74th Street

- First Avenue and East 73rd Street

- First Avenue and East 72nd Street

- First Avenue and East 66th Street

- First Avenue and East 65th Street

- Second Avenue and East 73rd Street

- Second Avenue and East 72nd Street

- Second Avenue and East 66th Street

- Second Avenue and East 65th Street

The detailed traffic analysis is provided in Section C, “Detailed Traffic Analysis.”

Transit

Subway

As presented in Table 9-5, the proposed project is expected to result in 786, 390, and 730 project-generated subway trips during the weekday morning, midday, and evening peak hours, respectively. These trips were assigned to the 68th Street-Hunter College (No. 6 line) and 77th Street (No. 6 line) subway stations along Lexington Avenue and the planned 72nd Street subway station (Q line) along Second Avenue.

In consultation with the Metropolitan Transportation Authority (MTA) New York City Transit (NYCT), the project-generated subway trips were distributed to the Lexington Avenue and Second Avenue subway lines, the results of which show that an analysis of station elements at the 68th Street and 77th Street Lexington Avenue subway stations are not warranted, but the planned 72nd Street Second Avenue subway station would require detailed analysis of station elements (i.e., fare control areas and station circulation elements such as escalators and stairways) for the weekday morning and evening peak periods (see transit map shown in Figure 9-5), as follows:

- 72nd Street (Q) subway station: control area and associated access elements, including five elevators on the southeast corner of East 72nd Street and Second Avenue, and three escalators on the northwest corner.

The 72nd Street subway station at Second Avenue is currently under Phase 1 construction, and is planned to open in 2016. The detailed subway station analysis is provided in Section D, “Detailed Transit Analysis.”

In addition, line haul capacities (i.e., the ability of transit systems to accommodate passenger loads) are evaluated when a proposed action is anticipated to generate a perceptible number of passengers to particular subway and bus routes. For subways, if a subway line is expected to incur 200 or more passengers in one direction of travel during the commuter peak hours, a detailed review of ridership levels at the maximum load point and/or other project-specific load points would be required to determine if the route’s guideline (or practical) capacity would be exceeded. Subway trips were distributed to local subway routes serving the project site. In consultation with NYCT, it was determined that the peak load point for the planned Q line extension along Second Avenue would be in the southbound direction during the AM peak hour at the Second Avenue and 63rd Street station. The AM load leaving this station will represent the critical load point, and therefore only the AM southbound peak is analyzed for the Q subway line, as presented in Section D, “Detailed Transit Analysis.”

Bus

As presented in Table 9-5, the proposed project is expected to result in 166, 327, and 246 project-generated bus trips during the weekday morning, midday, and evening peak hours, respectively. The project-generated bus trips were distributed to the M15, M31, M66, and M72 buses, which serve the study area. It is also anticipated that some subway riders would transfer to nearby buses including the M66 and M72. Based on the preliminary distribution of projected bus trips, the project-generated peak hour bus trips are not expected to add 50 or more peak hour bus trips in one direction on the M15 and M31 bus lines; therefore, a detailed bus line-haul analysis would not be required for the M15 and M31. However, it is anticipated that the project-generated peak hour bus trips would add 50 or more peak hour bus trips in one direction on the M66 and M72 bus lines; therefore, detailed bus line-haul analysis would be prepared for the M66 and M72. The detailed bus line-haul analysis of the M66 and M72 bus lines is provided in Section D, “Detailed Transit Analysis.”

Pedestrians

As shown in Table 9-5, the projected peak hour pedestrian increments would exceed the CEQR analysis threshold of 200 pedestrians during the weekday morning, midday, and evening peak hours. Level 2 pedestrian trip assignments were developed for all proposed components and are discussed as follows:

- Auto Trips – The majority of motorists would park at the nearest available public parking facilities and would walk to and from the project site; up to 250 accessory parking spaces would be provided on-site for MSK ACC patients and visitors. As described above, motorists dropping off patients at the ACC first and circulating to find off-site parking would also park at the nearest available public parking facilities and would walk to and from the project site.

- Taxi Trips – Taxi patrons would be dropped off and picked up along East 74th Street, East 73rd Street, and York Avenue.

- Ambulette Trips – Ambulette patrons would be dropped off and picked up in front of the MSK ACC main entrance on East 74th Street.

- Bus Trips – Project-generated bus riders would use the M15, M31, M66, and M72 buses and would get on and off at the bus stops nearest to the project site.

- Subway Trips – Project-generated subway riders were assigned to the 68th Street-Hunter College and 77th Street subway stations along the Lexington Avenue line and the planned 72nd Street subway station along the Second Avenue line. It is anticipated that some subway riders would transfer to the nearby buses such as the M66 and M72 to reach the project site. Subsequent to the initial subway pedestrian trip assignments, the allocation of subway pedestrian trips to the nearby subway stations were adjusted in consultation with NYCT. The volumes presented in this analysis reflect these adjustments.

- Walk-Only Trips – Pedestrian walk-only trip assignments were developed by distributing project-generated person trips to surrounding pedestrian facilities, including sidewalks, corner reservoirs, and crosswalks, bordering the project site as well as facilities located in the vicinity of the project site.

The project-generated pedestrian trip assignments are shown in Figures 9-6 to 9-8. Based on the above assignment of pedestrian trips and the Level 2 assessment criteria, 12 sidewalks, 11 crosswalks, and 23 corners are recommended for detailed analysis, as shown in Figure 9-9 and summarized in Table 9-6. The detailed analysis of these sidewalks, crosswalks, and corners is provided in Section E, “Detailed Pedestrian Analysis.”

- Detailed traffic analysis

As described above in Section B, “Preliminary Analysis Methodology and Screening Assessment,” the Level 1 and Level 2 screening analyses indicated a need for a detailed analysis of 19 traffic analysis locations in the weekday AM, midday, and PM peak periods. This section begins with an overview of the methodology and impact criteria for traffic analyses, and then provides information on existing conditions and the future without the proposed project. This section ends with a comparison of the future without the proposed project to the future with the proposed project for the purposes of identifying where the project has the potential to result in significant adverse traffic impacts.

Methodology and Impact Criteria

The operation of all of the signalized intersection analysis locations was assessed using methodologies presented in the 2000 Highway Capacity Manual (HCM) using the Highway Capacity Software (HCS+ 5.5), which is the analysis methodology approved for use by NYCDOT. The HCM procedure evaluates the levels of service (LOS) for signalized intersections using average stop control delay, in seconds per vehicle, as described below.

| Table 9-6 Pedestrian Study Locations |  |  |
| --- | --- | --- |
| No. | Study Location | Sidewalk |
| Sidewalks |  |  |
| 1 | York Avenue between East 75th Street and East 74th Street | East |
| 2 | East 74th Street between First Avenue and York Avenue | South |
| 3 | East 74th Street between York Avenue and FDR Drive | South |
| 4 | York Avenue between East 74th Street and East 73rd Street | East |
| 5 |  | West |
| 6 | East 73rd Street between York Avenue and FDR Drive | North |
| 7 | York Avenue between East 73rd Street and East 72nd Street | East |
| 8 |  | West |
| 9 | East 72nd Street between First Avenue and York Avenue | North |
| 10 |  | South |
| 11 | York Avenue between East 72nd Street and East 71st Street | East |
| 12 |  | West |
| Crosswalks |  |  |
| 1 | York Avenue and East 75th Street | East |
| 2 | York Avenue and East 74th Street | South |
| 3 |  | East |
| 4 | York Avenue and East 73rd Street | North |
| 5 |  | East |
| 6 |  | West |
| 7 | York Avenue and East 72nd Street | North |
| 8 |  | East |
| 9 |  | West |
| 10 | York Avenue and East 71st Street | East |
| 11 |  | West |
| Corners |  |  |
| 1 | York Avenue and East 75th Street | Northeast |
| 2 |  | Southeast |
| 3 |  | Southwest |
| 4 | York Avenue and East 74th Street | Northeast |
| 5 |  | Northwest |
| 6 |  | Southeast |
| 7 |  | Southwest |
| 8 | York Avenue and East 73rd Street | Northeast |
| 9 |  | Northwest |
| 10 |  | Southeast |
| 11 |  | Southwest |
| 12 | York Avenue and East 72nd Street | Northeast |
| 13 |  | Northwest |
| 14 |  | Southeast |
| 15 |  | Southwest |
| 16 | York Avenue and East 71st Street | Northeast |
| 17 |  | Northwest |
| 18 |  | Southeast |
| 19 |  | Southwest |
| 20 | York Avenue and East 68th Street | Northwest |
| 21 | First Avenue and East 72nd Street | Northeast |
| 22 |  | Northwest |
| 23 |  | Southeast |

The average control delay per vehicle is the basis for LOS determination for individual lane groups (grouping of movements in one or more travel lanes), the approaches, and the overall intersection. The levels of service are defined in Table 9-7.

| Table 9-7 LOS Criteria for Signalized Intersections |  |
| --- | --- |
| LOS | Average Control Delay |
| A | ≤ 10.0 seconds |
| B | >10.0 and ≤ 20.0 seconds |
| C | >20.0 and ≤ 35.0 seconds |
| D | >35.0 and ≤ 55.0 seconds |
| E | >55.0 and ≤ 80.0 seconds |
| F | >80.0 seconds |
| Source: Transportation Research Board. Highway Capacity Manual, 2000. |  |

Although the HCM methodology calculates a volume-to-capacity (v/c) ratio, there is no strict relationship between v/c ratios and LOS as defined in the HCM. A high v/c ratio indicates substantial traffic passing through an intersection, but a high v/c ratio combined with low average delay actually represents the most efficient condition in terms of traffic engineering standards, where an approach or the whole intersection processes traffic close to its theoretical maximum capacity with minimal delay. However, very high v/c ratios—especially those approaching or greater than 1.0—are often correlated with a deteriorated LOS. Other important variables affecting delay include cycle length, progression, and green time. LOS A and B indicate good operating conditions with minimal delay. At LOS C, the number of vehicles stopping is higher, but congestion is still fairly light. LOS D describes a condition where congestion levels are more noticeable and individual cycle failures (a condition where motorists may have to wait for more than one green phase to clear the intersection) can occur. Conditions at LOS E and F reflect poor service levels, and cycle breakdowns are frequent. The HCM methodology also provides for a summary of the total intersection operating conditions. The analysis chooses the two critical movements (the worst case from each roadway) and calculates a summary critical v/c ratio. The overall intersection delay, which determines the intersection’s LOS, is based on a weighted average of control delays of the individual lane groups. Within New York City, the midpoint of LOS D (45 seconds of delay) is generally considered as the threshold between acceptable and unacceptable operations.

According to the criteria presented in the CEQR Technical Manual, impacts are considered significant and require examination of mitigation if they result in an increase in the Build condition of 5 or more seconds of delay in a lane group over No Build levels beyond mid-LOS D. For No Build LOS E, a 4-second increase in delay is considered significant. For No Build LOS F, a 3-second increase in delay is considered significant. In addition, impacts are considered significant if levels of service deteriorate from acceptable A, B, or C in the No Build condition to marginally unacceptable LOS D (a delay in excess of 45 seconds, the midpoint of LOS D), or unacceptable LOS E or F in the Build condition.

2012 Existing Conditions

Roadway Network

The traffic study area characterizes the East Midtown grid pattern, with major north-south one-way flows on First and Second Avenues; major two-way north-south flows on York Avenue; major two-way east-west crosstown movements on East 79th and East 72nd Streets; and local east-west circulation on the narrower one-way side streets west of York Avenue.

York Avenue is a major two-way north-south street with three lanes (including a parking lane) in each direction. First Avenue is a major one-way northbound street with seven lanes (including a bus lane, parking lane, and a bicycle lane). Second Avenue is a major one-way southbound street with six lanes (including parking lanes and a bus lane). East 79th and East 72nd Streets are major east-west streets with three lanes (including a parking lane) in each direction. The local east-west streets generally provide two to three lanes (including a parking lane). East of York Avenue, most of these east-west streets operate as two-way streets and terminate at the FDR Drive.

Traffic in the area is also fed by or into the FDR Drive. In the northbound direction, entry to the FDR Drive is available at East 62nd Street and East 96th Street while exit from the FDR Drive is available at East 61st Street and East 96th Street. In the southbound direction, entry to the FDR Drive is available at East 63rd Street, East 73rd Street, East 79th Street, and East 92nd Street while exit from the FDR Drive is available at East 63rd Street, East 71st Street, and East 96th Street.

Traffic Conditions

Existing traffic volumes for the study area intersections, as shown in Figures 9-10 to 9-12, are established based on field counts (including manual turning movement counts and Automatic Traffic Recorder [ATR] counts) collected in September 2012. In addition, inventories of roadway geometry, traffic controls, bus stops, and parking regulations/activities were recorded to provide appropriate inputs for the operational analyses. Official signal timings were also obtained from NYCDOT for use in the analysis of the study area signalized intersections.

As per the CEQR Technical Manual, the typical weekday analysis peak hours for Manhattan are 8:00 AM to 9:00 AM, 12:00 PM to 1:00 PM, and 5:00 PM to 6:00 PM. For analysis, the highest peak hour traffic volumes (from 7:45 AM to 8:45 AM, 1:00 PM to 2:00 PM, and 5:30 PM to 6:30 PM) during the respective peak periods based on the collected data were used.

During the 2012 data collection, due to ongoing construction of the Second Avenue Subway (Phase 1 expected to be complete by 2016), the number of lanes on Second Avenue was reduced at some of the analyzed study area intersections. Specifically, at the intersections of East 73rd, East 72nd, and East 66th Streets on Second Avenue, four moving traffic lanes were maintained but the parking and bus lanes were temporarily unavailable to accommodate the ongoing construction.

Levels of Service

Table 9-8 provides an overview of the levels of service that characterize existing overall intersection conditions during the weekday AM, midday, and PM peak hours.

The detailed traffic levels of service tables are provided in Section H, “Detailed Analysis Results Tables,” at the end of this chapter. The analysis results shown in Table 9-44 indicate that most of the study area’s intersection approaches/lane groups operate acceptably—at mid-LOS D (delays of 45 seconds per vehicle [spv] or less for signalized intersections) or better for the analysis peak hours. Approaches/lane groups operating at worse than mid-LOS D and those with v/c ratios of 0.90 or greater are listed below.

| Table 9-8 Existing Traffic Level of Service Summary |  |  |  |
| --- | --- | --- | --- |
|  | Weekday |  |  |
|  | AM Peak Hour | Midday Peak Hour | PM Peak Hour |
| Intersections at Overall LOS A/B/C | 16 | 17 | 16 |
| Intersections at Overall LOS D | 2 | 2 | 1 |
| Intersections at Overall LOS E | 1 | 0 | 2 |
| Intersections Overall LOS F | 0 | 0 | 0 |
| Number of individual traffic lane groups at LOS E or F | 7 | 2 | 7 |
| Notes: Nineteen intersections were analyzed. * The number of individual traffic lane groups at LOS E or F is of 60 movements analyzed during the weekday AM and PM peak hours and 59 movements analyzed during the weekday midday peak hour. |  |  |  |

York Avenue

- Eastbound approach at the York Avenue and East 79th Street intersection (LOS E with a v/c ratio of 0.97 during the weekday AM peak hour; LOS D with a delay of 49.0 spv during the weekday midday peak hour; and LOS E with a v/c ratio of 0.96 during the weekday PM peak hour);

- Northbound approach at the York Avenue and East 79th Street intersection (LOS E with a v/c ratio of 1.05 during the weekday AM peak hour; and LOS E with a v/c ratio of 1.05 during the weekday PM peak hour);

- Southbound approach at the York Avenue and East 79th Street intersection (LOS D with a delay of 45.6 spv during the weekday AM peak hour; and LOS D with a delay of 46.6 spv during the weekday PM peak hour);

- Eastbound approach at the York Avenue and East 76th Street intersection (LOS E during the weekday AM peak hour; LOS D with a delay of 45.2 spv during the weekday midday peak hour; and LOS F with a v/c ratio of 1.00 during the weekday PM peak hour);

- Westbound approach at the York Avenue and East 76th Street intersection (LOS D with a delay of 54.6 spv during the weekday AM peak hour);

- Westbound approach at the York Avenue and East 73rd Street intersection (LOS D with a delay of 45.3 spv during the weekday midday peak hour);

- Northbound approach at the York Avenue and East 73rd Street intersection (LOS D with a delay of 51.3 spv and a v/c ratio of 0.97 during the weekday AM peak hour; and LOS E with a v/c ratio of 1.02 during the weekday PM peak hour);

- Southbound defacto left-turn at the York Avenue and East 73rd Street intersection (LOS E with a v/c ratio of 0.93 during the weekday AM peak hour; and LOS F with a v/c ratio of 1.05 during the weekday PM peak hour);

- Southbound through/right-turn at the York Avenue and East 73rd Street intersection (LOS D with a v/c ratio of 0.96 during the weekday AM peak hour; and LOS D with a v/c ratio of 0.94 during the weekday PM peak hour);

- Eastbound defacto left-turn at the York Avenue and East 72nd Street intersection (LOS F with a v/c ratio of 1.00 during the weekday AM peak hour; LOS D with a delay of 48.3 spv during the weekday midday peak hour; and LOS D with a delay of 46.2 spv during the weekday PM peak hour);

- Northbound approach at the York Avenue and East 72nd Street intersection (LOS D with a delay of 50.5 spv and a v/c ratio of 0.98 during the weekday AM peak hour; and LOS D with a delay of 50.8 spv and a v/c ratio of 0.98 during the weekday PM peak hour);

- Northbound approach at the York Avenue and East 66th Street intersection (LOS D with a v/c ratio of 0.95 during the weekday midday peak hour); and

- Eastbound approach at the York Avenue and East 65th Street intersection (LOS E with a v/c ratio of 0.95 during the weekday AM peak hour; LOS E during the weekday midday peak hour; and LOS F with a v/c ratio of 1.05 during the weekday PM peak hour).

First Avenue

- Eastbound approach at the First Avenue and East 72nd Street intersection (LOS D with a v/c ratio of 0.96 during the weekday AM peak hour);

- Northbound approach at the First Avenue and East 66th Street intersection (LOS C with a v/c ratio of 0.93 during the weekday AM peak hour; and LOS C with a v/c ratio of 0.91 during the weekday midday peak hour); and

- Eastbound approach at the First Avenue and East 65th Street intersection (LOS E with a v/c ratio of 0.92 during the weekday AM peak hour; LOS E with a v/c ratio of 0.94 during the weekday midday peak hour; and LOS F with a v/c ratio of 1.04 during the weekday PM peak hour).

Future without the Proposed Project (2019 No Build Condition)

The No Build condition was developed by increasing existing (2012) traffic levels by the expected growth in overall travel through and within the study area. As per CEQR Technical Manual guidelines, an annual background growth rate of 0.25 percent was assumed for the first five years (year 2012 to year 2017) and then 0.125 percent for the remaining years (year 2017 to year 2019). In addition, a total of 12 development projects expected to occur in the No Build condition (No Build projects) were identified as being planned for the study area (see Figure 9-13). However, many of these planned projects are modest in size and would be very modest traffic generators. After reviewing the development programs for each of the planned projects, it was determined that background growth will address the increase in traffic and pedestrian levels for 5 of the small- to moderate-sized projects in the study area. Trips generated by the Cornell NYC Tech project are expected to be part of the background growth in the proposed project’s study area. Person and vehicle trips generated by the remaining projects were then determined and incorporated into the No Build traffic analysis. Table 9-9 and Figure 9-13 summarize the projects that were accounted for in this future 2019 baseline, including those that were considered as part of the study area background growth.

| Table 9-9 Planned Projects Within or Near the Study Area by 2019 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Map No. | Location | Description | Transportation Assumptions | Build Year |
| 1 | HSS East 74th Street (Block 1485, Lots 11, 14, and 40) | 13-story, approximately 213,775-sf Ambulatory Surgery Center | Transportation assumptions from Hospital for Special Surgery Ambulatory Surgery Center EAS (2012) | 2016 |
| 2 | NYPH MCF (Block 1463, Lots 21 and 31) | 15-story, approximately 733,272-sf Medical Care Facility | Transportation assumptions from 1285 York Avenue – New York-Presbyterian Hospital Medical Care Facility Draft Travel Demand Factors Memorandum (November 26, 2012) | 2020 |
| 3 | The Charles (Block 1447, Lot 27) | Residential development with 51 units | Included in background growth | 2013 |
| 4 | Belfer Research Building (Block 1464, Lot 8) | 18-story, approximately 480,000-sf Biomedical Research Building | Transportation assumptions from Weill Cornell Medical College – Biomedical Research Building EAS (2008) | 2014 |
| 5 | 350 East 81st Street (Block 1543, Lot 32) | Residential development with 4 units | Included in background growth | 2012 |
| 6 | 234 East 70th Street (Block 1424, Lot 128) | Residential development with 8 units | Included in background growth | 2019 |
| 7 | 1162 2nd Avenue (Block 1436, Lot 1) | Mixed-use development with 30 residential units and 1,035-sf retail | Included in background growth | 2019 |
| 8 | 337 East 62nd Street (Block 1437, Lot 15) | Residential development with 22 units | Included in background growth | 2019 |
| 9 | Rockefeller University (Block 1480, Lot 10) | Approximately 175,000-sf laboratory building and one-story recreation building | Based on current development information, the new laboratory building and new recreation building would better accommodate Rockefeller University’s existing needs and would not result in an increase in population at the University. Therefore, the Rockefeller University project would not be expected to result in any additional vehicular or transit trips. | 2019 |
| 10 | MSK – 1133 York Avenue (Block 1456, Lot 21) | 15-story, approximately 175,701-sf Outpatient Surgical Center | Transportation assumptions from 1133 York Avenue – Memorial Sloan Kettering Outpatient Surgical Center EAS (2012) | 2015 |
| 11 | 1113 York Avenue (Block 1455, Lot 21) | Residential development with 173 units | Assumptions from CEQR Technical Manual and Western Rail Yard FEIS (2009), with modal splits and auto occupancies based on 2006-2010 ACS Estimates | 2015 |
| 12 | Cornell NYC Tech (Block 1373, Lot 20; Block 1372, Lot 1) | 200,000-sf of academic facilities; 100,000-sf of corporate co-location; 170,000-sf of executive education center; 300,000-sf of housing facilities; and 10,000-sf of university retail | Cornell NYC Tech project generated trips expected to be part of the background growth in the proposed project’s traffic study area | 2018 |
| Sources: AKRF, Inc., New York City Department of City Planning. |  |  |  |  |

Traffic Operations

The No Build condition traffic volumes are shown in Figures 9-14 to 9-16 for the weekday AM, midday, and PM peak hours. The No Build condition traffic volumes were projected by layering on top of the existing traffic volumes the following: background growth and trips generated by the discrete No Build projects in the area. Table 9-10 shows an overall comparison of traffic levels of service for existing and No Build conditions.

Under existing conditions, the number of lanes on Second Avenue at some of the analyzed study area intersections was affected by the ongoing construction of the Second Avenue Subway (see discussion above). Phase 1 of the Second Avenue Subway is expected to be complete by 2016; therefore, in the 2019 No Build condition, the number of lanes on Second Avenue will have reverted back to what exists currently. In addition, the recently approved new signal timing plan at the intersection of York Avenue and East 66th Street has also been incorporated into the No Build analysis.

Based on the analysis results presented in Table 9-45 in Section H, “Detailed Analysis Results Tables,” the majority of the approaches/lane-groups will operate at the same LOS as in existing conditions with the following notable exceptions:

| Table 9-10 Traffic Level of Service Summary Comparison: 2012 Existing vs. 2019 No Build Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Existing |  |  | 2019 No Build |  |  |
|  | Weekday Peak Hours |  |  | Weekday Peak Hours |  |  |
|  | AM | Midday | PM | AM | Midday | PM |
| Intersections at Overall LOS A/B/C | 16 | 17 | 16 | 15 | 15 | 16 |
| Intersections at Overall LOS D | 2 | 2 | 1 | 1 | 4 | 0 |
| Intersections at Overall LOS E | 1 | 0 | 2 | 2 | 0 | 1 |
| Intersections at Overall LOS F | 0 | 0 | 0 | 1 | 0 | 2 |
| Number of individual traffic lane groups at LOS E or F | 7 | 2 | 7 | 12 | 4 | 9 |
| Notes: Nineteen intersections were analyzed. * The number of individual traffic lane groups at LOS E or F is of 60 movements analyzed during the weekday AM and PM peak hours and 59 movements analyzed during the weekday midday peak hour for the 2012 Existing condition. The number of individual traffic lane groups at LOS E or F is 60 movements analyzed during the weekday AM peak hour, 59 movements analyzed during the weekday midday peak hour, and 61 movements analyzed during the weekday PM peak hour for the 2019 No Build condition. |  |  |  |  |  |  |

York Avenue

- Eastbound approach at the York Avenue and East 79th Street intersection will deteriorate to LOS F with a v/c ratio of 1.06 and a delay of 95.5 spv during the weekday AM peak hour and to LOS F with a v/c ratio of 1.02 and a delay of 82.5 spv during the weekday PM peak hour;

- Northbound approach at the York Avenue and East 79th Street intersection will deteriorate to LOS F with a v/c ratio of 1.13 and a delay of 107.2 spv during the weekday AM peak hour, within LOS D with a v/c ratio of 0.97 and a delay of 54.7 spv during the weekday midday peak hour, and to LOS F with a v/c ratio of 1.21 and a delay of 137.2 spv during the weekday PM peak hour;

- Westbound approach at the York Avenue and East 76th Street intersection will deteriorate to LOS E with a v/c ratio of 0.74 and a delay of 55.7 spv during the weekday AM peak hour;

- Westbound approach at the York Avenue and East 73rd Street intersection will deteriorate within LOS D with a v/c ratio of 0.20 and a delay of 45.9 spv during the weekday AM peak hour and within LOS D with a v/c ratio of 0.43 and a delay of 52.3 spv during the weekday PM peak hour;

- Northbound approach at the York Avenue and East 73rd Street intersection will deteriorate to LOS E with a v/c ratio of 1.04 and a delay of 71.4 spv during the weekday AM peak hour and to LOS F with a v/c ratio of 1.15 and a delay of 111.0 spv during the weekday PM peak hour;

- Southbound defacto left-turn at the York Avenue and East 73rd Street intersection will deteriorate to LOS F with a v/c ratio of 1.00 and a delay of 81.9 spv during the weekday AM peak hour;

- Southbound approach at the York Avenue and East 73rd Street intersection will deteriorate to LOS E with a v/c ratio of 1.01 and a delay of 56.7 spv during the weekday midday peak hour;

- Southbound through/right-turn at the York Avenue and East 73rd Street intersection will deteriorate to LOS E with a v/c ratio of 1.06 and a delay of 75.4 spv during the weekday AM peak hour and to LOS E with a v/c ratio of 1.00 and a delay of 58.1 spv during the weekday PM peak hour;

- Northbound approach at the York Avenue and East 72nd Street intersection will deteriorate to LOS F with a v/c ratio of 1.09 and a delay of 83.5 spv during the weekday AM peak hour, to LOS D with a v/c ratio of 0.96 and a delay of 46.3 spv during the weekday midday peak hour, and to LOS F with a v/c ratio of 1.10 and a delay of 86.0 spv during the weekday PM peak hour;

- Northbound approach at the York Avenue and East 66th Street intersection will deteriorate to LOS E with a v/c ratio of 1.07 and a delay of 74.2 spv during the weekday midday peak hour;

- Southbound approach at the York Avenue and East 66th Street intersection will deteriorate to LOS D with a v/c ratio of 0.96 and a delay of 46.2 spv during the weekday PM peak hour; and

- Eastbound approach at the York Avenue and East 65th Street intersection will deteriorate to LOS F with a v/c ratio of 1.03 and a delay of 97.6 spv during the weekday AM peak hour;

First Avenue

- Eastbound approach at the First Avenue and East 72nd Street intersection will deteriorate to LOS E with a v/c ratio of 0.98 and a delay of 57.1 spv during the weekday AM peak hour; and

- Northbound left-turn at the First Avenue and East 72nd Street intersection will deteriorate within LOS D with a v/c ratio of 0.60 and a delay of 45.4 spv during the weekday AM peak hour.

probable impacts of the proposed project (2019 Build Condition)

As discussed above in the “Level 2 Screening Assessment” section of Section B, “Preliminary Analysis Methodology and Screening Assessment,” the project-generated vehicle trips were assigned to the proposed MSK ACC accessory garage, to other public parking facilities in the study area, and to various project block fronts at or in the immediate vicinity of the project site. Overall, the 2019 completion of the proposed project would result in approximately 316, 329, and 375 incremental vehicle trips during the weekday AM, midday, and PM peak hours, respectively. The related peak hour traffic assignments are discussed above in Section B and the project-generated peak hour trips are shown in Figures 9-1 to 9-3.

The proposed project would displace the existing 128 space surface public parking lot located on the western portion of the project site. In/out counts conducted at this parking lot in October 2012 showed low entering and exiting vehicle trips (up to 16 vehicle trips [total of ins and outs]) during the analysis peak hours. Therefore, for the purposes of a conservative traffic analysis, the trips associated with this displaced parking demand were maintained on the surrounding blocks and were not reassigned to other public parking facilities in the study area.

Based on the parking demand analysis presented in Section G, “Parking Conditions Assessment,” the number of permitted accessory parking spaces would be insufficient to accommodate all the parking demand generated by the MSK ACC patients and visitors. Patients and visitors vehicle trips that would not be accommodated by the on-site accessory parking spaces would be required to drop off the patients first, followed by the driver circulating for off-site parking. The reverse trip-making pattern would occur during departure. Although a Special Permit is being requested to increase the number of accessory parking spaces up to 250 spaces, the as-of-right 166 accessory parking spaces were conservatively assumed for the MSK accessory parking garage so that the maximum number of patients would be required to be dropped off first resulting in the maximum amount of re-circulation. The vehicle trips associated with the re-circulation are reflected in the project-generated peak hour trip figures shown in Figures 9-1 to 9-3.

Traffic Operations

The Build condition traffic volumes are shown in Figures 9-17 to 9-19 for the weekday AM, midday, and PM peak hours. The Build condition traffic volumes were constructed by layering on top of the No Build volumes the project-generated trips shown in Figures 9-1 to 9-3. Table 9-11 shows an overall comparison of traffic levels of service for the No Build and Build conditions.

Based on the criteria presented in the CEQR Technical Manual and discussed previously in “Methodology and Impact Criteria,” significant adverse traffic impacts were identified and are denoted by the “+” symbol in the analysis summary table––Table 9-46 in Section H, “Detailed Analysis Results Tables.” The following section summarizes the identified significant adverse impacts.

Significant Adverse Impacts

Significant adverse traffic impacts were identified at 20 approaches/lane groups (of 11 different intersections). Potential measures that can be implemented to mitigate these significant adverse traffic impacts are discussed in Chapter 17, “Mitigation.”

| Table 9-11 Traffic Level of Service Summary Comparison: 2019 No Build vs. 2019 Build Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2019 No Build |  |  | 2019 Build |  |  |
|  | Weekday Peak Hours |  |  | Weekday Peak Hours |  |  |
|  | AM | Midday | PM | AM | Midday | PM |
| Intersections at Overall LOS A/B/C | 15 | 15 | 16 | 13 | 15 | 13 |
| Intersections at Overall LOS D | 1 | 4 | 0 | 3 | 1 | 3 |
| Intersections at Overall LOS E | 2 | 0 | 1 | 0 | 2 | 1 |
| Intersections at Overall LOS F | 1 | 0 | 2 | 3 | 1 | 2 |
| Number of intersections with significant impacts | - | - | - | 8 | 8 | 8 |
| Number of individual traffic lane groups at LOS E or F | 12 | 4 | 9 | 13 | 11 | 14 |
| Notes: Nineteen intersections were analyzed. * The number of individual traffic lane groups at LOS E or F is of 60 movements analyzed during the weekday AM peak hour, 59 movements analyzed during the weekday midday peak hour, and 61 movements analyzed during the weekday PM peak hour for the 2019 No Build condition. The number of individual traffic lane groups at LOS E or F is of 60 movements analyzed during the weekday AM, 59 movements analyzed during the weekday midday peak hour, and 63 movements analyzed during the weekday PM peak hour for the 2019 Build condition. |  |  |  |  |  |  |

York Avenue

- The eastbound approach at the intersection of York Avenue and East 79th Street would deteriorate within LOS F (from a v/c ratio of 1.06 and 95.5 spv of delay to a v/c ratio of 1.12 and 114.3 spv of delay), from LOS D (v/c ratio of 0.86 and 54.1 spv of delay) to LOS E (v/c ratio of 0.91 and 60.1 spv of delay), and within LOS F (from a v/c ratio of 1.02 and 82.5 spv of delay to a v/c ratio of 1.05 and 90.2 spv of delay), increases in delay of more than three, five, and three seconds, during the weekday AM, midday, and PM peak hours, respectively. These projected increases in delay constitute significant adverse impacts.

- The northbound approach at the intersection of York Avenue and East 79th Street would deteriorate within LOS F (from a v/c ratio of 1.13 and 107.2 spv of delay to a v/c ratio of 1.17 and 121.1 spv of delay), from LOS D (v/c ratio of 0.97 and 54.7 spv of delay) to LOS F (v/c ratio of 1.07 and 81.8 spv of delay), and within LOS F (from the northbound approach with a v/c ratio of 1.21 and 137.2 spv of delay to the northbound through/right-turn with a v/c ratio of 1.39 and 217.5 spv of delay), increases in delay of more than three, five, and three seconds during the weekday AM, midday, and PM peak hours, respectively. These projected increases in delay constitute significant adverse impacts.

- The northbound approach at the intersection of York Avenue and East 75th Street would deteriorate from LOS C (v/c ratio of 0.87 and 29.5 spv of delay) to LOS D (v/c ratio of 0.98 and 47.0 spv of delay), an increase in delay of more than five seconds, during the weekday midday peak hour. This projected increase in delay constitutes a significant adverse impact.

- The eastbound approach at the intersection of York Avenue and East 74th Street would deteriorate with LOS D (from a v/c ratio of 0.56 and 36.0 spv of delay to a v/c ratio of 0.76 and 46.6 spv of delay, from a v/c ratio of 0.59 and 36.7 spv of delay to a v/c ratio of 0.84 and 52.9 spv of delay, and from a v/c ratio of 0.59 and 36.9 spv of delay to a v/c ratio of 0.81 and 51.1 spv of delay), increases in delay of more than five seconds during the weekday AM, midday, and PM peak hours, respectively. These projected increases in delay constitute significant adverse impacts.

- The westbound approach at the intersection of York Avenue and East 74th Street would deteriorate from LOS C (v/c ratio of 0.09 and 26.0 spv of delay) to LOS F (v/c ratio of 1.03 and 111.2 spv of delay) and from LOS C (v/c ratio of 0.09 and 26.1 spv of delay) to LOS E (v/c ratio of 0.85 and 68.6 spv of delay), increases in delay of more than five seconds during the weekday midday and PM peak hours, respectively. These projected increases in delay constitute significant adverse impacts.

- The westbound approach at the intersection of York Avenue and East 73rd Street would deteriorate from LOS D (v/c ratio of 0.43 and 52.3 spv of delay) to LOS E (v/c ratio of 0.56 and 58.4 spv of delay), an increase in delay of more than five seconds during the weekday PM peak hour. This projected increase in delay constitutes a significant adverse impact.

- The northbound approach at the intersection of York Avenue and East 73rd Street would deteriorate from LOS E (v/c ratio of 1.04 and 71.4 spv of delay) to LOS F (v/c ratio of 1.17 and 119.9 spv of delay), from LOS D (v/c ratio of 0.89 and 40.3 spv of delay) to LOS E (v/c ratio of 1.00 and 61.5 spv of delay), and within LOS F (from a v/c ratio of 1.15 and 111.0 spv of delay to a v/c ratio of 1.28 and 164.9 spv of delay), increases in delay of more than four, five, and three seconds during the weekday AM, midday, and PM peak hours, respectively. These projected increases in delay constitute significant adverse impacts.

- The southbound defacto left-turn at the intersection of York Avenue and East 73rd Street would deteriorate within LOS F (from a v/c ratio of 1.00 and 81.9 spv of delay to a v/c ratio of 1.09 and 112.8 spv of delay) and within LOS F (from a v/c ratio of 1.17 and 137.4 spv of delay to a v/c ratio of 1.35 and 211.1 spv of delay), increases in delay of more than three seconds during the weekday AM and PM peak hours, respectively. These projected increases in delay constitute significant adverse impacts.

- The southbound approach at the intersection of York Avenue and East 73rd Street would deteriorate from LOS E (v/c ratio of 1.01 and 56.7 spv of delay) to LOS F (v/c ratio of 1.23 and 138.3 spv of delay), an increase in delay of more than four seconds during the weekday midday peak hour. This projected increase in delay constitutes a significant adverse impact.

- The southbound through/right-turn at the intersection of York Avenue and East 73rd Street would deteriorate from LOS E (v/c ratio of 1.06 and 75.4 spv of delay) to LOS F (v/c ratio of 1.16 and 113.2 spv of delay) and from LOS E (v/c ratio of 1.00 and 58.1 spv of delay) to LOS F (v/c ratio of 1.17 and 113.6 spv of delay), increases in delay of more than four seconds during the weekday AM and PM peak hours, respectively. These projected increases in delay constitute significant adverse impacts.

- The eastbound defacto left-turn at the intersection of York Avenue and East 72nd Street would deteriorate within LOS F (from a v/c ratio of 1.05 and 109.1 spv of delay to a v/c ratio of 1.13 and 136.1 spv of delay), from LOS D (v/c ratio of 0.73 and 50.7 spv of delay) to LOS E (v/c ratio of 0.81 and 59.9 spv of delay), and from LOS D (v/c ratio of 0.72 and 49.0 spv of delay) to LOS E (v/c ratio of 0.82 and 60.5 spv of delay), increases in delay of more than three, five, and five seconds during the weekday AM, midday, and PM peak hours, respectively. These projected increases in delay constitute significant adverse impacts.

- The northbound approach at the intersection of York Avenue and East 72nd Street would deteriorate within LOS F (from a v/c ratio of 1.09 and 83.5 spv of delay to a v/c ratio of 1.22 and 137.0 spv of delay), from LOS D (v/c ratio of 0.96 and 46.3 spv of delay) to LOS E (v/c ratio of 1.06 and 72.2 spv of delay), and within LOS F (from a v/c ratio of 1.10 and 86.0 spv of delay to a v/c ratio of 1.17 and 113.8 spv of delay), increases in delay of more than three, five, and three seconds during the weekday AM, midday, and PM peak hours, respectively. These projected increases in delay constitute significant adverse impacts.

- The northbound approach at the intersection of York Avenue and East 71st Street would deteriorate from LOS D (v/c ratio of 0.89 and 38.2 spv of delay) to LOS E (v/c ratio of 0.98 and 52.4 spv of delay), an increase in delay of more than five seconds during the weekday AM peak hour. This projected increase in delay constitutes a significant adverse impact.

- The northbound approach at the intersection of York Avenue and East 66th Street would deteriorate from LOS E (v/c ratio of 1.07 and 74.2 spv of delay) to LOS F (v/c ratio of 1.15 and 103.8 spv of delay), an increase in delay of more than four seconds during the weekday midday peak hour. This projected increase in delay constitutes a significant adverse impact.

- The southbound approach at the intersection of York Avenue and East 66th Street would deteriorate from LOS D (v/c ratio of 0.96 and 46.2 spv of delay) to LOS E (v/c ratio of 1.01 and 56.4 spv of delay), an increase in delay of more than five seconds during the weekday PM peak hour. This projected increase in delay constitutes a significant adverse impact.

- The eastbound approach at the intersection of York Avenue and East 65th Street would deteriorate within LOS F (from a v/c ratio of 1.03 and 97.6 spv of delay to a v/c ratio of 1.10 and 118.1 spv of delay), from LOS E (v/c ratio of 0.92 and 71.0 spv of delay) to LOS F (v/c ratio of 0.98 and 83.4 spv of delay), and within LOS F (from a v/c ratio of 1.10 and 118.3 spv of delay to a v/c ratio of 1.14 and 131.3 spv of delay), increases in delay of more than three, four, and three seconds during the weekday AM, midday, and PM peak hours, respectively. These projected increases in delay constitute significant adverse impacts.

- The westbound right-turn at the intersection of York Avenue and East 61st Street would deteriorate within LOS D (from a v/c ratio of 0.80 and 43.1 spv of delay to a v/c ratio of 0.88 and 50.5 spv of delay), an increase in delay of more than five seconds of delay during the weekday AM peak hour. This projected increase in delay constitutes a significant adverse impact.

First Avenue

- The eastbound approach at the intersection of First Avenue and East 72nd Street would deteriorate from LOS C (eastbound approach with a v/c ratio of 0.73 and 30.5 spv of delay) to LOS D (eastbound defacto left-turn with a v/c ratio of 0.77 and 46.8 spv of delay), an increase in delay of more than five seconds during the weekday PM peak hour. This projected increase in delay constitutes a significant adverse impact.

- The eastbound approach at the intersection of First Avenue and East 65th Street would deteriorate from LOS E (v/c ratio of 1.01 and 76.4 spv of delay) to LOS F (v/c ratio of 1.07 and 92.0 spv of delay), from LOS E (v/c ratio of 0.99 and 69.4 spv of delay) to LOS F (v/c ratio of 1.03 and 80.2 spv of delay), and within LOS F (from a v/c ratio of 1.10 and 101.8 spv of delay to a v/c ratio of 1.13 and 113.2 spv of delay), increases in delay of more than four, four, and three seconds during the weekday AM, midday, and PM peak hours, respectively. These projected increases in delay constitute significant adverse impacts.

- Detailed Transit Analysis

Mass transit options serving the study area, provided by NYCT, currently include the No. 6 subway line at the 68th Street-Hunter College subway station and the 77th Street subway station, and will include the Q subway line at the 72nd Street-Second Avenue subway station when Phase 1 construction of the station is complete in 2016. In addition, transit options include the M15, M31, M66, and M72 bus routes. These facilities are illustrated in Figure 9-5.

As discussed in Section B, “Preliminary Analysis Methodology and Screening Assessment,” an analysis of subway station elements at the planned 72nd Street subway station is warranted as is an analysis of subway line-haul on the Q line, and bus line-haul on the M66 and M72 bus routes. A bus line-haul analysis of the M15 and M31 bus routes is not warranted. The following sections are organized first by the subway line-haul analysis, followed by subway station analysis, and concluding with the bus analysis. It was determined that the peak load point for the planned Q line extension along Second Avenue would be in the southbound direction during the AM peak hour at the Second Avenue and 63rd Street station. The AM load leaving this station will represent the critical load point, and therefore only the AM southbound peak is analyzed for the Q subway line. For the other study area transit facilities, a detailed analysis of transit operations during the critical weekday AM and PM peak periods is presented below. During other time periods, background transit ridership and station utilization, as well as project trip generation, are comparatively lower. Hence, potential transit impacts were evaluated only for the weekday AM and PM peak periods.

subway line-haul analysis

subway LINE-HAUL ANALYSIS—Methodology and Impact Criteria

As per the CEQR Technical Manual, line-haul capacities of a subway line are evaluated at its maximum load point and/or other project-specific load points to determine if the route’s guideline (or practical) capacity would be exceeded. NYCT operates six different types of subway cars with different seating and guideline capacities. The peak period guideline capacity of a subway car, which ranges from 110 to 175 passengers, is compared with ridership levels to determine the acceptability of conditions. Projected increases from the No Build condition within guideline capacity to a Build condition that exceeds guideline capacity may be a significant adverse impact if the proposed project is expected to generate five more transit riders per car.

SUBWAY line-haul ANALYSIS—2012 EXISTING Conditions

The planned Q train extension along Second Avenue was analyzed. This line is currently under construction, and is described below for the future No Build and Build conditions.

SUBWAY line-haul ANALYSIS—2019 no build condition

As discussed in Section B, “Preliminary Analysis Methodology and Screening Assessment,” it was determined that the peak load point for the planned Q line extension along Second Avenue would be in the southbound direction during the AM peak hour at the Second Avenue and 63rd Street station. The AM load leaving this station will represent the critical load point, and therefore only the AM southbound peak is analyzed for the Q subway line. Projected subway ridership numbers for the Second Avenue subway Q line at its 2016 opening were provided by NYCT. These numbers were adjusted to 2019 levels using an annual background growth rate of 0.25 percent. Furthermore, trips associated with major new developments along the Q subway line were superimposed onto the 2019 background line-haul volumes to generate No Build peak period volumes for the subway line-haul analysis.

As shown in Table 9-12, in the No Build condition, the Q train is projected to operate within guideline capacity during the weekday AM peak period for the Manhattan southbound service.

| Table 9-12 2019 No Build Condition: AM Peak Hour Subway Line Haul |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Q Train Direction of Travel | Station | Trains/Hour | Cars/ Train | Volume | Leave Load |  | Available Capacity per Car |
|  |  |  |  |  | Guideline Capacity | Total Persons per Car |  |
| AM Peak Period |  |  |  |  |  |  |  |
| Southbound | Second Avenue and | 12 | 10 | 16,909 | 145 | 140.91 | 4.09 |
| Manhattan | 63rd Street |  |  |  |  |  |  |
| Sources: New York City Transit |  |  |  |  |  |  |  |

SUBWAY line-haul ANALYSIS—build (2019) condition

Trips associated with the proposed project were superimposed onto the No Build line-haul volumes to generate the Build peak period volumes for the subway line-haul analysis. As shown in Table 9-13, as with the 2019 No Build condition, the Manhattan southbound Q service would continue to operate within guideline capacity during the weekday AM peak period under the 2019 Build condition. Hence, the proposed project would not result in a significant adverse impact on subway line-haul conditions.

| Table 9-13 2019 Build Condition: AM Peak Hour Subway Line Haul |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q Train Direction of Travel | Station | Trains/Hour | Cars/ Train | Volume | Leave Load |  | Available Capacity per Car |  |
|  |  |  |  |  | Guideline Capacity | Total Persons per Car |  |  |
| AM Peak Period |  |  |  |  |  |  |  |  |
| Southbound | Second Avenue and | 12 | 10 | 17,039 | 145 | 141.99 | 3.01 |  |
| Manhattan | 63rd Street |  |  |  |  |  |  |  |
| Sources: New York City Transit |  |  |  |  |  |  |  |  |

Subway Station Analysis

SUBWAY STATION ANALYSIS—Methodology and Impact Criteria

As detailed in Section B, “Preliminary Analysis Methodology and Screening Assessment,” it was determined that an analysis of subway station elements (i.e., fare control areas and station circulation elements such as escalators and elevators) is warranted for the planned 72nd Street subway station. The methodology for assessing station circulation (escalators) and fare control elements (regular turnstiles) compares the user volume with the analyzed element’s design capacity, resulting in a volume-to-capacity (v/c) ratio. For elevators, service levels are assessed by their ability to process passengers on a minute-by-minute basis.

For escalators and turnstiles, capacities are measured by the number and width of an element and the NYCT optimum capacity per element, also account for the potential for surging of exiting pedestrians (up to 25 percent capacity reduction is applied to account for detraining surges near platforms). The estimated v/c ratio is compared with NYCT criteria to determine a LOS for the operation of an element, as summarized in Table 9-14. At LOS A (“free flow”) and B (“fluid flow”), there is sufficient area to allow pedestrians to freely select their walking speed and bypass slower pedestrians. When cross and reverse flow movement exists, only minor conflicts may occur. At LOS C (“fluid, somewhat restricted”), movement is fluid although somewhat restricted. While there is sufficient room for standing without personal contact, circulation through queuing areas may require adjustments to walking speed. At LOS D (“crowded, walking speed restricted”), walking speed is restricted and reduced. Reverse and cross flow movement is severely restricted because of congestion and the difficult passage of slower moving pedestrians. At LOS E (“congested, some shuffling and queuing”) and F (“severely congested, queued”), walking speed is restricted. There is also insufficient area to bypass others, and opposing movement is difficult. Often, forward progress is achievable only through shuffling, with queues forming.

| Table 9-14 LOS Criteria for Non-Elevator Subway Station Elements |  |
| --- | --- |
| LOS | V/C Ratio |
| A | 0.00 to 0.45 |
| B | 0.45 to 0.70 |
| C | 0.70 to 1.00 |
| D | 1.00 to 1.33 |
| E | 1.33 to 1.67 |
| F | Above 1.67 |
| Source: New York City Mayor's Office of Environmental Coordination, CEQR Technical Manual (February 2012). |  |

As per NYCT guidance, for elevators, the design capacity considers the number of elevators available at street level and at platform or mezzanine level for each minute in a 15-minute period, the interior cab size, and maximum capacity of each elevator, travel time to and from the platform or mezzanine, and surging of entering and exiting pedestrians.

The determination of significant impacts for escalators and control area elements is based on the projected v/c ratios. Impacts are significant if the proposed project causes a v/c ratio to increase from below 1.00 to 1.00 or greater. Where a facility is already at or above its capacity (a v/c of 1.00 or greater) in the No Build condition, a 0.01 increase in v/c ratio is also significant.

The impact of the proposed project on elevator capacity is described qualitatively, as per NYCT guidance. The qualitative discussion consists of a comparison of the number of passengers left waiting each minute and the number of elevator cycles and the total time that is required to clear the passenger surges during peak AM 15-minute period in the existing, No Build, and Build conditions.

Subway Station study area

Below is a summary of subway lines that would most likely serve the project site, as shown in Figure 9-5.

- The No. 6 subway line (Lexington Avenue Local) operates between Pelham Bay Park, Queens and Brooklyn Bridge – City Hall, Manhattan, at all times.

- The Q subway line, when the Second Avenue Subway phase 1 construction is completed in 2016, will operate between 96th Street and Second Avenue, Manhattan and Coney Island – Stillwell Avenue, Brooklyn, via Broadway in Manhattan.

SUBWAY STATION ANALYSIS—2012 EXISTING Conditions

A planned subway station located at East 72nd Street and Second Avenue was analyzed. This station is currently under construction, and is described below for the future No Build and Build conditions.

SUBWAY STATION ANALYSIS—future without the proposed project (2019 No Build Condition)

As mentioned above, there is a planned 72nd Street subway station on the Second Avenue Subway that will become operational by 2016, prior to the proposed project’s analysis year. At the project’s analysis year, the planned station will be served by the Q line along Second Avenue and will have entrances at the intersections of East 69th Street and Second Avenue and East 72nd Street and Second Avenue. The planned station access elements to be analyzed are a bank of five elevators on the southeast corner of Second Avenue and East 72nd Street, three escalators on the northwest corner of the intersection, and corresponding fare control elements. NYCT has designed the station and its access and fare control elements to accommodate their projections of future demand.

Projected subway ridership numbers and station element designs for the 72nd Street subway station at its 2016 opening were provided by NYCT. Ridership volumes were adjusted to 2019 levels using an annual background growth rate of 0.25 percent. In addition, trips associated with major new developments along the Q subway line were incorporated into the No Build station volumes. In accordance with NYCT guidance, in addition to the surging factor for exiting patrons, a surging factor of 0.9 was applied for turnstile entries to account for mini-surges of customers exiting elevators at the mezzanine.

As per NYCT guidance, the elevators at the 72nd Street subway station were assumed to have a capacity of 16 riders each. The elevator analysis assumed all 5 elevators would be operational at the same time. Passenger surges into elevators at street level were determined using previous NYCT station analyses, which showed that 6 percent of entries during peak hours would cross Second Avenue just before reaching the elevators, 22 percent would cross 72nd Street, and 72 percent would not cross either street to access the elevators. Platform level ridership surges into elevators were determined using the projected average number of Q trains per hour.

As shown in Tables 9-15 through 9-17, all station escalators and control area elements will operate at acceptable levels during the weekday AM and PM peak periods. Elevators on the street level would clear the street of waiting riders in approximately 1 minute, and would have 9 waiting riders at the end of the AM 15-minute peak. Elevators on the platform level would clear the platform of waiting riders in approximately 2 minutes, and would have zero waiting riders at the end of the AM 15-minute peak.

| Table 9-15 2019 No Build Condition: Subway Escalator Analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Second Avenue – 72nd Street (Q Line) Station | Quantity | Tread Width (in.) | Capacity (people/minute) | Surge Factor Exiting | 1-Hour Pedestrian Volume |  | Peak 15-Min. Capacity (w/o Surge) | V/C ratio | LOS |
|  |  |  |  |  | Up | Down |  |  |  |
| AM Peak Period |  |  |  |  |  |  |  |  |  |
| Escalator - Down | 1 | 40 | 70 | 0.80 | 0 | 1,049 | 1,050 | 0.39 | A |
| Escalator - Up | 2 | 40 | 70 | 0.80 | 2,286 | 0 | 2,100 | 0.43 | A |
| PM Peak Period |  |  |  |  |  |  |  |  |  |
| Escalator - Down | 1 | 40 | 70 | 0.80 | 0 | 1,673 | 1,050 | 0.62 | B |
| Escalator - Up | 2 | 40 | 70 | 0.80 | 621 | 0 | 2,100 | 0.12 | A |
| Notes: |  |  |  |  |  |  |  |  |  |
| Capacities were calculated based on rates presented in the 2012 CEQR Technical Manual. |  |  |  |  |  |  |  |  |  |
| Surging factors are only applied to the exiting pedestrian volume (2012 CEQR Technical Manual). |  |  |  |  |  |  |  |  |  |
| V/C = V/ GCap* Sf |  |  |  |  |  |  |  |  |  |
| Where |  |  |  |  |  |  |  |  |  |
| V = Peak 15-minute passenger volume |  |  |  |  |  |  |  |  |  |
| GCap = Guideline Capacity for the escalator |  |  |  |  |  |  |  |  |  |
| Sf = Surging factor (if applicable) |  |  |  |  |  |  |  |  |  |

| Table 9-16 2019 No Build Condition: AM Peak Hour Subway Elevator Analysis |  |  |  |
| --- | --- | --- | --- |
| Minute | Arrival Surge Volumes | Elevator Capacity | Waiting Pedestrians |
| Second Avenue – 72nd Street Station (Q Line) – Street Level Elevators |  |  |  |
| 1 | 41 | 32 | 9 |
| 2 | 39 | 48 | 0 |
| 3 | 41 | 32 | 9 |
| 4 | 39 | 48 | 0 |
| 5 | 41 | 32 | 9 |
| 6 | 39 | 48 | 0 |
| 7 | 41 | 32 | 9 |
| 8 | 39 | 48 | 0 |
| 9 | 41 | 32 | 9 |
| 10 | 39 | 48 | 0 |
| 11 | 41 | 32 | 9 |
| 12 | 39 | 48 | 0 |
| 13 | 41 | 32 | 9 |
| 14 | 39 | 48 | 0 |
| 15 | 41 | 32 | 9 |
| Second Avenue – 72nd Street Station (Q Line) – Mezzanine Level Elevators |  |  |  |
| 1 | 80 | 48 | 32 |
| 2 | 0 | 32 | 0 |
| 3 | 0 | 48 | 0 |
| 4 | 0 | 32 | 0 |
| 5 | 0 | 48 | 0 |
| 6 | 0 | 32 | 0 |
| 7 | 104 | 48 | 56 |
| 8 | 0 | 32 | 24 |
| 9 | 0 | 48 | 0 |
| 10 | 0 | 32 | 0 |
| 11 | 0 | 48 | 0 |
| 12 | 0 | 32 | 0 |
| 13 | 0 | 48 | 0 |
| 14 | 67 | 32 | 35 |
| 15 | 0 | 48 | 0 |

| Table 9-17 2019 No Build Condition: Subway Control Area Analysis |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station Elements | Qty. | 1-Hour Pedestrian Volumes |  | Surging Factor – Entering | Surging Factor - Exiting | Friction Factor | V/C Ratio | LOS |
|  |  | Into Control Area | Out from Control Area |  |  |  |  |  |
| AM Peak Hour |  |  |  |  |  |  |  |  |
| Second Avenue – 72nd Street Station (Q Line) |  |  |  |  |  |  |  |  |
| Two-Way Turnstiles | 8 | 2,760 | 3,089 | 0.90 | 0.80 | 0.90 | 0.58 | B |
| PM Peak Hour |  |  |  |  |  |  |  |  |
| Second Avenue – 72nd Street Station (Q Line) |  |  |  |  |  |  |  |  |
| Two-Way Turnstiles | 8 | 2,261 | 1,636 | 0.90 | 0.80 | 0.90 | 0.40 | A |
| Notes: Capacities were calculated based on rates presented in the CEQR Technical Manual. V/C =Vin / (Cin x Sfin x Ff)+ Vx / (Cx x Sfout x Ff), where Vin = Peak 15 Min Entering Passenger Volume Cin= Total 15-Minute Capacity of all turnstiles for entering Passengers Vx = Peak 15- Minute Exiting Passenger Cx = Total 15-minute Capacity of all turnstile for exiting Passengers Sfin = Surging Factor Entering Passenger Sfout = Surging Factor Exiting Passenger Ff = Friction Factor |  |  |  |  |  |  |  |  |

SUBWAY STATION ANALYSIS—probable impacts of the proposed project (2019 Build Condition)

As shown in Table 9-5, the incremental subway trips with the proposed project were projected to be 786 (770 in and 16 out) during the weekday AM peak hour and 730 (200 in and 530 out) during the weekday PM peak hour. As discussed above in Section B, all of the total project-generated subway trips are expected to be served by the three nearest subway stations––the 68th Street (No. 6) subway station, the 77th Street (No. 6) subway station, and the planned 72nd Street (Q) subway station. The planned subway station located at East 72nd Street and Second Avenue was analyzed. The combination of the project-generated subway trips with the 2019 No Build volumes would result in the 2019 Build condition. As in the No Build analysis, in addition to the surging factor for exiting patrons, a surging factor of 0.9 was applied for turnstile entries to account for mini-surges of customers exiting elevators at the mezzanine.

As shown in Tables 9-18 through 9-20, all station escalators and control elements would continue to operate at acceptable levels during the weekday AM and PM peak periods. Elevators on the street level would clear the street of waiting riders in approximately 1 minute, and would have 10 waiting riders at the end of the AM 15-minute peak. Elevators on the platform level would clear the platform of waiting riders in approximately 2 minutes, and would have zero waiting riders at the end of the AM 15-minute peak. These operating conditions are similar to those in the No Build condition. Therefore, the proposed project would not result in any significant adverse impacts to the analyzed subway station elements.

| Table 9-18 2019 Build Condition: Subway Escalator Analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Second Avenue – 72nd Street (Q Line) Station | Quantity | Tread Width (in.) | Capacity (people/minute) | Surge Factor Exiting | 1-Hour Pedestrian Volume |  | Peak 15-Min. Capacity (w/o Surge) | V/C ratio | LOS |
|  |  |  |  |  | Up | Down |  |  |  |
| AM Peak Period |  |  |  |  |  |  |  |  |  |
| Escalator - Down | 1 | 40 | 70 | 0.80 | 0 | 1,053 | 1,050 | 0.39 | A |
| Escalator - Up | 2 | 40 | 70 | 0.80 | 2,656 | 0 | 2,100 | 0.49 | B |
| PM Peak Period |  |  |  |  |  |  |  |  |  |
| Escalator - Down | 1 | 40 | 70 | 0.80 | 0 | 1,927 | 1,050 | 0.72 | C |
| Escalator - Up | 2 | 40 | 70 | 0.80 | 671 | 0 | 2,100 | 0.13 | A |
| Notes: |  |  |  |  |  |  |  |  |  |
| Capacities were calculated based on rates presented in the 2012 CEQR Technical Manual. |  |  |  |  |  |  |  |  |  |
| Surging factors are only applied to the exiting pedestrian volume (2012 CEQR Technical Manual). |  |  |  |  |  |  |  |  |  |
| V/C = V/ GCap* Sf |  |  |  |  |  |  |  |  |  |
| Where |  |  |  |  |  |  |  |  |  |
| V = Peak 15-minute passenger volume |  |  |  |  |  |  |  |  |  |
| GCap = Guideline Capacity for the escalator |  |  |  |  |  |  |  |  |  |
| Sf = Surging factor (if applicable) |  |  |  |  |  |  |  |  |  |

| Table 9-19 2019 Build Condition: AM Peak Hour Subway Elevator Analysis |  |  |  |
| --- | --- | --- | --- |
| Minute | Arrival Surge Volumes | Elevator Capacity | Waiting Pedestrians |
| Second Avenue – 72nd Street Station (Q Line) – Street Level Elevators |  |  |  |
| 1 | 42 | 32 | 10 |
| 2 | 40 | 48 | 0 |
| 3 | 42 | 32 | 10 |
| 4 | 40 | 48 | 0 |
| 5 | 42 | 32 | 10 |
| 6 | 40 | 48 | 0 |
| 7 | 42 | 32 | 10 |
| 8 | 40 | 48 | 0 |
| 9 | 42 | 32 | 10 |
| 10 | 40 | 48 | 0 |
| 11 | 42 | 32 | 10 |
| 12 | 40 | 48 | 0 |
| 13 | 42 | 32 | 10 |
| 14 | 40 | 48 | 0 |
| 15 | 42 | 32 | 10 |
| Second Avenue – 72nd Street Station (Q Line) – Mezzanine Level Elevators |  |  |  |
| 1 | 93 | 48 | 45 |
| 2 | 0 | 32 | 13 |
| 3 | 0 | 48 | 0 |
| 4 | 0 | 32 | 0 |
| 5 | 0 | 48 | 0 |
| 6 | 0 | 32 | 0 |
| 7 | 121 | 48 | 73 |
| 8 | 0 | 32 | 41 |
| 9 | 0 | 48 | 0 |
| 10 | 0 | 32 | 0 |
| 11 | 0 | 48 | 0 |
| 12 | 0 | 32 | 0 |
| 13 | 0 | 48 | 0 |
| 14 | 78 | 32 | 46 |
| 15 | 0 | 48 | 0 |

| Table 9-20 2019 Build Condition: Subway Control Area Analysis |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station Elements | Qty. | 1-Hour Pedestrian Volumes |  | Surging Factor – Entering | Surging Factor - Exiting | Friction Factor | V/C Ratio | LOS |
|  |  | Into Control Area | Out from Control Area |  |  |  |  |  |
| AM Peak Hour |  |  |  |  |  |  |  |  |
| Second Avenue – 72nd Street Station (Q Line) |  |  |  |  |  |  |  |  |
| Two-Way Turnstiles | 8 | 2,770 | 3,590 | 0.90 | 0.80 | 0.90 | 0.62 | B |
| PM Peak Hour |  |  |  |  |  |  |  |  |
| Second Avenue – 72nd Street Station (Q Line) |  |  |  |  |  |  |  |  |
| Two-Way Turnstiles | 8 | 2,605 | 1,766 | 0.90 | 0.80 | 0.90 | 0.45 | A |
| Notes: Capacities were calculated based on rates presented in the CEQR Technical Manual. V/C =Vin / (Cin x Sfin x Ff)+ Vx / (Cx x Sfout x Ff), where Vin = Peak 15 Min Entering Passenger Volume Cin= Total 15-Minute Capacity of all turnstiles for entering Passengers Vx = Peak 15- Minute Exiting Passenger Cx = Total 15-minute Capacity of all turnstile for exiting Passengers Sfin = Surging Factor Entering Passenger Sfout = Surging Factor Exiting Passenger Ff = Friction Factor |  |  |  |  |  |  |  |  |

Bus line-haul Analysis

BUS LINE-HAUL ANALYSIS—Methodology and Impact Criteria

As detailed in Section B, “Preliminary Analysis Methodology and Screening Assessment,” it was determined that a bus line-haul analysis of the M66 and M72 bus routes is warranted. The assessment of bus line-haul conditions involves analyzing bus routes at their peak load points and, if necessary, also their bus stops closest to the project site to identify the potential for the analyzed routes to exceed their guideline (or practical) capacities. NYCT and the MTA Bus Company operate three types of buses: standard and articulated buses, and over-the-road coaches. During peak hours, standard buses operate with up to 54 passengers per bus, articulated buses operate with up to 85 passengers per bus, and over-the-road coaches operate with up to 55 passengers per bus.

An increase in bus load levels greater than the maximum capacity at any load point is defined as a significant adverse impact. While subject to operational and fiscal constraints, bus impacts can typically be mitigated by increasing service frequency. Therefore, mitigation of bus line-haul capacity impacts, where appropriate, would be recommended for NYCT’s approval.

BUS LINE-HAUL ANALYSIS—2012 Existing Conditions

As stated above, a bus line-haul analysis is provided for the M66 and M72 bus routes but is not warranted for the M15 and M31 bus routes. The M15 and M31 bus routes are described in this section for informational purposes only. Table 9-21 provides information on the NYCT local bus routes that serve the study area. Table 9-22 provides existing operating characteristics and peak load point and local ridership volumes on the M66 and M72 bus routes.

For both the M66 and M72 bus routes, a detailed allocation of incremental bus riders onto specific segments of each bus route was performed to determine the appropriate bus stop locations for the line-haul analysis. The M66 peak load points correspond well with where the project-generated trips would traverse, while the M72 peak load points are recorded at bus stops west of where project-generated trips would have an effect on ridership levels. Therefore, original bus ridership data were collected at bus stops closer to the project site in January 2013 to more accurately reflect the proposed project’s effects on the M72 bus route. As shown, both routes currently operate within the guideline capacity during the weekday AM and PM peak periods.

| Table 9-21 NYCT Local Bus Routes Serving The Study Area |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Bus Route | Start Point | End Point | Routing in Study Area | Freq. of Bus Service (Headway in Minutes) |  |
|  |  |  |  | AM | PM |
| M15/SBS (NB/SB) | Second Avenue and E. 126th Street | South Ferry | First Avenue/Second Avenue | (14/8): M15 (7/4): SBS | (17/10): M15 (5/7): SBS |
| M31 (NB/SB) | E. 92nd Street and York Avenue | Eleventh Avenue and W. 54th Street | York Avenue | (9/6) | (9/9) |
| M66 (EB/WB) | E. 67th Street and York Avenue | W. 66th Street and West End Avenue | York Avenue/E. 68th Street | (5/20) | (5/8) |
| M72 (EB/WB) | E. 72nd Street and York Avenue | W. 66th Street and Freedom Place | York Avenue/E. 72nd Street | (8/7) | (10/9) |
| Source: MTA Bus Timetables (2012). |  |  |  |  |  |

| Table 9-22 2012 Existing Conditions: Bus Line-Haul Analysis |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Route | Direction | Load Point | Hourly Volumes | Buses/ Hour | AP |
| AM Peak Hour |  |  |  |  |  |
| M66 | East | East 68th Street and Lexington Avenue | 788 | 16 | 49 |
|  | West | East 67th Street and Lexington Avenue | 423 | 12 | 35 |
| M72 | East | West 72nd Street East of Second Avenue | 174 | 6 | 29 |
|  | West | East 72nd Street West of First Avenue | 372 | 8 | 47 |
| PM Peak Hour |  |  |  |  |  |
| M66 | East | East 68th Street and Lexington Avenue | 346 | 9 | 39 |
|  | West | East 67th Street and Lexington Avenue | 712 | 17 | 42 |
| M72 | East | West 72nd Street East of Second Avenue | 93 | 6 | 16 |
|  | West | East 72nd Street West of First Avenue | 276 | 7 | 40 |
| Notes: AP=average passengers per bus Sources: M66 and M72 Maximum Load Point data from NYCT; M72 ridership data collected in January 2013 by AKRF, Inc. |  |  |  |  |  |

BUS LINE-HAUL ANALYSIS—Future without the Proposed Project (2019 No Build Condition)

Estimates of peak hour bus volumes in the No Build condition were developed by applying the CEQR Technical Manual recommended annual background growth rates as previously described. In addition, trips associated with No Build projects were incorporated into the future No Build bus volumes.

As shown in Table 9-23, the M66 and M72 bus routes would continue to operate within their guideline capacities during the weekday AM and PM peak periods, though the eastbound M66 route is nearing guideline capacity

| Table 9-23 2019 No Build Condition: Bus Line-Haul Analysis |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Route | Direction | Load Point | Hourly Volumes | Buses/ Hour | AP | Bus Demand at Guideline Capacity |
| AM Peak Hour |  |  |  |  |  |  |
| M66 | East | East 68th Street and Lexington Avenue | 839 | 16 | 52 | 16 |
|  | West | East 67th Street and Lexington Avenue | 430 | 12 | 36 | 8 |
| M72 | East | West 72nd Street East of Second Avenue | 197 | 6 | 33 | 4 |
|  | West | East 72nd Street West of First Avenue | 379 | 8 | 47 | 8 |
| PM Peak Hour |  |  |  |  |  |  |
| M66 | East | East 68th Street and Lexington Avenue | 358 | 9 | 40 | 7 |
|  | West | East 67th Street and Lexington Avenue | 765 | 17 | 45 | 15 |
| M72 | East | West 72nd Street East of Second Avenue | 101 | 6 | 17 | 2 |
|  | West | East 72nd Street West of First Avenue | 305 | 7 | 44 | 6 |
| Notes: AP=average passengers per bus Sources: M66 and M72 Maximum Load Point data from NYCT; M72 ridership data collected in January 2013 by AKRF, Inc. |  |  |  |  |  |  |

BUS LINE-HAUL ANALYSIS—probable impacts of the Proposed project (2019 Build Condition)

Peak period bus ridership for the Build condition was generated by adding the incremental trips associated with the proposed project to the No Build bus line-haul volumes. As described above, impacts on bus line-haul levels are considered significant if a proposed project would result in operating conditions above guideline capacities. As shown in Table 9-24, the M66 and M72 bus routes would continue to operate within the guideline capacity, and therefore the proposed project would not result in any significant adverse impacts to bus line-haul capacities.

| Table 9-24 2019 Build Condition: Bus Line-Haul Analysis |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Route | Direction | Load Point | Hourly Volumes | Buses/ Hour | AP | Bus Demand at Guideline Capacity |
| AM Peak Hour |  |  |  |  |  |  |
| M66 | East | East 68th Street and Lexington Avenue | 851 | 16 | 54 | 16 |
|  | West | East 67th Street and Lexington Avenue | 430 | 12 | 36 | 8 |
| M72 | East | West 72nd Street East of Second Avenue | 242 | 6 | 41 | 5 |
|  | West | East 72nd Street West of First Avenue | 379 | 8 | 48 | 8 |
| PM Peak Hour |  |  |  |  |  |  |
| M66 | East | East 68th Street and Lexington Avenue | 397 | 9 | 45 | 8 |
|  | West | East 67th Street and Lexington Avenue | 774 | 17 | 46 | 15 |
| M72 | East | West 72nd Street East of Second Avenue | 151 | 6 | 26 | 3 |
|  | West | East 72nd Street West of First Avenue | 337 | 7 | 49 | 7 |
| Notes: AP=average passengers per bus Sources: M66 and M72 Maximum Load Point data from NYCT; M72 ridership data collected in January 2013 by AKRF, Inc. |  |  |  |  |  |  |

- Detailed Pedestrian Analysis

As described above in Section B, “Preliminary Analysis Methodology and Screening Assessment,” the Level 1 and Level 2 screening analyses indicated a need for a detailed analysis of 12 sidewalk, 23 corner reservoir, and 11 crosswalk locations in the weekday AM, midday, and PM peak periods. This section begins with an overview of the methodology and impact criteria for pedestrian analyses, and then provides information on existing conditions and the future without the proposed project. This section ends with a comparison of the future without the proposed project to the future with the proposed project for the purposes of identifying where the project has the potential to result in significant adverse pedestrian impacts.

Methodology and Impact Criteria

The adequacy of the study area’s sidewalks, crosswalks, and corner reservoir capacities in relation to the demand imposed on them is evaluated based on the methodologies presented in the 2010 HCM, pursuant to procedures detailed in the CEQR Technical Manual.

Sidewalks are analyzed in terms of pedestrian flow. The calculation of the average pedestrians per minute per foot (PMF) of effective walkway width is the basis for a sidewalk LOS analysis. The determination of walkway LOS is also dependent on whether the pedestrian flow being analyzed is best described as “non-platoon” or “platoon.” Non-platoon flow occurs when pedestrian volume within the peak 15-minute period is relatively uniform, whereas, platoon flow occurs when pedestrian volumes vary significantly with the peak 15-minute period. Such variation typically occurs near bus stops, subway stations, and/or where adjacent crosswalks account for much of the walkway’s pedestrian volume.

Crosswalks and street corners are not easily measured in terms of free pedestrian flow, as they are influenced by the effects of traffic signals. Street corners must be able to provide sufficient space for a mix of standing pedestrians (queued to cross a street) and circulating pedestrians (crossing the street or moving around the corner). The HCM methodologies apply a measure of time and space availability based on the area of the corner, the timing of the intersection signal, and the estimated space used by circulating pedestrians.

The total “time-space” available for these activities, expressed in sf-second, is calculated by multiplying the net area of the corner (in sf) by the signal’s cycle length. The analysis then determines the total circulation time for all pedestrian movements at the corner per signal cycle (expressed as pedestrians per second). The ratio of net time-space divided by the total pedestrian circulation volume per signal cycle provides the LOS measurement of sf per pedestrian (SFP).

Crosswalk LOS is also a function of time and space. Similar to the street corner analysis, crosswalk conditions are first expressed as a measurement of the available area (the crosswalk width multiplied by the width of the street) and the permitted crossing time. This measure is expressed in sf-second. The average time required for a pedestrian to cross the street is calculated based on the width of the street and an assumed walking speed. The ratio of time-space available in the crosswalk to the total crosswalk pedestrian occupancy time is the LOS measurement of available sf per pedestrian. The LOS analysis also accounts for vehicular turning movements that traverse the crosswalk. The LOS standards for sidewalks, corner reservoirs, and crosswalks are summarized in Table 9-25. The CEQR Technical Manual specifies acceptable LOS in Central Business District (CBD) areas is mid-LOS D or better.

The determination of significant pedestrian impacts considers the level of predicted deterioration in pedestrian flow or decrease in pedestrian space between the No Build and Build conditions. For different pedestrian elements, flow conditions, and area types, the CEQR procedure for impact determination corresponds with various sliding-scale formulas, as further detailed below.

| Table 9-25 Level of Service Criteria for Pedestrian Elements |  |  |  |
| --- | --- | --- | --- |
| LOS | Sidewalks |  | Corner Reservoirs and Crosswalks |
|  | Non-Platoon Flow | Platoon Flow |  |
| A |  5 PMF |  0.5 PMF | > 60 SFP |
| B | > 5 and  7 PMF | > 0.5 and  3 PMF | > 40 and  60 SFP |
| C | > 7 and  10 PMF | > 3 and  6 PMF | > 24 and  40 SFP |
| D | > 10 and  15 PMF | > 6 and  11 PMF | > 15 and  24 SFP |
| E | > 15 and  23 PMF | > 11 and  18 PMF | > 8 and  15 SFP |
| F | > 23 PMF | > 18 PMF |  8 SFP |
| Notes: PMF = pedestrians per minute per foot; SFP = square feet per pedestrian. Source: New York City Mayor’s Office of Environmental Coordination, CEQR Technical Manual. |  |  |  |

There are two sliding-scale formulas for determining significant sidewalk impacts. For non-platoon flow, the increase in average pedestrian flow rate (Y) in PMF needs to be greater or equal to 3.5 minus X divided by 8.0 (where X is the No Build pedestrian flow rate in PMF [Y  3.5 – X/8.0]) for it to be a significant impact. For platoon flow, the sliding-scale formula is Y  3.0 – X/8.0. Since deterioration in pedestrian flow within acceptable levels would not constitute a significant impact, these formulas would apply only if the Build pedestrian flow exceeds LOS C in non-CBD areas or mid-LOS D in CBD areas. Table 9-26 summarizes the sliding scale guidance provided by the CEQR Technical Manual for determining potential significant sidewalk impacts.

| Table 9-26 Significant Impact Guidance for Sidewalks |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Platoon Flow |  |  |  | Platoon Flow |  |  |  |
| Sliding Scale Formula: Y  3.5 – X/8.0 |  |  |  | Sliding Scale Formula: Y  3.0 – X/8.0 |  |  |  |
| Non-CBD Areas |  | CBD Areas |  | Non-CBD Areas |  | CBD Areas |  |
| No Build Ped. Flow (X, PMF) | Build Ped. Flow Incr. (Y, PMF) | No Build Ped. Flow (X, PMF) | Build Ped. Flow Incr. (Y, PMF) | No Build Ped. Flow (X, PMF) | Build Ped. Flow Incr. (Y, PMF) | No Build Ped. Flow (X, PMF) | Build Ped. Flow Incr. (Y, PMF) |
| 7.4 to 7.8 |  2.6 | – | – | 3.4 to 3.8 |  2.6 | – | – |
| 7.9 to 8.6 |  2.5 | – | – | 3.9 to 4.6 |  2.5 | – | – |
| 8.7 to 9.4 |  2.4 | – | – | 4.7 to 5.4 |  2.4 | – | – |
| 9.5 to 10.2 |  2.3 | – | – | 5.5 to 6.2 |  2.3 | – | – |
| 10.3 to 11.0 |  2.2 | 10.3 to 11.0 |  2.2 | 6.3 to 7.0 |  2.2 | 6.3 to 7.0 |  2.2 |
| 11.1 to 11.8 |  2.1 | 11.1 to 11.8 |  2.1 | 7.1 to 7.8 |  2.1 | 7.1 to 7.8 |  2.1 |
| 11.9 to 12.6 |  2.0 | 11.9 to 12.6 |  2.0 | 7.9 to 8.6 |  2.0 | 7.9 to 8.6 |  2.0 |
| 12.7 to 13.4 |  1.9 | 12.7 to 13.4 |  1.9 | 8.7 to 9.4 |  1.9 | 8.7 to 9.4 |  1.9 |
| 13.5 to 14.2 |  1.8 | 13.5 to 14.2 |  1.8 | 9.5 to 10.2 |  1.8 | 9.5 to 10.2 |  1.8 |
| 14.3 to 15.0 |  1.7 | 14.3 to 15.0 |  1.7 | 10.3 to 11.0 |  1.7 | 10.3 to 11.0 |  1.7 |
| 15.1 to 15.8 |  1.6 | 15.1 to 15.8 |  1.6 | 11.1 to 11.8 |  1.6 | 11.1 to 11.8 |  1.6 |
| 15.9 to 16.6 |  1.5 | 15.9 to 16.6 |  1.5 | 11.9 to 12.6 |  1.5 | 11.9 to 12.6 |  1.5 |
| 16.7 to 17.4 |  1.4 | 16.7 to 17.4 |  1.4 | 12.7 to 13.4 |  1.4 | 12.7 to 13.4 |  1.4 |
| 17.5 to 18.2 |  1.3 | 17.5 to 18.2 |  1.3 | 13.5 to 14.2 |  1.3 | 13.5 to 14.2 |  1.3 |
| 18.3 to 19.0 |  1.2 | 18.3 to 19.0 |  1.2 | 14.3 to 15.0 |  1.2 | 14.3 to 15.0 |  1.2 |
| 19.1 to 19.8 |  1.1 | 19.1 to 19.8 |  1.1 | 15.1 to 15.8 |  1.1 | 15.1 to 15.8 |  1.1 |
| 19.9 to 20.6 |  1.0 | 19.9 to 20.6 |  1.0 | 15.9 to 16.6 |  1.0 | 15.9 to 16.6 |  1.0 |
| 20.7 to 21.4 |  0.9 | 20.7 to 21.4 |  0.9 | 16.7 to 17.4 |  0.9 | 16.7 to 17.4 |  0.9 |
| 21.5 to 22.2 |  0.8 | 21.5 to 22.2 |  0.8 | 17.5 to 18.2 |  0.8 | 17.5 to 18.2 |  0.8 |
| 22.3 to 23.0 |  0.7 | 22.3 to 23.0 |  0.7 | 18.3 to 19.0 |  0.7 | 18.3 to 19.0 |  0.7 |
| > 23.0 |  0.6 | > 23.0 |  0.6 | > 19.0 |  0.6 | > 19.0 |  0.6 |
| Notes: PMF = pedestrians per minute per foot; Y = increase in average pedestrian flow rate in PMF; X = No Build pedestrian flow rate in PMF. Sources: New York City Mayor’s Office of Environmental Coordination, CEQR Technical Manual. |  |  |  |  |  |  |  |

The determination of significant corner and crosswalk impacts is also based on a sliding scale using the following formula: Y  X/9.0 – 0.3, where Y is the decrease in pedestrian space in SFP and X is the No Build pedestrian space in SFP. Since a decrease in pedestrian space within acceptable levels would not constitute a significant impact, this formula would apply only if the Build pedestrian space falls short of LOS C in non-CBD areas or mid-LOS D in CBD areas. Table 9-27 summarizes the sliding scale guidance provided by the CEQR Technical Manual for determining potential significant corner reservoir and crosswalk impacts.

The proposed project is located in a CBD area. Therefore, the above CBD area significant impact guidelines for pedestrian elements would be applicable.

2012 Existing Conditions

Pedestrian data were collected in September and October 2012 in accordance with procedures outlined in the CEQR Technical Manual.

Peak hours were determined by comparing rolling hourly averages; peak 15-minute pedestrian volumes were developed by dividing the hourly incremental volumes by four and accounting for peaking characteristics within the peak hours for each pedestrian element. The existing peak 1-Hour weekday AM, midday, and PM pedestrian volumes are presented in Figures 9-20 to 9-22. Tables 9-28 to 9-30 provide overall summaries of pedestrian levels of service under 2012 existing conditions. As shown in Tables 9-47 to 9-49 in Section H, “Detailed Analysis Results Tables,” all sidewalks, corner reservoirs, and crosswalk analysis locations operate at acceptable mid-LOS D or better (maximum of 8.5 PMF platoon flows for sidewalks; minimum of 19.5 SFP for corners and crosswalks).

| Table 9-27 Significant Impact Guidance for Corners and Crosswalks |  |  |  |
| --- | --- | --- | --- |
| Sliding Scale Formula: Y  X/9.0 – 0.3 |  |  |  |
| Non-CBD Areas |  | CBD Areas |  |
| No Build Pedestrian Space (X, SFP) | Build Pedestrian Space Reduction (Y, SFP) | No Build Pedestrian Space (X, SFP) | Build Pedestrian Space Reduction (Y, SFP) |
| 25.8 to 26.6 |  2.6 | – | – |
| 24.9 to 25.7 |  2.5 | – | – |
| 24.0 to 24.8 |  2.4 | – | – |
| 23.1 to 23.9 |  2.3 | – | – |
| 22.2 to 23.0 |  2.2 | – | – |
| 21.3 to 22.1 |  2.1 | 21.3 to 21.5 |  2.1 |
| 20.4 to 21.2 |  2.0 | 20.4 to 21.2 |  2.0 |
| 19.5 to 20.3 |  1.9 | 19.5 to 20.3 |  1.9 |
| 18.6 to 19.4 |  1.8 | 18.6 to 19.4 |  1.8 |
| 17.7 to 18.5 |  1.7 | 17.7 to 18.5 |  1.7 |
| 16.8 to 17.6 |  1.6 | 16.8 to 17.6 |  1.6 |
| 15.9 to 16.7 |  1.5 | 15.9 to 16.7 |  1.5 |
| 15.0 to 15.8 |  1.4 | 15.0 to 15.8 |  1.4 |
| 14.1 to 14.9 |  1.3 | 14.1 to 14.9 |  1.3 |
| 13.2 to 14.0 |  1.2 | 13.2 to 14.0 |  1.2 |
| 12.3 to 13.1 |  1.1 | 12.3 to 13.1 |  1.1 |
| 11.4 to 12.2 |  1.0 | 11.4 to 12.2 |  1.0 |
| 10.5 to 11.3 |  0.9 | 10.5 to 11.3 |  0.9 |
| 9.6 to 10.4 |  0.8 | 9.6 to 10.4 |  0.8 |
| 8.7 to 9.5 |  0.7 | 8.7 to 9.5 |  0.7 |
| 7.8 to 8.6 |  0.6 | 7.8 to 8.6 |  0.6 |
| 6.9 to 7.7 |  0.5 | 6.9 to 7.7 |  0.5 |
| 6.0 to 6.8 |  0.4 | 6.0 to 6.8 |  0.4 |
| 5.1 to 5.9 |  0.3 | 5.1 to 5.9 |  0.3 |
| < 5.1 |  0.2 | < 5.1 |  0.2 |
| Notes: SFP = square feet per pedestrian; Y = decrease in pedestrian space in SFP; X = No Build pedestrian space in SFP. Sources: New York City Mayor’s Office of Environmental Coordination, CEQR Technical Manual. |  |  |  |

| Table 9-28 2012 Existing Conditions Pedestrian Sidewalk Level of Service Summary |  |  |  |
| --- | --- | --- | --- |
|  | Weekday |  |  |
|  | AM Peak Hour | Midday Peak Hour | PM Peak Hour |
| Overall LOS A/B/C | 12 | 12 | 12 |
| Overall LOS D | 0 | 0 | 0 |
| Overall LOS E | 0 | 0 | 0 |
| Overall LOS F | 0 | 0 | 0 |
| Note: Includes 12 sidewalk analysis locations. |  |  |  |

| Table 9-29 2012 Existing Conditions Pedestrian Corner Level of Service Summary |  |  |  |
| --- | --- | --- | --- |
|  | Weekday |  |  |
|  | AM Peak Hour | Midday Peak Hour | PM Peak Hour |
| Overall LOS A/B/C | 23 | 23 | 23 |
| Overall LOS D | 0 | 0 | 0 |
| Overall LOS E | 0 | 0 | 0 |
| Overall LOS F | 0 | 0 | 0 |
| Note: Includes 23 corner analysis locations. |  |  |  |

| Table 9-30 2012 Existing Conditions Pedestrian Crosswalk Level of Service Summary |  |  |  |
| --- | --- | --- | --- |
|  | Weekday |  |  |
|  | AM Peak Hour | Midday Peak Hour | PM Peak Hour |
| Overall LOS A/B/C | 11 | 11 | 11 |
| Overall LOS D | 0 | 0 | 0 |
| Overall LOS E | 0 | 0 | 0 |
| Overall LOS F | 0 | 0 | 0 |
| Note: Includes 11 crosswalk analysis locations. |  |  |  |

Future Without the proposed project (2019 No Build Condition)

No Build condition pedestrian volumes were estimated by increasing existing pedestrian levels to reflect expected growth in overall travel through and within the study area. As per CEQR Technical Manual guidelines, an annual background growth rate of 0.25 percent was assumed for the first five years (year 2012 to year 2017) and then 0.125 percent for the remaining years (year 2017 to year 2019). Pedestrian volumes from projects that are anticipated to be completed in the study area, absent the proposed project, were also added to arrive at the No Build condition pedestrian volumes. The total No Build peak 1-Hour pedestrian volumes for the weekday AM, midday, and PM peak periods are presented in Figures 9-23 to 9-25.

Tables 9-31 to 9-33 show comparisons of pedestrian levels of service for the existing and No Build conditions. As summarized in Tables 9-50 to 9-52 in Section H, “Detailed Analysis Results Tables,” all sidewalk, corner reservoir, and crosswalk analysis locations will continue to operate at acceptable mid-LOS D or better (maximum of 8.5 PMF platoon flows for sidewalks; minimum of 19.5 SFP for corners and crosswalks).

| Table 9-31 2019 No Build Condition Pedestrian Sidewalk Level of Service Summary |  |  |  |
| --- | --- | --- | --- |
|  | Weekday |  |  |
|  | AM Peak Hour | Midday Peak Hour | PM Peak Hour |
| Overall LOS A/B/C | 12 | 12 | 12 |
| Overall LOS D | 0 | 0 | 0 |
| Overall LOS E | 0 | 0 | 0 |
| Overall LOS F | 0 | 0 | 0 |
| Note: Includes 12 sidewalk analysis locations. |  |  |  |

| Table 9-32 2019 No Build Condition Pedestrian Corner Level of Service Summary |  |  |  |
| --- | --- | --- | --- |
|  | Weekday |  |  |
|  | AM Peak Hour | Midday Peak Hour | PM Peak Hour |
| Overall LOS A/B/C | 23 | 23 | 23 |
| Overall LOS D | 0 | 0 | 0 |
| Overall LOS E | 0 | 0 | 0 |
| Overall LOS F | 0 | 0 | 0 |
| Note: Includes 23 corner analysis locations. |  |  |  |

| Table 9-33 2019 No Build Condition Pedestrian Crosswalk Level of Service Summary |  |  |  |
| --- | --- | --- | --- |
|  | Weekday |  |  |
|  | AM Peak Hour | Midday Peak Hour | PM Peak Hour |
| Overall LOS A/B/C | 11 | 11 | 11 |
| Overall LOS D | 0 | 0 | 0 |
| Overall LOS E | 0 | 0 | 0 |
| Overall LOS F | 0 | 0 | 0 |
| Note: Includes 11 crosswalk analysis locations. |  |  |  |

probable impacts of the Proposed project (2019 Build Condition)

The project-generated pedestrian volumes were assigned to the pedestrian network considering current land uses in the area, nearby parking locations, available transit services, and pedestrian pathways connecting to/from the project site. Based on the incremental peak hour pedestrian trips presented on Figures 9-6 to 9-8 in Section B, “Preliminary Analysis Methodology and Screening Assessment,” peak 15-minute incremental pedestrian volumes were developed by dividing the hourly incremental volumes by four and accounting for peaking characteristics within the peak hours. These pedestrian volumes were added to the No Build volumes to arrive at the Build pedestrian volumes for analysis. The total Build peak 1-Hour pedestrian volumes are presented in Figures 9-26 to 9-28.

- The pedestrian analyses conducted for the Build condition accounted for the project-generated pedestrian volumes and anticipated physical changes to the pedestrian environment. Tables 9-34 to 9-36 show pedestrian levels of service for the Build condition.

| Table 9-34 2019 Build Condition Pedestrian Sidewalk Level of Service Summary |  |  |  |
| --- | --- | --- | --- |
|  | Weekday |  |  |
|  | AM Peak Hour | Midday Peak Hour | PM Peak Hour |
| Overall LOS A/B/C | 11 | 11 | 10 |
| Overall LOS D | 1 | 1 | 2 |
| Overall LOS E | 0 | 0 | 0 |
| Overall LOS F | 0 | 0 | 0 |
| Note: Includes 12 sidewalk analysis locations. |  |  |  |

| Table 9-35 2019 Build Condition Pedestrian Corner Level of Service Summary |  |  |  |
| --- | --- | --- | --- |
|  | Weekday |  |  |
|  | AM Peak Hour | Midday Peak Hour | PM Peak Hour |
| Overall LOS A/B/C | 23 | 23 | 23 |
| Overall LOS D | 0 | 0 | 0 |
| Overall LOS E | 0 | 0 | 0 |
| Overall LOS F | 0 | 0 | 0 |
| Note: Includes 23 corner analysis locations. |  |  |  |

| Table 9-36 2019 Build Condition Pedestrian Crosswalk Level of Service Summary |  |  |  |
| --- | --- | --- | --- |
|  | Weekday |  |  |
|  | AM Peak Hour | Midday Peak Hour | PM Peak Hour |
| Overall LOS A/B/C | 11 | 10 | 10 |
| Overall LOS D | 0 | 1 | 1 |
| Overall LOS E | 0 | 0 | 0 |
| Overall LOS F | 0 | 0 | 0 |
| Note: Includes 11 crosswalk analysis locations. |  |  |  |

As summarized in Tables 9-53 to 9-55 in Section H, “Detailed Analysis Results Tables,” all sidewalk, corner reservoir, and crosswalk locations would continue to operate at acceptable levels (within mid-LOS D, with a maximum of 8.5 PMF in sidewalk platoon flows or a minimum of 19.5 SFP for corners and crosswalks) during all peak hours. Therefore, the proposed project is not expected to result in any significant adverse pedestrian impacts.

- Vehicular and Pedestrian safety Evaluation

Methodology

An evaluation of vehicular and pedestrian safety is necessary for locations within the traffic and pedestrian study areas that have been identified as high accident locations, where 48 or more total reportable and non-reportable crashes or five or more vehicular-pedestrian/bicyclist injury crashes occurred in any consecutive 12 months of the most recent 3-year period for which data are available. For these locations, accident trends are identified to determine whether projected vehicular and pedestrian traffic would further impact safety at these locations. The determination of potential significant safety impacts depends on the type of area where the project site is located, traffic volumes, accident types and severity, and other contributing factors. Where appropriate, measures to improve traffic and pedestrian safety are identified and coordinated with NYCDOT.

aCCIDENT DATA

Crash data for the study area intersections were obtained from NYSDOT for the time period between January 1, 2009 and December 31, 2011. The data obtained quantify the total number of reportable accidents (involving fatality, injury, or more than $1,000 in property damage), fatalities, and injuries during the study period, as well as a yearly breakdown of vehicular crashes with pedestrians and bicycles at each location.

During the January 1, 2009 to December 31, 2011 three-year period, a total of 280 reportable and non-reportable accidents, zero fatalities, 209 injuries, and 68 pedestrian/bicyclist-related accidents occurred at the study area intersections. A rolling total of accident data identifies two study area intersections as high accident locations in the 2009 to 2011 period. These locations are First Avenue at East 72nd Street and York Avenue at East 72nd Street. Table 9-37 depicts total accident characteristics by intersection during the study period, as well as a breakdown of pedestrian and bicycle accidents by year and location. Table 9-38 shows a detailed description of each accident at the intersections of First Avenue at East 72nd Street and York Avenue at East 72nd Street during the three-year period.

| Table 9-37 Accident Summary |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intersection |  | Study Period |  |  |  |  | Accidents by Year |  |  |  |  |  |
| North-South Roadway | East-West Roadway | All Accidents by Year |  |  | Total Fatalities | Total Injuries | Pedestrian |  |  | Bicycle |  |  |
|  |  | 2009 | 2010 | 2011 |  |  | 2009 | 2010 | 2011 | 2009 | 2010 | 2011 |
| Second Avenue | E. 65th Street | 7 | 9 | 5 | 0 | 8 | 1 | 1 |  |  | 1 |  |
| Second Avenue | E. 66th Street | 8 | 3 | 8 | 0 | 14 |  | 1 | 3 |  |  | 1 |
| Second Avenue | E. 72nd Street | 4 | 6 | 9 | 0 | 21 |  | 1 |  | 1 | 1 |  |
| Second Avenue | E. 73rd Street | 4 | 4 | 4 | 0 | 12 | 1 |  | 2 |  | 2 | 1 |
| First Avenue | E. 65th Street | 5 | 6 | 6 | 0 | 16 | 2 | 1 | 1 | 1 | 1 | 1 |
| First Avenue | E. 66th Street | 5 | 5 | 3 | 0 | 6 |  | 1 |  |  | 1 |  |
| First Avenue | E. 72nd Street | 4 | 13 | 6 | 0 | 18 | 1 | 3 | 2 | 2 | 2 |  |
| First Avenue | E. 73rd Street | 1 | 2 | 1 | 0 | 3 |  |  |  |  | 1 | 1 |
| First Avenue | E. 74th Street | 4 | 3 | 1 | 0 | 9 | 1 | 1 |  | 1 |  | 1 |
| York Avenue | E. 61st Street | 13 | 6 | 19 | 0 | 26 |  |  | 2 | 1 |  | 1 |
| York Avenue | E. 65th Street | 3 | 2 | 4 | 0 | 4 | 1 |  |  |  |  |  |
| York Avenue | E. 66th Street | 2 | 7 | 7 | 0 | 9 |  |  |  |  |  |  |
| York Avenue | E. 68th Street | 2 | 0 | 1 | 0 | 1 |  |  |  |  |  |  |
| York Avenue | E. 72nd Street | 8 | 8 | 13 | 0 | 33 | 1 | 2 | 5 | 1 |  |  |
| York Avenue | E. 73rd Street | 6 | 3 | 2 | 0 | 10 | 2 | 2 |  |  |  | 1 |
| York Avenue | E. 74th Street | 4 | 0 | 4 | 0 | 3 |  |  |  | 1 |  |  |
| York Avenue | E. 75th Street | 3 | 3 | 0 | 0 | 1 |  |  |  |  |  |  |
| York Avenue | E. 76th Street | 5 | 4 | 0 | 0 | 4 | 1 |  |  |  | 1 |  |
| York Avenue | E. 79th Street | 7 | 4 | 4 | 0 | 11 | 2 |  | 1 |  |  | 1 |
| Note: Bold intersections are high accident locations. Source: NYSDOT January 1, 2009 to December 31, 2011 accident data. |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 9-38 Vehicle and Pedestrian Accident Details |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intersection | Year | Date | Time | Accident Class |  | Action of Vehicle | Action of Pedestrian | Cause of Accident |  |  |  |
|  |  |  |  | Injured | Killed |  |  | Left / Right Turns | Pedestrian Error/ Confusion | Driver Inattention | Other |
| First Avenue @ East 72nd Street | 2009 | 2/14 | 20:00 PM | X |  | Making left turn – Northwest | Going straight – (Bike) | X |  | X | Turning Improper |
|  |  | 4/7 | 9:15 AM | X |  | Making left turn – East | Crossing with signal | X | X |  |  |
|  |  | 12/7 | 15:00 PM | X |  | Other – East | Going straight – (Bike) |  |  |  | Unknown |
|  | 2010 | 3/19 | 19:45 PM | X |  | Going straight – West | Going straight – North (Bike) |  |  |  | Unknown |
|  |  | 7/6 | 13:00 PM | X |  | Unknown | Unknown |  |  |  | Unknown |
|  |  | 8/2 | 14:54 PM | X |  | Making left turn – North | Crossing with signal | X |  |  |  |
|  |  | 11/21 | 15:30 PM | X |  | Going straight – East | Stopped in traffic – East (Bike) |  |  |  | Failure to yield R.O.W., Aggressive Driving/Road Rage |
|  |  | 12/10 | 15:25 PM | X |  | Making left turn – Northeast | Crossing with signal | X |  |  | Driver Inexperience |
|  | 2011 | 1/24 | 14:40 PM | X |  | Making left turn – Northwest | Crossing with signal | X |  | X | Failure to yield R.O.W. |
|  |  | 7/5 | 15:38 PM | X |  | Making left turn – Northeast | Crossing with signal | X |  | X | Failure to yield R.O.W. |
| York Avenue @ East 72nd Street | 2009 | 5/13 | 18:45 PM | X |  | Going straight – North | Going straight – North (Bike) |  | X | X |  |
|  |  | 12/5 | N/A | X |  | Making left turn – Northeast | Crossing with signal | X |  |  |  |
|  | 2010 | 6/12 | 16:15 PM | X |  | Making left turn – Northeast | Crossing with signal | X |  | X |  |
|  |  | 12/6 | 16:10 PM | X |  | Going straight – North | Crossing against signal |  | X |  |  |
|  | 2011 | 1/18 | 8:00 AM | X |  | Making left turn – Northwest | Crossing with signal | X |  |  | Failure to yield R.O.W. |
|  |  | 2/1 | 22:27 PM | X |  | Making right turn – East | Crossing with signal | X |  | X | Failure to yield R.O.W. |
|  |  | 4/26 | 8:15 AM | X |  | Unknown | Unknown |  |  |  | Unknown |
|  |  | 8/15 | 16:20 PM | x |  | Going straight – South | Crossing against signal |  | X |  |  |
|  |  | 10/6 | 22:30 PM | X |  | Making left turn – Northeast | Crossing with signal | X |  |  | Reaction to other uninvolved vehicle |
| Source: NYSDOT January 1, 2009 to December 31, 2011 accident data. |  |  |  |  |  |  |  |  |  |  |  |

First Avenue and East 72nd Street

Based on the review of the accident history at the intersection of First Avenue and East 72nd Street, no prevailing trends with regard to geometric deficiencies were identified as the primary cause of recorded accidents. It is worth noting that 6 of the 10 accidents at this location involved turning vehicles. With respect to geometric deficiencies that could potentially cause safety hazards, the intersection of First Avenue and East 72nd Street is signalized and provides three school crosswalks and one high visibility crosswalk to the South. In terms of project-generated activity, the intersection would experience incremental peak-hour volume increases of approximately 70 or fewer vehicle trips and 190 or fewer pedestrian trips at any crosswalks at this intersection during each of the three analysis peak hours. As discussed above in Section C, “Detailed Traffic Analysis,” this intersection would be impacted during the weekday PM peak hour under the 2019 Build condition.

However, as described in Chapter 17, “Mitigation,” the predicted impact at this intersection could be fully mitigated with standard traffic engineering measures. Therefore, the proposed project is not anticipated to exacerbate any of the current causes of pedestrian-related accidents. Nonetheless, additional safety measures, such as the installation of countdown timers on all pedestrian crosswalks, the installation of pedestrian safety signs warning turning vehicles to yield to pedestrians in the crosswalk, and restriping both the faded north and south crosswalks, can be implemented to improve pedestrian safety at this intersection.

York Avenue and East 72nd Street

Based on the review of the accident history at the intersection of York Avenue and East 72nd Street, no prevailing trends with regard to geometric deficiencies were identified as the primary cause of recorded accidents. It is worth noting that two thirds (6 of 9) of the accidents coded for this intersection involve turning movements. With respect to geometric deficiencies that could potentially cause safety hazards, the intersection of York Avenue and East 72nd Street is signalized and provides two school crosswalks and two high visibility crosswalks. In terms of project-generated activity, the intersection would experience incremental peak-hour volume increases of approximately 130 or fewer vehicle trips and 360 or fewer pedestrian trips at any crosswalks at this intersection during each of the three analysis peak hours. As discussed above in Section C, “Detailed Traffic Analysis,” this intersection would be impacted during all three analysis peak hours under the 2019 Build condition.

However, as described in Chapter 17, “Mitigation,” the predicted impacts at this intersection could be fully mitigated with standard traffic engineering measures. Therefore, the proposed project is not anticipated to exacerbate any of the current causes of pedestrian-related accidents. Nonetheless, additional safety measures, such as the installation of countdown timers on all pedestrian crosswalks and the installation of pedestrian safety signs warning turning vehicles to yield to pedestrians in the crosswalk, can be implemented to improve pedestrian safety at this intersection.

- Parking conditions assessment

Methodology

The parking analysis identifies the extent to which off-street parking is available and utilized under existing and future conditions. It takes into consideration anticipated changes in area parking supply and provides a comparison of parking needs versus availability to determine if a parking shortfall is likely to result from parking displacement attributable to or additional demand generated by a proposed project. Typically, this analysis encompasses a study area within a ¼-mile of the project site. If the analysis concludes a shortfall in parking within the ¼-mile study area, the study area could be extended to a ½-mile to identify additional parking supply.

For proposed projects located in Manhattan or other CBD areas, the inability of the proposed project or the surrounding area to accommodate the project’s future parking demand is considered a parking shortfall, but is generally not considered significant due to the magnitude of available alternative modes of transportation. Additional factors, such as the availability and extent of transit in the area, proximity of the project to such transit, and patterns of automobile usage by area residents, could be considered to determine the significance of the identified parking shortfall.

2012 Existing Conditions

An inventory of on- and off-street parking within a ¼-mile of the project site was conducted in July 2012. The on-street survey involved recording curbside regulations and performing general observations of daytime utilization. The off-street survey provided an inventory of the area’s public parking facilities and their legal capacities and daytime utilization.

On-Street Parking

Curbside parking regulations within a ¼-mile of the project site are illustrated in Figure 9-29 and summarized in Table 9-39. The curbside regulations in the area generally include limited one-hour metered parking, no standing or no parking anytime except authorized vehicles, and alternate-side parking to accommodate street-cleaning. Based on field observations, on-street parking in the area is generally at or near full utilization during weekday daytime hours.

| Table 9-39 On-Street Parking Regulations |  |  |  |
| --- | --- | --- | --- |
| No. | Regulation | No. | Regulation |
| 1 | NP 9AM-10:30PM Mon & Thurs. | 17 | NS Ex. Authorized Veh. (Consul & Diplomat Plates) |
| 2 | NP 9AM-10:30PM Tue & Fri. | 18 | NP 7:30AM-8PM Except Sun. |
| 3 | NS Anytime | 19 | 1-Hr Parking 8AM-7PM Except Sun. |
| 4 | NP Anytime | 20 | NS Ex. Trucks Loading & Unloading 8AM-6PM Mon-Fri. |
| 5 | NS 7AM-4PM School Days | 21 | NS 7AM-10AM Mon-Fri. |
| 6 | NP 7:30AM-8AM Tue & Fri. | 22 | NS Ex Trucks Loading & Unloading 10AM-7PM Mon-Fri |
| 7 | NP 7AM-7PM Except Sun. | 23 | NS Ex Trucks Loading & Unloading 8AM-10AM Ex Sun |
| 8 | NP 8AM-8:30PM Except Sun. | 24 | 1-Hr Parking 10AM-7PM Except Sun. |
| 9 | 1-Hr Parking 8:30AM-7PM Except Sun. | 25 | NS Ex Trucks Loading & Unloading 8AM-5PM Tue & Fri |
| 10 | NS 7AM-10AM, 2PM-7PM Mon-Fri, 3 HR Limit Commercial Vehicles | 26 | NP 11AM-12:30PM Mon & Thurs. |
|  |  | 27 | NS Ex. Authorized Veh. (Ambulette) |
| 11 | 1-Hr Parking 9AM-7PM Sat. | 28 | No Stopping Anytime |
| 12 | NS Ex. Trucks Loading & Unloading 7AM-7PM Mon-Fri. | 29 | NS Ex. Emergency Veh. (H.S.S.) |
| 13 | NS Ex. Authorized Veh. (Ambulance) | 30 | NS 7AM-7PM Passenger Loading & Unloading Zone (NYPH) |
| 14 | NS 7AM-7PM Mon-Fri. |  |  |
| 15 | NP 8AM-6PM Mon-Fri. | 31 | NS Ex. Taxis 7AM-10AM Mon-Fri. |
| 16 | NP 8AM-8:30PM Mon & Thur. | 32 | NP 11AM-12:30PM Tue & Fri. |
| Notes: NP = No Parking; NS = No Standing; Sun = Sunday; Mon = Monday; Tue = Tuesday; Wed = Wednesday; Thu = Thursday; Fri = Friday; Sat = Saturday Sources: Surveys conducted by AKRF, Inc.; July, 2012 |  |  |  |

Off-Street Parking

Off-street publicly accessible parking lots and garages (see Figure 9-30) within ¼-mile of the project site were surveyed in July 2012. Each facility’s operating license and legal capacity were noted. Based on responses given by parking attendants and visual inspections, where possible, estimates were made on the parking occupancy or utilization at each facility for the weekday morning, midday, evening, and overnight time periods. A summary of the recorded information and the area’s overall off-street public parking supply and utilization is presented in Table 9-40.

Within the ¼-mile parking study area, 19 public parking facilities were inventoried. The combined capacity of these facilities totals 2,364 parking spaces. Overall, they were 46, 80, 59, and 24-percent utilized, with 1,268, 470, 976, and 1,762 parking spaces available during the weekday morning, midday, evening, and overnight time periods, respectively.

| Table 9-40 2012 Existing Off-Street Parking - ¼-Mile Weekday Utilization |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Map # | Name/Operator and Address/Location |  | License Number | Licensed Capacity | Utilization Rate |  |  |  | Utilized Spaces |  |  |  | Available Spaces |  |  |  |
|  |  |  |  |  | AM | MD | PM | ON | AM | MD | PM | ON | AM | MD | PM | ON |
| 1 | Surrey Garage Corp. - 439 E. 77th Street |  | 138694 | 96 | 50% | 90% | 45% | 25% | 48 | 86 | 43 | 24 | 48 | 10 | 53 | 72 |
| 2 | STA Parking Corp. - 434 E. 77th Street |  | 427101 | 131 | 40% | 80% | 60% | 20% | 52 | 105 | 79 | 26 | 79 | 26 | 52 | 105 |
| 3 | Alliance Impala Parking - 402 E. 76th Street |  | 1286774 | 58 | 50% | 70% | 50% | 50% | 29 | 41 | 29 | 29 | 29 | 17 | 29 | 29 |
| 4 | Capital Car Park - 1420 York Avenue |  | 1243101 | 28 | 50% | 95% | 60% | 50% | 14 | 27 | 17 | 14 | 14 | 1 | 11 | 14 |
| 5 | East 77th Realty LLC - 500 E. 77th Street |  | 1071280 | 300 | 25% | 75% | 50% | 20% | 75 | 225 | 150 | 60 | 225 | 75 | 150 | 240 |
| 6 | Kinney Parking System - 530 E. 76th Street |  | 1201685 | 23 | 90% | 90% | 90% | 50% | 21 | 21 | 21 | 12 | 2 | 2 | 2 | 11 |
| 7 | Samo Parking LLC - 401 E. 74th Street |  | 1423764 | 61 | 65% | 85% | 85% | 20% | 40 | 52 | 52 | 12 | 21 | 9 | 9 | 49 |
| 8 | River York Stratford LLC - 1385 York Avenue |  | 1070442 | 150 | 50% | 75% | 80% | 30% | 75 | 113 | 120 | 45 | 75 | 37 | 30 | 105 |
| 9 | 73 Operating LLC - 525-545 E. 73rd Street |  | 1382879 | 128 | 70% | 95% | 75% | 10% | 90 | 122 | 96 | 13 | 38 | 6 | 32 | 115 |
| 10 | Quik Park East 72 LLC - 525 E. 72nd Street |  | 1330577 | 146 | 65% | 65% | 65% | 15% | 95 | 95 | 95 | 22 | 51 | 51 | 51 | 124 |
| 11 | Quik Park E. 73rd St. LLC - 524 E. 73rd Street |  | 1376342 | 320 | 40% | 70% | 60% | 25% | 128 | 224 | 192 | 80 | 192 | 96 | 128 | 240 |
| 12 | E. 72nd Realty LLC - 1353-1367 York Avenue |  | 1070441 | 235 | 25% | 85% | 50% | 10% | 59 | 200 | 118 | 24 | 176 | 35 | 117 | 211 |
| 13 | 355 E. 72nd Garage Corp. - 355 E. 72nd Street |  | 1184091 | 31 | 66% | 100% | 66% | CLD | 20 | 31 | 20 | CLD | 11 | 0 | 11 | CLD |
| 14 | 420 E. 72nd Garage Corp. - 420 E. 72nd Street |  | 1412461 | 51 | 50% | 90% | 50% | 20% | 26 | 46 | 26 | 10 | 25 | 5 | 25 | 41 |
| 15 | E. River 72nd Garage LLC - 515 E. 72nd Street |  | 813280 | 130 | 25% | 90% | 40% | 15% | 33 | 117 | 52 | 20 | 97 | 13 | 78 | 110 |
| 16 | 72nd Street LLC - 517 E. 71st Street |  | 1152232 | 50 | 50% | 75% | 75% | 50% | 25 | 38 | 38 | 25 | 25 | 12 | 12 | 25 |
| 17 | Helmsley Medical Tower Garage - 507 E. 70th Street |  | 831026 | 175 | 85% | 85% | 85% | 50% | 149 | 149 | 149 | 88 | 26 | 26 | 26 | 87 |
| 18 | The NY Hospital Laurence G. Payson House - 426-438 E. 71st Street |  | 369314 | 174 | 45% | 85% | 30% | 30% | 78 | 148 | 52 | 52 | 96 | 26 | 122 | 122 |
| 19 | Independent Parking LLC - 417 E. 71st Street |  | 897040 | 77 | 50% | 70% | 50% | 20% | 39 | 54 | 39 | 15 | 38 | 23 | 38 | 62 |
|  |  |  |  | 2,364 | 46% | 80% | 59% | 24% | 1,096 | 1,894 | 1,388 | 571 | 1,268 | 470 | 976 | 1,762 |
| Notes: |  | MD = Midday; ON = Overnight; CLD = Closed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sources: |  | Survey conducted by AKRF Inc. July 2012. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Future without the Proposed Project (2019 No Build Condition)

Overall off-street public parking utilization is expected to experience the same growth as projected for traffic. No Build projects within the ¼-mile parking study area are expected to include a total of up to 322 off-street accessory parking spaces. As presented in Table 9-41, the addition of the accessory parking spaces, and the parking demand generated from background growth and discrete projects that would advance absent the proposed project, the No Build condition public parking utilization is expected to increase to 52, 88, 62, and 25 percent during the weekday morning, midday, evening, and overnight peak periods, respectively, in the ¼-mile off-street parking study area.

| Table 9-41 2012 Existing and 2019 No Build Parking Supply and Utilization |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Weekday | Weekday | Weekday | Weekday |
|  | AM | Midday | PM | Overnight |
| 2012 Public Parking Supply | 2,364 | 2,364 | 2,364 | 2,333 |
| 2012 Public Parking Demand | 1,096 | 1,894 | 1,388 | 571 |
| 2012 Public Parking Utilization | 46% | 80% | 59% | 24% |
| 2019 No Build Public Parking Supply Total | 2,364 | 2,364 | 2,364 | 2,333 |
| 2019 No Build Background Incremental Demand | 17 | 29 | 21 | 9 |
| Discrete No Build Projects Total Parking Demand | 342 | 461 | 228 | 51 |
| Discrete No Build Projects Accessory Parking Spaces | 322 | 322 | 322 | 322 |
| Discrete No Build Parking Demand Accommodated by Accessory Parking | 234 | 314 | 166 | 51 |
| Discrete No Build Parking Demand Accommodated by Public Parking | 108 | 147 | 62 | 0 |
| No Build Incremental Public Parking Demand | 125 | 176 | 83 | 9 |
| 2019 No Build Public Parking Demand Total | 1,221 | 2,070 | 1,471 | 580 |
| 2019 No Build Public Parking Utilization | 52% | 88% | 62% | 25% |
| 2019 No Build Available Spaces (Shortfall) | 1,143 | 294 | 893 | 1,753 |
| Sample Calculation: No Build Incremental Public Parking Demand = No Build Background Incremental Demand + Discrete No Build Parking Demand Accommodated by Public Parking Weekday AM No Build Incremental Public Parking Demand = 17 + 108 = 125 |  |  |  |  |

Probable Impacts of the Proposed Project (2019 Build Condition)

In the Build condition, expected future development projects (including No Build projects and the proposed project) are expected to displace the surface public parking lot on the western portion of the project site, for a total displacement of 128 parking spaces. The proposed project would include a total of up to 250 off-street accessory parking spaces; however, this analysis conservatively accounts for only the 166 spaces permitted as-of-right at the project site. The weekday incremental parking demand generated by the proposed project is presented in Table 9-42. As presented in Table 9-43, accounting for the displacement of public parking spaces, the addition of accessory parking spaces, and the parking demand generated from background growth, No Build projects and the proposed project, the Build public parking utilization is expected to increase to 63, 113, 77, and 26 percent during the weekday morning, midday, evening, and overnight peak periods, respectively. This represents a parking shortfall of 298 spaces during the weekday midday peak period.

Most of this excess demand is expected to be accommodated by parking facilities outside of the ¼-mile parking study area radius. However, some may seek parking on-street or choose alternate modes of transportation.

As stated in the CEQR Technical Manual and discussed in the above parking analysis methodology, a parking shortfall resulting from a project located in Manhattan and other CBD neighborhoods, does not constitute a significant adverse parking impact, due to the magnitude of available alternative modes of transportation.

| Table 9-42 Proposed Project Incremental Parking Demand |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hour |  |  | MSK ACC |  | CUNY-Hunter Building |  |  |  |  |  | Total |
|  |  |  |  | Patients | Undergraduate | Graduate | Research Staff/Faculty/ | Support |  | Lecture Hall |  |
|  |  |  | Staff | and Visitors | Students | Students | Adjunct Faculty | Staff | Visitors | Students |  |
| 12 AM | - | 01 AM | 9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 |
| 01 AM | - | 02 AM | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 |
| 02 AM | - | 03 AM | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 03 AM | - | 04 AM | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 04 AM | - | 05 AM | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 05 AM | - | 06 AM | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |
| 06 AM | - | 07 AM | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 |
| 07 AM | - | 08 AM | 28 | 14 | 2 | 2 | 7 | 0 | 0 | 0 | 53 |
| 08 AM | - | 09 AM | 77 | 71 | 4 | 4 | 45 | 7 | 3 | 0 | 211 |
| 09 AM | - | 10 AM | 113 | 156 | 9 | 19 | 74 | 9 | 4 | 0 | 384 |
| 10 AM | - | 11 AM | 122 | 241 | 9 | 20 | 97 | 11 | 5 | 0 | 505 |
| 11 AM | - | 12 PM | 121 | 326 | 9 | 22 | 100 | 11 | 6 | 0 | 595 |
| 12 PM | - | 01 PM | 122 | 354 | 8 | 28 | 100 | 11 | 7 | 0 | 630 |
| 01 PM | - | 02 PM | 125 | 354 | 7 | 29 | 100 | 11 | 9 | 0 | 635 |
| 02 PM | - | 03 PM | 128 | 340 | 6 | 30 | 100 | 11 | 10 | 0 | 625 |
| 03 PM | - | 04 PM | 125 | 312 | 5 | 34 | 67 | 8 | 2 | 0 | 553 |
| 04 PM | - | 05 PM | 108 | 284 | 4 | 36 | 34 | 2 | 0 | 0 | 468 |
| 05 PM | - | 06 PM | 68 | 220 | 2 | 41 | 17 | 1 | 0 | 0 | 349 |
| 06 PM | - | 07 PM | 35 | 135 | 2 | 42 | 13 | 0 | 0 | 0 | 227 |
| 07 PM | - | 08 PM | 20 | 78 | 2 | 42 | 9 | 0 | 0 | 0 | 151 |
| 08 PM | - | 09 PM | 15 | 21 | 1 | 23 | 6 | 0 | 0 | 0 | 66 |
| 09 PM | - | 10 PM | 11 | 0 | 0 | 0 | 4 | 0 | 0 | 0 | 15 |
| 10 PM | - | 11 PM | 9 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 11 |
| 11 PM | - | 12 AM | 9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 |

| Table 9-43 2019 No Build and Build Parking Supply and Utilization |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Weekday | Weekday | Weekday | Weekday |
|  | AM | Midday | PM | Overnight |
| 2019 No Build Public Parking Supply | 2,364 | 2,364 | 2,364 | 2,333 |
| 2019 No Build Public Parking Demand | 1,221 | 2,070 | 1,471 | 580 |
| 2019 No Build Public Parking Utilization | 52% | 88% | 62% | 25% |
| 2019 No Build Public Parking Supply | 2,364 | 2,364 | 2,364 | 2,333 |
| Displaced Public Parking Supply Total | -128 | -128 | -128 | -128 |
| 2019 Build Public Parking Supply Total | 2,236 | 2,236 | 2,236 | 2,205 |
| Proposed Project Parking Demand | 211 | 630 | 349 | 0 |
| Proposed Project Accessory Parking Spaces (1) | 166 | 166 | 166 | 166 |
| Proposed Project Parking Demand Accommodated by Accessory Parking | 33 | 166 | 104 | 0 |
| Proposed Project Parking Demand Accommodated by Public Parking | 178 | 464 | 245 | 0 |
| 2019 Build Public Parking Demand Total | 1,399 | 2,534 | 1,716 | 580 |
| 2019 Build Public Parking Utilization | 63% | 113% | 77% | 26% |
| 2019 Build Available Spaces (Shortfall) | 837 | (298) | 520 | 1,625 |
| Note: (1) As described in Section C, “Detailed Traffic Analysis,” for conservative traffic analysis, the as-of-right 166 accessory spaces was assumed that would result in the maximum number of patients being dropped off first resulting in the maximum amount of re-circulation. A Special Permit is being requested that would increase the number of maximum parking spaces up to 250. With 250 accessory parking spaces, the projected weekday midday shortfall would be 214 spaces. Sample Calculation: 2019 Build Public Parking Demand Total = 2019 No Build Public Parking Demand + Proposed Project Parking Demand Accommodated by Public Parking 2019 Weekday AM Build Public Parking Demand Total = 1,221 + 178 = 1,399 |  |  |  |  |

- Detailed Analysis Results Tables

| Table 9-44 2012 Existing Conditions Level of Service Analysis Signalized Intersections |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weekday AM |  |  |  | Weekday Midday |  |  |  | Weekday PM |  |  |  |
|  | Lane | v/c | Delay |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |
| Intersection | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |
| York Avenue & East 79th Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 0.97 | 70.5 | E | LTR | 0.80 | 49.0 | D | LTR | 0.96 | 68.0 | E |
| Westbound | LTR | 0.22 | 32.3 | C | LTR | 0.32 | 34.1 | C | LTR | 0.41 | 35.8 | D |
| Northbound | LTR | 1.05 | 77.1 | E | LTR | 0.85 | 37.4 | D | LTR | 1.05 | 78.2 | E |
| Southbound | LTR | 0.88 | 45.6 | D | LTR | 0.76 | 38.0 | D | LTR | 0.89 | 46.6 | D |
|  | Intersection |  | 61.9 | E | Intersection |  | 40.0 | D | Intersection |  | 61.5 | E |
| York Avenue & East 76th Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 0.79 | 57.1 | E | LTR | 0.64 | 45.2 | D | LTR | 1.00 | 94.6 | F |
| Westbound | LR | 0.73 | 54.6 | D | LR | 0.23 | 32.9 | C | LR | 0.29 | 34.0 | C |
| Northbound | TR | 0.43 | 13.3 | B | TR | 0.42 | 13.1 | B | TR | 0.45 | 13.6 | B |
| Southbound | LT | 0.64 | 17.5 | B | LT | 0.53 | 14.9 | B | LT | 0.53 | 15.0 | B |
|  | Intersection |  | 24.3 | C | Intersection |  | 18.4 | B | Intersection |  | 27.8 | C |
| York Avenue & East 75th Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.30 | 33.9 | C | LTR | 0.10 | 30.2 | C | LTR | 0.08 | 29.8 | C |
| Northbound | LTR | 0.78 | 22.5 | C | LTR | 0.80 | 24.3 | C | LTR | 0.55 | 15.3 | B |
| Southbound | LTR | 0.72 | 19.9 | B | LTR | 0.63 | 17.0 | B | LTR | 0.67 | 18.0 | B |
|  | Intersection |  | 21.8 | C | Intersection |  | 20.8 | C | Intersection |  | 17.0 | B |
| York Avenue & East 74th Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 0.51 | 34.2 | C | LTR | 0.54 | 35.0- | C | LTR | 0.52 | 34.4 | C |
| Westbound | LR | 0.06 | 25.5 | C | LR | 0.09 | 26.0 | C | LR | 0.09 | 26.0 | C |
| Northbound | TR | 0.52 | 17.7 | B | TR | 0.51 | 17.5 | B | TR | 0.46 | 16.7 | B |
| Southbound | LT | 0.65 | 20.7 | C | LT | 0.52 | 17.8 | B | LT | 0.65 | 20.7 | C |
|  | Intersection |  | 21.2 | C | Intersection |  | 20.3 | C | Intersection |  | 21.1 | C |
| York Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.13 | 44.6 | D | LTR | 0.16 | 45.3 | D | LTR | 0.05 | 43.3 | D |
| Northbound | LTR | 0.97 | 51.3 | D | LTR | 0.84 | 35.6 | D | LTR | 1.02 | 64.3 | E |
| Southbound | DefL | 0.93 | 59.6 | E | - | - | - | - | DefL | 1.05 | 89.2 | F |
|  | - |  | - | - | LTR | 0.95 | 41.3 | D | - |  | - | - |
|  | TR | 0.96 | 48.4 | D | - | - | - | - | TR | 0.94 | 42.9 | D |
|  | Intersection |  | 51.3 | D | Intersection |  | 38.7 | D | Intersection |  | 60.7 | E |
| York Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | DefL | 1.00 | 95.2 | F | DefL | 0.71 | 48.3 | D | DefL | 0.69 | 46.2 | D |
|  | TR | 0.51 | 35.4 | D | TR | 0.50 | 35.3 | D | TR | 0.46 | 34.4 | C |
|  | R | 0.45 | 35.6 | D | R | 0.47 | 36.7 | D | R | 0.45 | 36.1 | D |
| Westbound | LTR | 0.45 | 32.8 | C | LTR | 0.47 | 33.7 | C | LTR | 0.29 | 29.4 | C |
| Northbound | LTR | 0.98 | 50.5 | D | LTR | 0.89 | 34.8 | C | LTR | 0.98 | 50.8 | D |
| Southbound | LTR | 0.58 | 19.5 | B | LTR | 0.53 | 18.3 | B | LTR | 0.49 | 17.6 | B |
|  | Intersection |  | 43.7 | D | Intersection |  | 31.1 | C | Intersection |  | 38.2 | D |
| York Avenue & East 71st Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.81 | 38.2 | D | LTR | 0.67 | 32.2 | C | LTR | 0.70 | 33.4 | C |
| Northbound | LTR | 0.80 | 30.9 | C | LTR | 0.58 | 22.7 | C | LTR | 0.67 | 25.2 | C |
| Southbound | LTR | 0.55 | 22.0 | C | LTR | 0.57 | 22.4 | C | LTR | 0.57 | 22.3 | C |
|  | Intersection |  | 31.1 | C | Intersection |  | 25.8 | C | Intersection |  | 27.0 | C |
| York Avenue & East 66th Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.03 | 29.1 | C | LTR | 0.04 | 29.2 | C | LTR | 0.03 | 29.1 | C |
| Northbound | LTR | 0.77 | 20.4 | C | LTR | 0.95 | 38.9 | D | LTR | 0.45 | 13.5 | B |
| Southbound | LTR | 0.57 | 15.6 | B | LTR | 0.63 | 16.7 | B | LTR | 0.69 | 18.2 | B |
|  | Intersection |  | 18.6 | B | Intersection |  | 27.8 | C | Intersection |  | 16.3 | B |

| Table 9-44 (cont’d) 2012 Existing Conditions Level of Service Analysis Signalized Intersections |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weekday AM |  |  |  | Weekday Midday |  |  |  | Weekday PM |  |  |  |
|  | Lane | v/c | Delay |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |
| Intersection | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |
| York Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LR | 0.95 | 76.3 | E | LR | 0.86 | 61.4 | E | LR | 1.05 | 100.4 | F |
| Northbound | T | 0.47 | 13.5 | B | T | 0.57 | 15.5 | B | T | 0.29 | 11.4 | B |
| Southbound | T | 0.43 | 13.3 | B | T | 0.49 | 14.2 | B | T | 0.54 | 15.0 | B |
|  | Intersection |  | 23.0 | C | Intersection |  | 21.7 | C | Intersection |  | 30.2 | C |
| York Avenue & East 61st Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | L | 0.23 | 24.0 | C | L | 0.36 | 29.0 | C | L | 0.34 | 28.6 | C |
|  | LTR | 0.71 | 33.4 | C | LTR | 0.62 | 33.3 | C | LTR | 0.60 | 32.7 | C |
|  | R | 0.69 | 36.5 | D | R | 0.59 | 35.3 | D | R | 0.62 | 36.3 | D |
| Northbound | LT | 0.81 | 28.6 | C | LT | 0.42 | 17.0 | B | LT | 0.55 | 19.0 | B |
| Southbound | TR | 0.45 | 19.6 | B | TR | 0.84 | 29.1 | C | TR | 0.52 | 18.5 | B |
|  | Intersection |  | 28.1 | C | Intersection |  | 27.4 | C | Intersection |  | 23.4 | C |
| First Avenue & East 74th Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 0.53 | 27.6 | C | LT | 0.66 | 31.8 | C | LT | 0.59 | 29.0 | C |
| Northbound | TR | 0.76 | 18.1 | B | TR | 0.65 | 15.8 | B | TR | 0.78 | 18.3 | B |
|  | Intersection |  | 19.1 | B | Intersection |  | 18.3 | B | Intersection |  | 19.6 | B |
| First Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | TR | 0.31 | 23.3 | C | TR | 0.30 | 23.2 | C | TR | 0.35 | 24.2 | C |
| Northbound | LT | 0.77 | 18.3 | B | LT | 0.69 | 16.6 | B | LT | 0.78 | 18.5 | B |
|  | Intersection |  | 18.6 | B | Intersection |  | 17.1 | B | Intersection |  | 18.8 | B |
| First Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 0.96 | 52.0 | D | LT | 0.73 | 30.2 | C | LT | 0.71 | 29.5 | C |
| Westbound | TR | 0.44 | 21.8 | C | TR | 0.39 | 21.0 | C | TR | 0.40 | 21.2 | C |
| Northbound | L | 0.59 | 45.0 | D | L | 0.57 | 44.4 | D | L | 0.47 | 39.9 | D |
|  | TR | 0.74 | 19.8 | B | TR | 0.73 | 19.7 | B | TR | 0.74 | 19.7 | B |
|  | Intersection |  | 27.8 | C | Intersection |  | 22.9 | C | Intersection |  | 22.2 | C |
| First Avenue & East 66th Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | TR | 0.67 | 33.5 | C | TR | 0.57 | 30.1 | C | TR | 0.51 | 27.5 | C |
| Northbound | LT | 0.93 | 26.4 | C | LT | 0.91 | 24.5 | C | LT | 0.83 | 20.0+ | C |
|  | Intersection |  | 27.1 | C | Intersection |  | 25.0 | C | Intersection |  | 20.7 | C |
| First Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 0.92 | 56.8 | E | LT | 0.94 | 58.8 | E | LT | 1.04 | 85.4 | F |
| Northbound | TR | 0.87 | 21.8 | C | TR | 0.79 | 19.0 | B | TR | 0.80 | 18.9 | B |
|  | Intersection |  | 26.7 | C | Intersection |  | 25.3 | C | Intersection |  | 29.8 | C |
| Second Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LT | 0.51 | 27.6 | C | LT | 0.53 | 28.0 | C | LT | 0.33 | 23.8 | C |
| Southbound | TR | 0.47 | 13.0 | B | TR | 0.47 | 13.1 | B | TR | 0.46 | 13.0 | B |
|  | Intersection |  | 14.9 | B | Intersection |  | 15.3 | B | Intersection |  | 14.0 | B |
| Second Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | TR | 0.63 | 24.4 | C | TR | 0.47 | 21.6 | C | TR | 0.58 | 23.5 | C |
| Westbound | LT | 0.63 | 25.1 | C | LT | 0.47 | 21.8 | C | LT | 0.44 | 21.2 | C |
| Southbound | LTR | 0.52 | 16.4 | B | LTR | 0.53 | 16.6 | B | LTR | 0.52 | 16.5 | B |
|  | Intersection |  | 20.3 | C | Intersection |  | 18.6 | B | Intersection |  | 19.1 | B |
| Second Avenue & East 66th Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LT | 0.48 | 21.1 | C | LT | 0.60 | 23.5 | C | LT | 0.49 | 21.2 | C |
| Southbound | TR | 0.49 | 16.7 | B | TR | 0.49 | 16.6 | B | TR | 0.47 | 16.4 | B |
|  | Intersection |  | 17.9 | B | Intersection |  | 18.7 | B | Intersection |  | 17.7 | B |
| Second Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | TR | 0.60 | 26.2 | C | TR | 0.47 | 23.5 | C | TR | 0.67 | 27.7 | C |
| Southbound | LT | 0.60 | 20.0+ | C | LT | 0.61 | 20.3 | C | LT | 0.57 | 19.6 | B |
|  | Intersection |  | 21.7 | C | Intersection |  | 21.0 | C | Intersection |  | 22.0 | C |
| Notes: L = Left Turn, T = Through, R = Right Turn, DefL = Defacto Left Turn, LOS = Level of Service |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 9-45 2012 Existing and 2019 No Build Conditions Level of Service Analysis Signalized Intersections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weekday AM |  |  |  |  |  |  |  | Weekday Midday |  |  |  |  |  |  |  | Weekday PM |  |  |  |  |  |  |  |
|  | 2012 Existing |  |  |  | 2019 No Build |  |  |  | 2012 Existing |  |  |  | 2019 No Build |  |  |  | 2012 Existing |  |  |  | 2019 No Build |  |  |  |
|  | Lane | v/c | Delay |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |
| Intersection | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |
| York Avenue & East 79th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 0.97 | 70.5 | E | LTR | 1.06 | 95.5 | F | LTR | 0.80 | 49.0 | D | LTR | 0.86 | 54.1 | D | LTR | 0.96 | 68.0 | E | LTR | 1.02 | 82.5 | F |
| Westbound | LTR | 0.22 | 32.3 | C | LTR | 0.23 | 32.4 | C | LTR | 0.32 | 34.1 | C | LTR | 0.34 | 34.5 | C | LTR | 0.41 | 35.8 | D | LTR | 0.42 | 36.1 | D |
| Northbound | LTR | 1.05 | 77.1 | E | LTR | 1.13 | 107.2 | F | LTR | 0.85 | 37.4 | D | LTR | 0.97 | 54.7 | D | LTR | 1.05 | 78.2 | E | LTR | 1.21 | 137.2 | F |
| Southbound | TR | 0.88 | 45.6 | D | TR | 0.93 | 51.7 | D | TR | 0.76 | 38.0 | D | TR | 0.79 | 39.5 | D | TR | 0.89 | 46.6 | D | TR | 0.92 | 50.5 | D |
|  | Intersection |  | 61.9 | E | Intersection |  | 80.8 | F | Intersection |  | 40.0 | D | Intersection |  | 47.9 | D | Intersection |  | 61.5 | E | Intersection |  | 87.5 | F |
| York Avenue & East 76th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 0.79 | 57.1 | E | LTR | 0.80 | 58.5 | E | LTR | 0.64 | 45.2 | D | LTR | 0.65 | 45.6 | D | LTR | 1.00 | 94.6 | F | LTR | 1.02 | 98.1 | F |
| Westbound | LR | 0.73 | 54.6 | D | LR | 0.74 | 55.7 | E | LR | 0.23 | 32.9 | C | LR | 0.23 | 32.9 | C | LR | 0.29 | 34.0 | C | LR | 0.32 | 34.8 | C |
| Northbound | TR | 0.43 | 13.3 | B | TR | 0.45 | 13.6 | B | TR | 0.42 | 13.1 | B | TR | 0.45 | 13.6 | B | TR | 0.45 | 13.6 | B | TR | 0.52 | 14.7 | B |
| Southbound | LT | 0.64 | 17.5 | B | LT | 0.72 | 20.0- | B | LT | 0.53 | 14.9 | B | LT | 0.57 | 15.6 | B | LT | 0.53 | 15.0 | B | LT | 0.57 | 15.8 | B |
|  | Intersection |  | 24.3 | C | Intersection |  | 25.3 | C | Intersection |  | 18.4 | B | Intersection |  | 18.7 | B | Intersection |  | 27.8 | C | Intersection |  | 28.1 | C |
| York Avenue & East 75th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.30 | 33.9 | C | LTR | 0.30 | 33.9 | C | LTR | 0.10 | 30.2 | C | LTR | 0.10 | 30.2 | C | LTR | 0.08 | 29.8 | C | LTR | 0.08 | 29.8 | C |
| Northbound | LTR | 0.78 | 22.5 | C | LTR | 0.85 | 27.3 | C | LTR | 0.80 | 24.3 | C | LTR | 0.87 | 29.5 | C | LTR | 0.55 | 15.3 | B | LTR | 0.64 | 17.2 | B |
| Southbound | LTR | 0.72 | 19.9 | B | LTR | 0.79 | 22.4 | C | LTR | 0.63 | 17.0 | B | LTR | 0.67 | 18.0 | B | LTR | 0.67 | 18.0 | B | LTR | 0.71 | 19.0 | B |
|  | Intersection |  | 21.8 | C | Intersection |  | 25.2 | C | Intersection |  | 20.8 | C | Intersection |  | 23.8 | C | Intersection |  | 17.0 | B | Intersection |  | 18.3 | B |
| York Avenue & East 74th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 0.51 | 34.2 | C | LTR | 0.56 | 36.0 | D | LTR | 0.54 | 35.0- | C | LTR | 0.59 | 36.7 | D | LTR | 0.52 | 34.4 | C | LTR | 0.59 | 36.9 | D |
| Westbound | LR | 0.06 | 25.5 | C | LR | 0.06 | 25.6 | C | LR | 0.09 | 26.0 | C | LR | 0.09 | 26.0 | C | LR | 0.09 | 26.0 | C | LR | 0.09 | 26.1 | C |
| Northbound | TR | 0.52 | 17.7 | B | TR | 0.57 | 18.8 | B | TR | 0.51 | 17.5 | B | TR | 0.56 | 18.6 | B | TR | 0.46 | 16.7 | B | TR | 0.55 | 18.3 | B |
| Southbound | LT | 0.65 | 20.7 | C | LT | 0.74 | 23.4 | C | LT | 0.52 | 17.8 | B | LT | 0.57 | 18.9 | B | LT | 0.65 | 20.7 | C | LT | 0.71 | 22.3 | C |
|  | Intersection |  | 21.2 | C | Intersection |  | 23.1 | C | Intersection |  | 20.3 | C | Intersection |  | 21.4 | C | Intersection |  | 21.1 | C | Intersection |  | 22.7 | C |
| York Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.13 | 44.6 | D | LTR | 0.20 | 45.9 | D | LTR | 0.16 | 45.3 | D | LTR | 0.40 | 51.3 | D | LTR | 0.05 | 43.3 | D | LTR | 0.43 | 52.3 | D |
| Northbound | LTR | 0.97 | 51.3 | D | LTR | 1.04 | 71.4 | E | LTR | 0.84 | 35.6 | D | LTR | 0.89 | 40.3 | D | LTR | 1.02 | 64.3 | E | LTR | 1.15 | 111.0 | F |
| Southbound | DefL | 0.93 | 59.6 | E | DefL | 1.00 | 81.9 | F | - | - | - | - | - | - | - | - | DefL | 1.05 | 89.2 | F | DefL | 1.17 | 137.4 | F |
|  | - | - | - | - | - | - | - | - | LTR | 0.95 | 41.3 | D | LTR | 1.01 | 56.7 | E | - | - | - | - | - | - | - | - |
|  | TR | 0.96 | 48.4 | D | TR | 1.06 | 75.4 | E | - | - | - | - | - | - | - | - | TR | 0.94 | 42.9 | D | TR | 1.00 | 58.1 | E |
|  | Intersection |  | 51.3 | D | Intersection |  | 73.7 | E | Intersection |  | 38.7 | D | Intersection |  | 48.7 | D | Intersection |  | 60.7 | E | Intersection |  | 95.1 | F |
| York Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | DefL | 1.00 | 95.2 | F | DefL | 1.05 | 109.1 | F | DefL | 0.71 | 48.3 | D | DefL | 0.73 | 50.7 | D | DefL | 0.69 | 46.2 | D | DefL | 0.72 | 49.0 | D |
|  | TR | 0.51 | 35.4 | D | TR | 0.54 | 36.3 | D | TR | 0.50 | 35.3 | D | TR | 0.51 | 35.6 | D | TR | 0.46 | 34.4 | C | TR | 0.47 | 34.8 | C |
|  | R | 0.45 | 35.6 | D | R | 0.47 | 36.7 | D | R | 0.47 | 36.7 | D | R | 0.48 | 37.3 | D | R | 0.45 | 36.1 | D | R | 0.47 | 36.8 | D |
| Westbound | LTR | 0.45 | 32.8 | C | LTR | 0.46 | 33.1 | C | LTR | 0.47 | 33.8 | C | LTR | 0.48 | 34.1 | C | LTR | 0.29 | 29.4 | C | LTR | 0.31 | 29.7 | C |
| Northbound | LTR | 0.98 | 50.5 | D | LTR | 1.09 | 83.5 | F | LTR | 0.89 | 34.8 | C | LTR | 0.96 | 46.3 | D | LTR | 0.98 | 50.8 | D | LTR | 1.10 | 86.0 | F |
| Southbound | LTR | 0.58 | 19.5 | B | LTR | 0.64 | 20.9 | C | LTR | 0.53 | 18.3 | B | LTR | 0.57 | 19.1 | B | LTR | 0.49 | 17.6 | B | LTR | 0.54 | 18.4 | B |
|  | Intersection |  | 43.7 | D | Intersection |  | 59.1 | E | Intersection |  | 31.1 | C | Intersection |  | 36.2 | D | Intersection |  | 38.2 | D | Intersection |  | 55.5 | E |
| York Avenue & East 71st Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.81 | 38.2 | D | LTR | 0.83 | 39.3 | D | LTR | 0.67 | 32.2 | C | LTR | 0.68 | 32.7 | C | LTR | 0.70 | 33.4 | C | LTR | 0.72 | 34.1 | C |
| Northbound | LTR | 0.80 | 30.9 | C | LTR | 0.89 | 38.2 | D | LTR | 0.58 | 22.7 | C | LTR | 0.64 | 24.2 | C | LTR | 0.67 | 25.2 | C | LTR | 0.77 | 28.9 | C |
| Southbound | LTR | 0.55 | 22.0 | C | LTR | 0.61 | 23.4 | C | LTR | 0.57 | 22.4 | C | LTR | 0.61 | 23.3 | C | LTR | 0.57 | 22.3 | C | LTR | 0.61 | 23.3 | C |
|  | Intersection |  | 31.1 | C | Intersection |  | 34.3 | D | Intersection |  | 25.8 | C | Intersection |  | 26.6 | C | Intersection |  | 27.0 | C | Intersection |  | 28.8 | C |
| York Avenue & East 66th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.03 | 29.1 | C | LTR | 0.03 | 29.1 | C | LTR | 0.04 | 29.2 | C | LTR | 0.04 | 29.2 | C | LTR | 0.03 | 29.1 | C | LTR | 0.03 | 29.1 | C |
| Northbound | LTR | 0.77 | 20.4 | C | LTR | 0.85 | 24.5 | C | LTR | 0.95 | 38.9 | D | LTR | 1.07 | 74.2 | E | LTR | 0.45 | 13.5 | B | DefL | 0.54 | 29.7 | C |
|  | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | TR | 0.43 | 13.3 | B |
| Southbound | LTR | 0.57 | 15.6 | B | LTR | 0.74 | 27.8 | C | LTR | 0.63 | 16.7 | B | LTR | 0.82 | 31.6 | C | LTR | 0.69 | 18.2 | B | LTR | 0.96 | 46.2 | D |
|  | Intersection |  | 18.6 | B | Intersection |  | 25.7 | C | Intersection |  | 27.8 | C | Intersection |  | 52.7 | D | Intersection |  | 16.3 | B | Intersection |  | 33.6 | C |
| York Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LR | 0.95 | 76.3 | E | LR | 1.03 | 97.6 | F | LR | 0.86 | 61.4 | E | LR | 0.92 | 71.0 | E | LR | 1.05 | 100.4 | F | LR | 1.10 | 118.3 | F |
| Northbound | T | 0.47 | 13.5 | B | T | 0.53 | 14.4 | B | T | 0.57 | 15.5 | B | T | 0.61 | 16.2 | B | T | 0.29 | 11.4 | B | T | 0.31 | 11.7 | B |
| Southbound | T | 0.43 | 13.3 | B | T | 0.45 | 13.6 | B | T | 0.49 | 14.2 | B | T | 0.52 | 14.6 | B | T | 0.54 | 15.0 | B | T | 0.60 | 16.1 | B |
|  | Intersection |  | 23.0 | C | Intersection |  | 26.8 | C | Intersection |  | 21.7 | C | Intersection |  | 23.7 | C | Intersection |  | 30.2 | C | Intersection |  | 33.6 | C |
| York Avenue & East 61st Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | L | 0.23 | 24.0 | C | L | 0.23 | 24.1 | C | L | 0.36 | 29.0 | C | L | 0.37 | 29.1 | C | L | 0.34 | 28.6 | C | L | 0.35 | 28.8 | C |
|  | LTR | 0.71 | 33.4 | C | LTR | 0.75 | 34.6 | C | LTR | 0.62 | 33.3 | C | LTR | 0.65 | 34.0 | C | LTR | 0.60 | 32.7 | C | LTR | 0.62 | 33.3 | C |
|  | R | 0.69 | 36.5 | D | R | 0.80 | 43.1 | D | R | 0.59 | 35.3 | D | R | 0.64 | 37.0 | D | R | 0.62 | 36.3 | D | R | 0.66 | 38.4 | D |
| Northbound | LT | 0.81 | 28.6 | C | LT | 0.84 | 29.7 | C | LT | 0.42 | 17.0 | B | LT | 0.43 | 17.1 | B | LT | 0.55 | 19.0 | B | LT | 0.57 | 19.3 | B |
| Southbound | TR | 0.45 | 19.6 | B | TR | 0.47 | 19.8 | B | TR | 0.84 | 29.1 | C | TR | 0.57 | 19.3 | B | TR | 0.52 | 18.5 | B | TR | 0.55 | 18.9 | B |
|  | Intersection |  | 28.1 | C | Intersection |  | 29.7 | C | Intersection |  | 27.4 | C | Intersection |  | 24.1 | C | Intersection |  | 23.4 | C | Intersection |  | 24.0 | C |
| First Avenue & East 74th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 0.53 | 27.6 | C | LT | 0.55 | 28.2 | C | LT | 0.66 | 31.8 | C | LT | 0.69 | 32.9 | C | LT | 0.59 | 29.0 | C | LT | 0.61 | 29.8 | C |
| Northbound | TR | 0.76 | 18.1 | B | TR | 0.78 | 18.6 | B | TR | 0.65 | 15.8 | B | TR | 0.67 | 16.2 | B | TR | 0.78 | 18.3 | B | TR | 0.80 | 18.9 | B |
|  | Intersection |  | 19.1 | B | Intersection |  | 19.7 | B | Intersection |  | 18.3 | B | Intersection |  | 18.9 | B | Intersection |  | 19.6 | B | Intersection |  | 20.2 | C |
| First Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | TR | 0.31 | 23.3 | C | TR | 0.33 | 23.6 | C | TR | 0.30 | 23.2 | C | TR | 0.36 | 24.3 | C | TR | 0.35 | 24.2 | C | TR | 0.44 | 26.1 | C |
| Northbound | LT | 0.77 | 18.3 | B | LT | 0.79 | 18.7 | B | LT | 0.69 | 16.6 | B | LT | 0.71 | 16.9 | B | LT | 0.78 | 18.5 | B | LT | 0.80 | 19.0 | B |
|  | Intersection |  | 18.6 | B | Intersection |  | 19.1 | B | Intersection |  | 17.1 | B | Intersection |  | 17.6 | B | Intersection |  | 18.8 | B | Intersection |  | 19.5 | B |
| First Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 0.96 | 52.0 | D | LT | 0.98 | 57.1 | E | LT | 0.73 | 30.2 | C | LT | 0.75 | 30.9 | C | LT | 0.71 | 29.5 | C | LT | 0.73 | 30.5 | C |
| Westbound | TR | 0.44 | 21.8 | C | TR | 0.45 | 21.9 | C | TR | 0.39 | 21.0 | C | TR | 0.40 | 21.1 | C | TR | 0.40 | 21.2 | C | TR | 0.42 | 21.4 | C |
| Northbound | L | 0.59 | 45.0 | D | L | 0.60 | 45.4 | D | L | 0.57 | 44.4 | D | L | 0.58 | 44.8 | D | L | 0.47 | 39.9 | D | L | 0.48 | 40.1 | D |
|  | TR | 0.74 | 19.8 | B | TR | 0.76 | 20.3 | C | TR | 0.73 | 19.7 | B | TR | 0.75 | 20.1 | C | TR | 0.74 | 19.7 | B | TR | 0.76 | 20.1 | C |
|  | Intersection |  | 27.8 | C | Intersection |  | 29.1 | C | Intersection |  | 22.9 | C | Intersection |  | 23.3 | C | Intersection |  | 22.2 | C | Intersection |  | 22.7 | C |

| Table 9-45 (cont’d) 2012 Existing and 2019 No Build Conditions Level of Service Analysis Signalized Intersections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weekday AM |  |  |  |  |  |  |  | Weekday Midday |  |  |  |  |  |  |  | Weekday PM |  |  |  |  |  |  |  |
|  | 2012 Existing |  |  |  | 2019 No Build |  |  |  | 2012 Existing |  |  |  | 2019 No Build |  |  |  | 2012 Existing |  |  |  | 2019 No Build |  |  |  |
|  | Lane | v/c | Delay |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |
| Intersection | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |
| First Avenue & East 66th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | TR | 0.67 | 33.5 | C | TR | 0.69 | 34.7 | C | TR | 0.57 | 30.1 | C | TR | 0.62 | 31.9 | C | TR | 0.51 | 27.5 | C | TR | 0.56 | 28.9 | C |
| Northbound | LT | 0.93 | 26.4 | C | LT | 0.97 | 32.4 | C | LT | 0.91 | 24.5 | C | LT | 0.93 | 27.1 | C | LT | 0.83 | 20.0+ | C | LT | 0.85 | 21.1 | C |
|  | Intersection |  | 27.1 | C | Intersection |  | 32.6 | C | Intersection |  | 25.0 | C | Intersection |  | 27.5 | C | Intersection |  | 20.7 | C | Intersection |  | 21.9 | C |
| First Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 0.92 | 56.8 | E | LT | 1.01 | 76.4 | E | LT | 0.94 | 58.8 | E | LT | 0.99 | 69.4 | E | LT | 1.04 | 85.4 | F | LT | 1.10 | 101.8 | F |
| Northbound | TR | 0.87 | 21.8 | C | TR | 0.91 | 24.2 | C | TR | 0.79 | 19.0 | B | TR | 0.82 | 19.8 | B | TR | 0.80 | 18.9 | B | TR | 0.82 | 19.8 | B |
|  | Intersection |  | 26.7 | C | Intersection |  | 31.8 | C | Intersection |  | 25.3 | C | Intersection |  | 27.9 | C | Intersection |  | 29.8 | C | Intersection |  | 33.4 | C |
| Second Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LT | 0.51 | 27.6 | C | LT | 0.54 | 28.6 | C | LT | 0.53 | 28.0 | C | LT | 0.60 | 30.1 | C | LT | 0.33 | 23.8 | C | LT | 0.42 | 25.5 | C |
| Southbound | TR | 0.47 | 13.0 | B | TR | 0.51 | 13.6 | B | TR | 0.47 | 13.1 | B | TR | 0.51 | 13.6 | B | TR | 0.46 | 13.0 | B | TR | 0.51 | 13.6 | B |
|  | Intersection |  | 14.9 | B | Intersection |  | 15.6 | B | Intersection |  | 15.3 | B | Intersection |  | 16.2 | B | Intersection |  | 14.0 | B | Intersection |  | 14.9 | B |
| Second Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | TR | 0.63 | 24.4 | C | TR | 0.64 | 24.7 | C | TR | 0.47 | 21.6 | C | TR | 0.49 | 21.8 | C | TR | 0.58 | 23.5 | C | TR | 0.60 | 23.7 | C |
| Westbound | LT | 0.63 | 25.1 | C | LT | 0.64 | 25.6 | C | LT | 0.47 | 21.8 | C | LT | 0.48 | 22.0 | C | LT | 0.44 | 21.2 | C | LT | 0.47 | 21.8 | C |
| Southbound | LTR | 0.52 | 16.4 | B | LTR | 0.55 | 16.9 | B | LTR | 0.53 | 16.6 | B | LTR | 0.56 | 17.1 | B | LTR | 0.52 | 16.5 | B | LTR | 0.56 | 17.1 | B |
|  | Intersection |  | 20.3 | C | Intersection |  | 20.7 | C | Intersection |  | 18.6 | B | Intersection |  | 19.0 | B | Intersection |  | 19.1 | B | Intersection |  | 19.6 | B |
| Second Avenue & East 66th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LT | 0.48 | 21.1 | C | LT | 0.50 | 21.4 | C | LT | 0.60 | 23.5 | C | LT | 0.62 | 24.0 | C | LT | 0.49 | 21.2 | C | LT | 0.52 | 21.7 | C |
| Southbound | TR | 0.49 | 16.7 | B | TR | 0.54 | 17.4 | B | TR | 0.49 | 16.6 | B | TR | 0.54 | 17.3 | B | TR | 0.47 | 16.4 | B | TR | 0.55 | 17.4 | B |
|  | Intersection |  | 17.9 | B | Intersection |  | 18.4 | B | Intersection |  | 18.7 | B | Intersection |  | 19.3 | B | Intersection |  | 17.7 | B | Intersection |  | 18.6 | B |
| Second Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | TR | 0.60 | 26.2 | C | TR | 0.64 | 27.2 | C | TR | 0.47 | 23.5 | C | TR | 0.49 | 23.8 | C | TR | 0.67 | 27.7 | C | TR | 0.70 | 28.6 | C |
| Southbound | LT | 0.60 | 20.0+ | C | LT | 0.62 | 20.4 | C | LT | 0.61 | 20.3 | C | LT | 0.64 | 20.7 | C | LT | 0.57 | 19.6 | B | LT | 0.62 | 20.4 | C |
|  | Intersection |  | 21.7 | C | Intersection |  | 22.3 | C | Intersection |  | 21.0 | C | Intersection |  | 21.4 | C | Intersection |  | 22.0 | C | Intersection |  | 22.8 | C |
| Notes: L = Left Turn, T = Through, R = Right Turn, DefL = Defacto Left Turn, LOS = Level of Service |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 9-46 2019 No Build and Build Conditions Level of Service Analysis Signalized Intersections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weekday AM |  |  |  |  |  |  |  |  | Weekday Midday |  |  |  |  |  |  |  |  | Weekday PM |  |  |  |  |  |  |  |  |
|  | 2019 No Build |  |  |  | 2019 Build |  |  |  |  | 2019 No Build |  |  |  | 2019 Build |  |  |  |  | 2019 No Build |  |  |  | 2019 Build |  |  |  |  |
|  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |  |
| Intersection | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |  | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |  | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |  |
| York Avenue & East 79th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 1.06 | 95.5 | F | LTR | 1.12 | 114.3 | F | + | LTR | 0.86 | 54.1 | D | LTR | 0.91 | 60.1 | E | + | LTR | 1.02 | 82.5 | F | LTR | 1.05 | 90.2 | F | + |
| Westbound | LTR | 0.23 | 32.4 | C | LTR | 0.23 | 32.5 | C |  | LTR | 0.34 | 34.5 | C | LTR | 0.34 | 34.6 | C |  | LTR | 0.42 | 36.1 | D | LTR | 0.42 | 36.2 | D |  |
| Northbound | - | - | - | - | - | - | - | - |  | - | - | - | - | - | - | - | - |  | - | - | - | - | DefL | 0.89 | 81.3 | F |  |
|  | LTR | 1.13 | 107.2 | F | LTR | 1.17 | 121.1 | F | + | LTR | 0.97 | 54.7 | D | LTR | 1.07 | 81.8 | F | + | LTR | 1.21 | 137.2 | F | - | - | - | - |  |
|  | - | - | - | - | - | - | - | - |  | - | - | - | - | - | - | - | - |  | - | - | - | - | TR | 1.39 | 217.5 | F | + |
| Southbound | TR | 0.93 | 51.7 | D | TR | 0.96 | 56.2 | E |  | TR | 0.79 | 39.5 | D | TR | 0.82 | 40.7 | D |  | TR | 0.92 | 50.5 | D | TR | 0.93 | 52.7 | D |  |
|  | Intersection |  | 80.8 | F | Intersection |  | 92.0 | F |  | Intersection |  | 47.9 | D | Intersection |  | 59.7 | E |  | Intersection |  | 87.5 | F | Intersection |  | 110.0 | F |  |
| York Avenue & East 76th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 0.80 | 58.5 | E | LTR | 0.80 | 58.5 | E |  | LTR | 0.65 | 45.6 | D | LTR | 0.65 | 45.6 | D |  | LTR | 1.02 | 98.1 | F | LTR | 1.02 | 98.1 | F |  |
| Westbound | LR | 0.74 | 55.7 | E | LR | 0.74 | 56.0 | E |  | LR | 0.23 | 32.9 | C | LR | 0.31 | 34.6 | C |  | LR | 0.32 | 34.8 | C | LR | 0.49 | 40.4 | D |  |
| Northbound | TR | 0.45 | 13.6 | B | TR | 0.47 | 14.0 | B |  | TR | 0.45 | 13.6 | B | TR | 0.49 | 14.2 | B |  | TR | 0.52 | 14.7 | B | TR | 0.56 | 15.4 | B |  |
| Southbound | LT | 0.72 | 20.0- | B | LT | 0.84 | 26.3 | C |  | LT | 0.57 | 15.6 | B | LT | 0.62 | 16.6 | B |  | LT | 0.57 | 15.8 | B | LT | 0.61 | 16.6 | B |  |
|  | Intersection |  | 25.3 | C | Intersection |  | 28.0 | C |  | Intersection |  | 18.7 | B | Intersection |  | 19.4 | B |  | Intersection |  | 28.1 | C | Intersection |  | 28.6 | C |  |
| York Avenue & East 75th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.30 | 33.9 | C | LTR | 0.30 | 33.9 | C |  | LTR | 0.10 | 30.2 | C | LTR | 0.10 | 30.2 | C |  | LTR | 0.08 | 29.8 | C | LTR | 0.08 | 29.9 | C |  |
| Northbound | LTR | 0.85 | 27.3 | C | LTR | 0.95 | 40.7 | D |  | LTR | 0.87 | 29.5 | C | LTR | 0.98 | 47.0 | D | + | LTR | 0.64 | 17.2 | B | LTR | 0.73 | 20.1 | C |  |
| Southbound | LTR | 0.79 | 22.4 | C | LTR | 0.82 | 23.9 | C |  | LTR | 0.67 | 18.0 | B | LTR | 0.71 | 19.3 | B |  | LTR | 0.71 | 19.0 | B | LTR | 0.74 | 20.0+ | C |  |
|  | Intersection |  | 25.2 | C | Intersection |  | 32.1 | C |  | Intersection |  | 23.8 | C | Intersection |  | 33.0 | C |  | Intersection |  | 18.3 | B | Intersection |  | 20.2 | C |  |
| York Avenue & East 74th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LTR | 0.56 | 36.0 | D | LTR | 0.76 | 46.6 | D | + | LTR | 0.59 | 36.7 | D | LTR | 0.84 | 52.9 | D | + | LTR | 0.59 | 36.9 | D | LTR | 0.81 | 51.1 | D | + |
| Westbound | LR | 0.06 | 25.6 | C | LR | 0.33 | 31.5 | C |  | LR | 0.09 | 26.0 | C | LR | 1.03 | 111.2 | F | + | LR | 0.09 | 26.1 | C | LR | 0.85 | 68.6 | E | + |
| Northbound | TR | 0.57 | 18.8 | B | TR | 0.63 | 20.1 | C |  | TR | 0.56 | 18.6 | B | TR | 0.65 | 20.6 | C |  | TR | 0.55 | 18.3 | B | TR | 0.60 | 19.6 | B |  |
| Southbound | LT | 0.74 | 23.4 | C | LT | 0.81 | 27.1 | C |  | LT | 0.57 | 18.9 | B | LT | 0.74 | 24.1 | C |  | LT | 0.71 | 22.3 | C | LT | 0.82 | 27.4 | C |  |
|  | Intersection |  | 23.1 | C | Intersection |  | 27.1 | C |  | Intersection |  | 21.4 | C | Intersection |  | 35.0- | C |  | Intersection |  | 22.7 | C | Intersection |  | 30.7 | C |  |
| York Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.20 | 45.9 | D | LTR | 0.24 | 46.9 | D |  | LTR | 0.40 | 51.3 | D | LTR | 0.50 | 55.5 | E |  | LTR | 0.43 | 52.3 | D | LTR | 0.56 | 58.4 | E | + |
| Northbound | LTR | 1.04 | 71.4 | E | LTR | 1.17 | 119.9 | F | + | LTR | 0.89 | 40.3 | D | LTR | 1.01 | 61.5 | E | + | LTR | 1.15 | 111.0 | F | LTR | 1.28 | 164.9 | F | + |
| Southbound | DefL | 1.00 | 81.9 | F | DefL | 1.09 | 112.8 | F | + | - | - | - | - | - | - | - | - |  | DefL | 1.17 | 137.4 | F | DefL | 1.35 | 211.1 | F | + |
|  | - | - | - | - | - | - | - | - |  | LTR | 1.01 | 56.7 | E | LTR | 1.23 | 138.3 | F | + | - | - | - | - | - | - | - | - |  |
|  | TR | 1.06 | 75.4 | E | TR | 1.16 | 113.2 | F | + | - | - | - | - | - | - | - | - |  | TR | 1.00 | 58.1 | E | TR | 1.17 | 113.6 | F | + |
|  | Intersection |  | 73.7 | E | Intersection |  | 115.1 | F |  | Intersection |  | 48.7 | D | Intersection |  | 99.2 | F |  | Intersection |  | 95.1 | F | Intersection |  | 149.8 | F |  |
| York Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | DefL | 1.05 | 109.1 | F | DefL | 1.13 | 136.1 | F | + | DefL | 0.73 | 50.7 | D | DefL | 0.81 | 59.9 | E | + | DefL | 0.72 | 49.0 | D | DefL | 0.82 | 60.5 | E | + |
|  | TR | 0.54 | 36.3 | D | TR | 0.56 | 37.1 | D |  | TR | 0.51 | 35.6 | D | TR | 0.53 | 36.4 | D |  | TR | 0.47 | 34.8 | C | TR | 0.50 | 35.6 | D |  |
|  | R | 0.47 | 36.7 | D | R | 0.50 | 38.2 | D |  | R | 0.48 | 37.3 | D | R | 0.52 | 39.7 | D |  | R | 0.47 | 36.8 | D | R | 0.50 | 38.8 | D |  |
| Westbound | LTR | 0.46 | 33.1 | C | LTR | 0.47 | 33.3 | C |  | LTR | 0.48 | 34.1 | C | LTR | 0.54 | 35.9 | D |  | LTR | 0.31 | 29.7 | C | LTR | 0.42 | 32.3 | C |  |
| Northbound | LTR | 1.09 | 83.5 | F | LTR | 1.22 | 137.0 | F | + | LTR | 0.96 | 46.3 | D | LTR | 1.06 | 72.2 | E | + | LTR | 1.10 | 86.0 | F | LTR | 1.17 | 113.8 | F | + |
| Southbound | LTR | 0.64 | 20.9 | C | LTR | 0.71 | 23.4 | C |  | LTR | 0.57 | 19.1 | B | LTR | 0.65 | 21.3 | C |  | LTR | 0.54 | 18.4 | B | LTR | 0.57 | 19.2 | B |  |
|  | Intersection |  | 59.1 | E | Intersection |  | 86.9 | F |  | Intersection |  | 36.2 | D | Intersection |  | 48.9 | D |  | Intersection |  | 55.5 | E | Intersection |  | 69.8 | E |  |
| York Avenue & East 71st Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.83 | 39.3 | D | LTR | 0.85 | 40.6 | D |  | LTR | 0.68 | 32.7 | C | LTR | 0.68 | 32.9 | C |  | LTR | 0.72 | 34.1 | C | LTR | 0.73 | 34.6 | C |  |
| Northbound | LTR | 0.89 | 38.2 | D | LTR | 0.98 | 52.4 | D | + | LTR | 0.64 | 24.2 | C | LTR | 0.70 | 26.1 | C |  | LTR | 0.77 | 28.9 | C | LTR | 0.82 | 31.7 | C |  |
| Southbound | LTR | 0.61 | 23.4 | C | LTR | 0.62 | 23.6 | C |  | LTR | 0.61 | 23.3 | C | LTR | 0.63 | 23.7 | C |  | LTR | 0.61 | 23.3 | C | LTR | 0.64 | 24.0 | C |  |
|  | Intersection |  | 34.3 | D | Intersection |  | 40.4 | D |  | Intersection |  | 26.6 | C | Intersection |  | 27.4 | C |  | Intersection |  | 28.8 | C | Intersection |  | 30.1 | C |  |
| York Avenue & East 66th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LTR | 0.03 | 29.1 | C | LTR | 0.03 | 29.1 | C |  | LTR | 0.04 | 29.2 | C | LTR | 0.04 | 29.2 | C |  | LTR | 0.03 | 29.1 | C | LTR | 0.03 | 29.1 | C |  |
| Northbound | LTR | 0.85 | 24.5 | C | LTR | 0.91 | 29.1 | C |  | LTR | 1.07 | 74.2 | E | LTR | 1.15 | 103.8 | F | + | DefL | 0.54 | 29.7 | C | DefL | 0.57 | 32.3 | C |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TR | 0.43 | 13.3 | B | TR | 0.46 | 13.7 | B |  |
| Southbound | LTR | 0.74 | 27.8 | C | LTR | 0.75 | 28.5 | C |  | LTR | 0.82 | 31.6 | C | LTR | 0.86 | 33.9 | C |  | LTR | 0.96 | 46.2 | D | LTR | 1.01 | 56.4 | E | + |
|  | Intersection |  | 25.7 | C | Intersection |  | 28.9 | C |  | Intersection |  | 52.7 | D | Intersection |  | 69.2 | E |  | Intersection |  | 33.6 | C | Intersection |  | 39.5 | D |  |
| York Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LR | 1.03 | 97.6 | F | LR | 1.10 | 118.1 | F | + | LR | 0.92 | 71.0 | E | LR | 0.98 | 83.4 | F | + | LR | 1.10 | 118.3 | F | LR | 1.14 | 131.3 | F | + |
| Northbound | T | 0.53 | 14.4 | B | T | 0.56 | 14.9 | B |  | T | 0.61 | 16.2 | B | T | 0.64 | 17.0 | B |  | T | 0.31 | 11.7 | B | T | 0.33 | 11.9 | B |  |
| Southbound | T | 0.45 | 13.6 | B | T | 0.45 | 13.6 | B |  | T | 0.52 | 14.6 | B | T | 0.53 | 14.7 | B |  | T | 0.60 | 16.1 | B | T | 0.61 | 16.3 | B |  |
|  | Intersection |  | 26.8 | C | Intersection |  | 30.5 | C |  | Intersection |  | 23.7 | C | Intersection |  | 26.2 | C |  | Intersection |  | 33.6 | C | Intersection |  | 36.2 | D |  |
| York Avenue & East 61st Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | L | 0.23 | 24.1 | C | L | 0.23 | 24.1 | C |  | L | 0.37 | 29.1 | C | L | 0.37 | 29.1 | C |  | L | 0.35 | 28.8 | C | L | 0.35 | 28.8 | C |  |
|  | LTR | 0.75 | 34.6 | C | LTR | 0.77 | 35.4 | D |  | LTR | 0.65 | 34.0 | C | LTR | 0.66 | 34.3 | C |  | LTR | 0.62 | 33.3 | C | LTR | 0.63 | 33.5 | C |  |
|  | R | 0.80 | 43.1 | D | R | 0.88 | 50.5 | D | + | R | 0.64 | 37.0 | D | R | 0.68 | 39.2 | D |  | R | 0.66 | 38.4 | D | R | 0.70 | 40.1 | D |  |
| Northbound | LT | 0.84 | 29.7 | C | LT | 0.85 | 30.2 | C |  | LT | 0.43 | 17.1 | B | LT | 0.44 | 17.2 | B |  | LT | 0.57 | 19.3 | B | LT | 0.57 | 19.4 | B |  |
| Southbound | TR | 0.47 | 19.8 | B | TR | 0.47 | 19.9 | B |  | TR | 0.57 | 19.3 | B | TR | 0.57 | 19.4 | B |  | TR | 0.55 | 18.9 | B | TR | 0.55 | 19.0 | B |  |
|  | Intersection |  | 29.7 | C | Intersection |  | 31.1 | C |  | Intersection |  | 24.1 | C | Intersection |  | 24.5 | C |  | Intersection |  | 24.0 | C | Intersection |  | 24.3 | C |  |
| First Avenue & East 74th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 0.55 | 28.2 | C | LT | 0.64 | 31.5 | C |  | LT | 0.69 | 32.9 | C | LT | 0.77 | 37.8 | D |  | LT | 0.61 | 29.8 | C | LT | 0.67 | 32.1 | C |  |
| Northbound | TR | 0.78 | 18.6 | B | TR | 0.81 | 19.3 | B |  | TR | 0.67 | 16.2 | B | TR | 0.70 | 16.7 | B |  | TR | 0.80 | 18.9 | B | TR | 0.81 | 19.5 | B |  |
|  | Intersection |  | 19.7 | B | Intersection |  | 20.8 | C |  | Intersection |  | 18.9 | B | Intersection |  | 20.3 | C |  | Intersection |  | 20.2 | C | Intersection |  | 21.1 | C |  |
| First Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | TR | 0.33 | 23.6 | C | TR | 0.39 | 24.8 | C |  | TR | 0.36 | 24.3 | C | TR | 0.51 | 27.4 | C |  | TR | 0.44 | 26.1 | C | TR | 0.66 | 33.2 | C |  |
| Northbound | LT | 0.79 | 18.7 | B | LT | 0.80 | 19.2 | B |  | LT | 0.71 | 16.9 | B | LT | 0.72 | 17.2 | B |  | LT | 0.80 | 19.0 | B | LT | 0.81 | 19.2 | B |  |
|  | Intersection |  | 19.1 | B | Intersection |  | 19.6 | B |  | Intersection |  | 17.6 | B | Intersection |  | 18.4 | B |  | Intersection |  | 19.5 | B | Intersection |  | 20.7 | C |  |

| Table 9-46 (cont’d) 2019 No Build and Build Conditions Level of Service Analysis Signalized Intersections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weekday AM |  |  |  |  |  |  |  |  | Weekday Midday |  |  |  |  |  |  |  |  | Weekday PM |  |  |  |  |  |  |  |  |
|  | 2019 No Build |  |  |  | 2019 Build |  |  |  |  | 2019 No Build |  |  |  | 2019 Build |  |  |  |  | 2019 No Build |  |  |  | 2019 Build |  |  |  |  |
|  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |  | Lane | v/c | Delay |  | Lane | v/c | Delay |  |  |
| Intersection | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |  | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |  | Group | Ratio | (sec) | LOS | Group | Ratio | (sec) | LOS |  |
| First Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | - | - | - | - | - | - | - | - |  | - | - | - | - | - | - | - | - |  | - | - | - | - | DefL | 0.77 | 46.8 | D | + |
|  | LT | 0.98 | 57.1 | E | LT | 0.99 | 59.6 | E |  | LT | 0.75 | 30.9 | C | LT | 0.76 | 31.5 | C |  | LT | 0.73 | 30.5 | C | - | - | - | - |  |
|  | - | - | - | - | - | - | - | - |  | - | - | - | - | - | - | - | - |  | - | - | - | - | T | 0.62 | 27.8 | C |  |
| Westbound | TR | 0.45 | 21.9 | C | TR | 0.46 | 22.0 | C |  | TR | 0.40 | 21.1 | C | TR | 0.41 | 21.3 | C |  | TR | 0.42 | 21.4 | C | TR | 0.45 | 21.9 | C |  |
| Northbound | L | 0.60 | 45.4 | D | L | 0.60 | 45.4 | D |  | L | 0.58 | 44.8 | D | L | 0.58 | 44.8 | D |  | L | 0.48 | 40.1 | D | L | 0.48 | 40.1 | D |  |
|  | TR | 0.76 | 20.3 | C | TR | 0.79 | 21.1 | C |  | TR | 0.75 | 20.1 | C | TR | 0.77 | 20.8 | C |  | TR | 0.76 | 20.1 | C | TR | 0.77 | 20.5 | C |  |
|  | Intersection |  | 29.1 | C | Intersection |  | 30.1 | C |  | Intersection |  | 23.3 | C | Intersection |  | 23.8 | C |  | Intersection |  | 22.7 | C | Intersection |  | 23.6 | C |  |
| First Avenue & East 66th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | TR | 0.69 | 34.7 | C | TR | 0.70 | 35.1 | D |  | TR | 0.62 | 31.9 | C | TR | 0.65 | 33.1 | C |  | TR | 0.56 | 28.9 | C | TR | 0.59 | 30.0 | C |  |
| Northbound | LT | 0.97 | 32.4 | C | LT | 1.00 | 37.6 | D |  | LT | 0.93 | 27.1 | C | LT | 0.95 | 29.3 | C |  | LT | 0.85 | 21.1 | C | LT | 0.87 | 21.7 | C |  |
|  | Intersection |  | 32.6 | C | Intersection |  | 37.3 | D |  | Intersection |  | 27.5 | C | Intersection |  | 29.7 | C |  | Intersection |  | 21.9 | C | Intersection |  | 22.5 | C |  |
| First Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | LT | 1.01 | 76.4 | E | LT | 1.07 | 92.0 | F | + | LT | 0.99 | 69.4 | E | LT | 1.03 | 80.2 | F | + | LT | 1.10 | 101.8 | F | LT | 1.13 | 113.2 | F | + |
| Northbound | TR | 0.91 | 24.2 | C | TR | 0.93 | 26.3 | C |  | TR | 0.82 | 19.8 | B | TR | 0.84 | 20.5 | C |  | TR | 0.82 | 19.8 | B | TR | 0.84 | 20.2 | C |  |
|  | Intersection |  | 31.8 | C | Intersection |  | 36.1 | D |  | Intersection |  | 27.9 | C | Intersection |  | 30.4 | C |  | Intersection |  | 33.4 | C | Intersection |  | 35.9 | D |  |
| Second Avenue & East 73rd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LT | 0.54 | 28.6 | C | LT | 0.64 | 32.5 | C |  | LT | 0.60 | 30.1 | C | LT | 0.78 | 39.7 | D |  | LT | 0.42 | 25.5 | C | LT | 0.61 | 30.9 | C |  |
| Southbound | TR | 0.51 | 13.6 | B | TR | 0.51 | 13.6 | B |  | TR | 0.51 | 13.6 | B | TR | 0.51 | 13.6 | B |  | TR | 0.51 | 13.6 | B | TR | 0.51 | 13.6 | B |  |
|  | Intersection |  | 15.6 | B | Intersection |  | 16.3 | B |  | Intersection |  | 16.2 | B | Intersection |  | 18.4 | B |  | Intersection |  | 14.9 | B | Intersection |  | 16.1 | B |  |
| Second Avenue & East 72nd Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | TR | 0.64 | 24.7 | C | TR | 0.64 | 24.8 | C |  | TR | 0.49 | 21.8 | C | TR | 0.50 | 22.0 | C |  | TR | 0.60 | 23.7 | C | TR | 0.60 | 23.9 | C |  |
| Westbound | LT | 0.64 | 25.6 | C | LT | 0.66 | 26.1 | C |  | LT | 0.48 | 22.0 | C | LT | 0.50 | 22.6 | C |  | LT | 0.47 | 21.8 | C | LT | 0.56 | 23.7 | C |  |
| Southbound | LTR | 0.55 | 16.9 | B | LTR | 0.56 | 17.0 | B |  | LTR | 0.56 | 17.1 | B | LTR | 0.58 | 17.3 | B |  | LTR | 0.56 | 17.1 | B | LTR | 0.58 | 17.4 | B |  |
|  | Intersection |  | 20.7 | C | Intersection |  | 20.9 | C |  | Intersection |  | 19.0 | B | Intersection |  | 19.2 | B |  | Intersection |  | 19.6 | B | Intersection |  | 20.1 | C |  |
| Second Avenue & East 66th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Westbound | LT | 0.50 | 21.4 | C | LT | 0.50 | 21.4 | C |  | LT | 0.62 | 24.0 | C | LT | 0.63 | 24.3 | C |  | LT | 0.52 | 21.7 | C | LT | 0.53 | 21.9 | C |  |
| Southbound | TR | 0.54 | 17.4 | B | TR | 0.55 | 17.5 | B |  | TR | 0.54 | 17.3 | B | TR | 0.55 | 17.6 | B |  | TR | 0.55 | 17.4 | B | TR | 0.58 | 17.9 | B |  |
|  | Intersection |  | 18.4 | B | Intersection |  | 18.5 | B |  | Intersection |  | 19.3 | B | Intersection |  | 19.5 | B |  | Intersection |  | 18.6 | B | Intersection |  | 19.0 | B |  |
| Second Avenue & East 65th Street |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eastbound | TR | 0.64 | 27.2 | C | TR | 0.67 | 27.8 | C |  | TR | 0.49 | 23.8 | C | TR | 0.50 | 24.1 | C |  | TR | 0.70 | 28.6 | C | TR | 0.71 | 29.0 | C |  |
| Southbound | LT | 0.62 | 20.4 | C | LT | 0.63 | 20.5 | C |  | LT | 0.64 | 20.7 | C | LT | 0.66 | 21.0 | C |  | LT | 0.62 | 20.4 | C | LT | 0.66 | 21.0 | C |  |
|  | Intersection |  | 22.3 | C | Intersection |  | 22.6 | C |  | Intersection |  | 21.4 | C | Intersection |  | 21.7 | C |  | Intersection |  | 22.8 | C | Intersection |  | 23.3 | C |  |
| Notes: L = Left Turn, T = Through, R = Right Turn, DefL = Defacto Left Turn, LOS = Level of Service + Denotes a significant adverse traffic impact |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 9-47 2012 Existing Conditions Sidewalk Analysis |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Intersection No. | Location | Sidewalk | Effective Width (ft) | 1 Hour Two-Way Volume | Platoon Flow |  |
|  |  |  |  |  | PMF | LOS |
| AM Peak Period |  |  |  |  |  |  |
| 2 | York Avenue between East 74th Street and East 75th Street | East | 10.0 | 779 | 1.50 | B |
|  | East 74th Street between York Avenue and the FDR | South | 6.0 | 86 | 0.30 | A |
|  | York Avenue between East 73rd Street and East 74th Street | West | 9.0 | 557 | 1.29 | B |
|  | East 74th Street between York Avenue and First Avenue | South | 4.0 | 282 | 1.37 | B |
| 3 | York Avenue between East 73rd Street and East 74th Street | East | 9.0 | 728 | 1.52 | B |
|  | East 73rd Street between York Avenue and the FDR | North | 2.0 | 82 | 0.73 | B |
|  | York Avenue between East 73rd Street and East 72nd Street | East | 9.0 | 1019 | 2.20 | B |
|  |  | West | 10.0 | 487 | 0.86 | B |
| 4 | York Avenue between East 72nd Street and East 71st Street | East | 14.0 | 948 | 1.41 | B |
|  |  | West | 10.0 | 551 | 1.06 | B |
|  | East 72nd Street between York Avenue and First Avenue | South | 13.0 | 428 | 0.67 | B |
|  |  | North | 13.0 | 292 | 0.47 | A |
| Midday Peak Period |  |  |  |  |  |  |
| 2 | York Avenue between East 74th Street and East 75th Street | East | 10.0 | 221 | 0.46 | A |
|  | East 74th Street between York Avenue and the FDR | South | 6.0 | 150 | 0.44 | A |
|  | York Avenue between East 73rd Street and East 74th Street | West | 9.0 | 336 | 0.78 | B |
|  | East 74th Street between York Avenue and First Avenue | South | 4.0 | 230 | 1.20 | B |
| 3 | York Avenue between East 73rd Street and East 74th Street | East | 9.0 | 652 | 1.32 | B |
|  | East 73rd Street between York Avenue and the FDR | North | 2.0 | 66 | 0.63 | B |
|  | York Avenue between East 73rd Street and East 72nd Street | East | 9.0 | 793 | 1.64 | B |
|  |  | West | 10.0 | 355 | 0.71 | B |
| 4 | York Avenue between East 72nd Street and East 71st Street | East | 14.0 | 726 | 0.90 | B |
|  |  | West | 10.0 | 505 | 0.97 | B |
|  | East 72nd Street between York Avenue and First Avenue | South | 13.0 | 516 | 0.74 | B |
|  |  | North | 13.0 | 288 | 0.46 | A |
| PM Peak Period |  |  |  |  |  |  |
| 2 | York Avenue between East 74th Street and East 75th Street | East | 10.0 | 579 | 1.11 | B |
|  | East 74th Street between York Avenue and the FDR | South | 6.0 | 75 | 0.26 | A |
|  | York Avenue between East 73rd Street and East 74th Street | West | 9.0 | 564 | 1.12 | B |
|  | East 74th Street between York Avenue and First Avenue | South | 4.0 | 261 | 1.25 | B |
| 3 | York Avenue between East 73rd Street and East 74th Street | East | 9.0 | 598 | 1.19 | B |
|  | East 73rd Street between York Avenue and the FDR | North | 2.0 | 84 | 0.88 | B |
|  | York Avenue between East 73rd Street and East 72nd Street | East | 9.0 | 670 | 1.31 | B |
|  |  | West | 10.0 | 528 | 1.10 | B |
| 4 | York Avenue between East 72nd Street and East 71st Street | East | 14.0 | 624 | 0.92 | B |
|  |  | West | 10.0 | 551 | 0.97 | B |
|  | East 72nd Street between York Avenue and First Avenue | South | 13.0 | 440 | 0.68 | B |
|  |  | North | 13.0 | 411 | 0.65 | B |

| Table 9-48 2012 Existing Conditions Corner Analysis |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intersection No. | Location | Corner | AM Peak Period |  | Midday Peak Period |  | PM Peak Period |  |
|  |  |  | SFP | LOS | SFP | LOS | SFP | LOS |
| 1 | York Avenue and East 75th Street | Northeast | 65.8 | A | 179.4 | A | 113.4 | A |
|  |  | Southwest | 230.0 | A | 486.1 | A | 314.7 | A |
|  |  | Southeast | 91.4 | A | 193.8 | A | 149.8 | A |
| 2 | York Avenue and East 74th Street | Northwest | 213.6 | A | 423.6 | A | 351.7 | A |
|  |  | Northeast | 179.4 | A | 272.6 | A | 227.1 | A |
|  |  | Southwest | 140.4 | A | 223.9 | A | 226.0 | A |
|  |  | Southeast | 186.7 | A | 236.6 | A | 241.7 | A |
| 3 | York Avenue and East 73rd Street | Northwest | 230.8 | A | 322.4 | A | 243.8 | A |
|  |  | Northeast | 171.1 | A | 177.1 | A | 216.5 | A |
|  |  | Southwest | 218.3 | A | 289.9 | A | 225.8 | A |
|  |  | Southeast | 122.7 | A | 124.4 | A | 162.6 | A |
| 4 | York Avenue and East 72nd Street | Northwest | 203.1 | A | 280.6 | A | 225.9 | A |
|  |  | Northeast | 359.3 | A | 355.7 | A | 376.8 | A |
|  |  | Southeast | 365.9 | A | 335.7 | A | 381.5 | A |
| 5 | York Avenue and East 71st Street | Northwest | 118.7 | A | 154.2 | A | 183.9 | A |
|  |  | Northeast | 114.3 | A | 115.5 | A | 119.5 | A |
|  |  | Southwest | 141.8 | A | 144.0 | A | 173.0 | A |
|  |  | Southeast | 137.8 | A | 125.0 | A | 127.5 | A |
| 6 | York Avenue and East 68th Street | Northwest | 76.1 | A | 131.1 | A | 89.0 | A |
| 7 | First Avenue and East 72nd Street | Northwest | 169.5 | A | 175.2 | A | 156.0 | A |
|  |  | Northeast | 176.3 | A | 265.9 | A | 176.0 | A |
|  |  | Southwest | 180.4 | A | 187.0 | A | 144.6 | A |
|  |  | Southeast | 154.1 | A | 259.0 | A | 142.8 | A |
| Note: SFP = square feet per pedestrian |  |  |  |  |  |  |  |  |

| Table 9-49 2012 Existing Conditions Crosswalk Analysis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intersection No. | Location | Crosswalk | Street Width (feet) | Cross-walk Width (feet) | Conditions with conflicting vehicles |  |  |  |  |  |  |  |  |
|  |  |  |  |  | AM |  |  | Midday |  |  | PM |  |  |
|  |  |  |  |  | 2-way Volume | SFP | LOS | 2-way Volume | SFP | LOS | 2-way Volume | SFP | LOS |
| 1 | York Avenue and East 75th Street | East | 34.0 | 20.0 | 946 | 75.6 | A | 436 | 168.4 | A | 548 | 133.5 | A |
| 2 | York Avenue and East 74th Street | East | 34.0 | 18.0 | 720 | 91.0 | A | 542 | 127.3 | A | 532 | 117.1 | A |
|  |  | South | 60.0 | 12.0 | 126 | 181.2 | A | 115 | 199.6 | A | 91 | 249.2 | A |
| 3 | York Avenue and East 73rd Street | North | 60.0 | 12.0 | 42 | 544.5 | A | 70 | 318.8 | A | 54 | 471.0 | A |
|  |  | East | 40.0 | 17.0 | 617 | 108.3 | A | 540 | 136.1 | A | 397 | 157.7 | A |
|  |  | West | 30.0 | 16.0 | 513 | 171.3 | A | 341 | 262.9 | A | 468 | 179.2 | A |
| 4 | York Avenue and East 72nd Street | North | 60.0 | 15.0 | 309 | 83.8 | A | 321 | 96.4 | A | 345 | 77.8 | A |
|  |  | East | 40.0 | 20.0 | 658 | 94.1 | A | 802 | 84.4 | A | 556 | 110.9 | A |
|  |  | West | 55.0 | 17.0 | 684 | 82.6 | A | 442 | 128.0 | A | 524 | 108.1 | A |
| 5 | York Avenue and East 71st Street | East | 34.0 | 19.0 | 726 | 96.7 | A | 720 | 111.7 | A | 687 | 95.4 | A |
|  |  | West | 34.0 | 18.0 | 709 | 90.5 | A | 551 | 120.1 | A | 534 | 128.4 | A |
| Note: SFP = square feet per pedestrian |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 9-50 2019 No Build Condition Sidewalk Analysis |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Intersection No. | Location | Sidewalk | Effective Width (ft) | 1 Hour Two-Way Volume | Platoon Flow |  |
|  |  |  |  |  | PMF | LOS |
| AM Peak Period |  |  |  |  |  |  |
| 2 | York Avenue between East 74th Street and East 75th Street | East | 10.0 | 851 | 1.64 | B |
|  | East 74th Street between York Avenue and the FDR | South | 6.0 | 244 | 0.85 | B |
|  | York Avenue between East 73rd Street and East 74th Street | West | 9.0 | 567 | 1.31 | B |
|  | East 74th Street between York Avenue and First Avenue | South | 4.0 | 317 | 1.54 | B |
| 3 | York Avenue between East 73rd Street and East 74th Street | East | 9.0 | 795 | 1.15 | B |
|  | East 73rd Street between York Avenue and the FDR | North | 2.0 | 309 | 2.76 | B |
|  | York Avenue between East 73rd Street and East 72nd Street | East | 9.0 | 1130 | 2.44 | B |
|  |  | West | 10.0 | 527 | 0.93 | B |
| 4 | York Avenue between East 72nd Street and East 71st Street | East | 14.0 | 1011 | 1.50 | B |
|  |  | West | 10.0 | 590 | 1.14 | B |
|  | East 72nd Street between York Avenue and First Avenue | South | 13.0 | 461 | 0.72 | B |
|  |  | North | 13.0 | 317 | 0.51 | B |
| Midday Peak Period |  |  |  |  |  |  |
| 2 | York Avenue between East 74th Street and East 75th Street | East | 10.0 | 270 | 0.56 | B |
|  | East 74th Street between York Avenue and the FDR | South | 6.0 | 264 | 0.78 | B |
|  | York Avenue between East 73rd Street and East 74th Street | West | 9.0 | 348 | 0.81 | B |
|  | East 74th Street between York Avenue and First Avenue | South | 4.0 | 246 | 1.28 | B |
| 3 | York Avenue between East 73rd Street and East 74th Street | East | 9.0 | 682 | 0.95 | B |
|  | East 73rd Street between York Avenue and the FDR | North | 2.0 | 256 | 2.46 | B |
|  | York Avenue between East 73rd Street and East 72nd Street | East | 9.0 | 905 | 1.87 | B |
|  |  | West | 10.0 | 404 | 0.81 | B |
| 4 | York Avenue between East 72nd Street and East 71st Street | East | 14.0 | 788 | 0.98 | B |
|  |  | West | 10.0 | 544 | 1.05 | B |
|  | East 72nd Street between York Avenue and First Avenue | South | 13.0 | 551 | 0.79 | B |
|  |  | North | 13.0 | 321 | 0.51 | B |
| PM Peak Period |  |  |  |  |  |  |
| 2 | York Avenue between East 74th Street and East 75th Street | East | 10.0 | 695 | 1.33 | B |
|  | East 74th Street between York Avenue and the FDR | South | 6.0 | 372 | 1.29 | B |
|  | York Avenue between East 73rd Street and East 74th Street | West | 9.0 | 597 | 1.18 | B |
|  | East 74th Street between York Avenue and First Avenue | South | 4.0 | 313 | 1.50 | B |
| 3 | York Avenue between East 73rd Street and East 74th Street | East | 9.0 | 680 | 0.94 | B |
|  | East 73rd Street between York Avenue and the FDR | North | 2.0 | 506 | 5.27 | C |
|  | York Avenue between East 73rd Street and East 72nd Street | East | 9.0 | 841 | 1.65 | B |
|  |  | West | 10.0 | 612 | 1.28 | B |
| 4 | York Avenue between East 72nd Street and East 71st Street | East | 14.0 | 707 | 1.04 | B |
|  |  | West | 10.0 | 616 | 1.09 | B |
|  | East 72nd Street between York Avenue and First Avenue | South | 13.0 | 497 | 0.76 | B |
|  |  | North | 13.0 | 470 | 0.74 | B |

| Table 9-51 2019 No Build Condition Corner Analysis |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intersection No. | Location | Corner | AM Peak Period |  | Midday Peak Period |  | PM Peak Period |  |
|  |  |  | SFP | LOS | SFP | LOS | SFP | LOS |
| 1 | York Avenue and East 75th Street | Northeast | 89.4 | A | 245.5 | A | 131.9 | A |
|  |  | Southwest | 2915.4 | A | 2464.8 | A | 1546.3 | A |
|  |  | Southeast | 105.0 | A | 203.2 | A | 145.6 | A |
| 2 | York Avenue and East 74th Street | Northwest | 2920.3 | A | 2196.2 | A | 1989.3 | A |
|  |  | Northeast | 177.5 | A | 246.1 | A | 195.3 | A |
|  |  | Southwest | 491.2 | A | 455.6 | A | 405.9 | A |
|  |  | Southeast | 153.9 | A | 198.1 | A | 156.5 | A |
| 3 | York Avenue and East 73rd Street | Northwest | 198.3 | A | 243.1 | A | 171.4 | A |
|  |  | Northeast | 118.2 | A | 128.8 | A | 107.7 | A |
|  |  | Southwest | 244.2 | A | 325.3 | A | 219.6 | A |
|  |  | Southeast | 108.6 | A | 119.6 | A | 111.4 | A |
| 4 | York Avenue and East 72nd Street | Northwest | 190.4 | A | 255.2 | A | 198.0 | A |
|  |  | Northeast | 328.4 | A | 326.8 | A | 323.9 | A |
|  |  | Southeast | 483.5 | A | 446.8 | A | 508.2 | A |
| 5 | York Avenue and East 71st Street | Northwest | 183.9 | A | 239.0 | A | 236.6 | A |
|  |  | Northeast | 168.0 | A | 166.6 | A | 150.3 | A |
|  |  | Southwest | 188.7 | A | 228.2 | A | 255.1 | A |
|  |  | Southeast | 178.3 | A | 189.2 | A | 177.4 | A |
| 6 | York Avenue and East 68th Street | Northwest | 533.1 | A | 1017.2 | A | 675.8 | A |
| 7 | First Avenue and East 72nd Street | Northwest | 1046.0 | A | 1314.6 | A | 1205.6 | A |
|  |  | Northeast | 2625.8 | A | 1157.5 | A | 1138.0 | A |
|  |  | Southwest | 1389.1 | A | 1543.2 | A | 1855.2 | A |
|  |  | Southeast | 1069.7 | A | 1141.1 | A | 899.6 | A |
| Note: SFP = square feet per pedestrian |  |  |  |  |  |  |  |  |

| Table 9-52 2019 No Build Condition Crosswalk Analysis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intersection No. | Location | Crosswalk | Street Width (feet) | Cross-walk Width (feet) | Conditions with conflicting vehicles |  |  |  |  |  |  |  |  |
|  |  |  |  |  | AM |  |  | Midday |  |  | PM |  |  |
|  |  |  |  |  | 2-way Volume | SFP | LOS | 2-way Volume | SFP | LOS | 2-way Volume | SFP | LOS |
| 1 | York Avenue and East 75th Street | East | 34.0 | 20.0 | 1016 | 70.0 | A | 479 | 152.6 | A | 649 | 111.7 | A |
| 2 | York Avenue and East 74th Street | East | 34.0 | 18.0 | 808 | 78.9 | A | 615 | 109.5 | A | 682 | 88.8 | A |
|  |  | South | 60.0 | 12.0 | 177 | 126.9 | A | 157 | 144.9 | A | 202 | 109.6 | A |
| 3 | York Avenue and East 73rd Street | North | 60.0 | 12.0 | 84 | 268.4 | A | 135 | 161.8 | A | 182 | 134.1 | A |
|  |  | East | 40.0 | 17.0 | 838 | 77.3 | A | 686 | 105.5 | A | 731 | 82.7 | A |
|  |  | West | 30.0 | 16.0 | 554 | 158.2 | A | 393 | 227.0 | A | 558 | 149.1 | A |
| 4 | York Avenue and East 72nd Street | North | 60.0 | 15.0 | 329 | 77.9 | A | 345 | 89.1 | A | 384 | 69.3 | A |
|  |  | East | 40.0 | 20.0 | 731 | 83.9 | A | 881 | 76.4 | A | 668 | 91.4 | A |
|  |  | West | 55.0 | 17.0 | 720 | 78.1 | A | 481 | 117.1 | A | 588 | 95.6 | A |
| 5 | York Avenue and East 71st Street | East | 34.0 | 19.0 | 769 | 90.9 | A | 762 | 105.2 | A | 748 | 87.0 | A |
|  |  | West | 34.0 | 18.0 | 759 | 83.9 | A | 596 | 110.4 | A | 604 | 112.5 | A |
| Note: SFP = square feet per pedestrian |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 9-53 2019 Build Condition Sidewalk Analysis |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Intersection No. | Location | Sidewalk | Effective Width (ft) | 1 Hour Two-Way Volume | Platoon Flow |  |
|  |  |  |  |  | PMF | LOS |
| AM Peak Period |  |  |  |  |  |  |
| 2 | York Avenue between East 74th Street and East 75th Street | East | 10.0 | 1111 | 2.14 | B |
|  | East 74th Street between York Avenue and the FDR | South | 6.0 | 1875 | 6.51 | D |
|  | York Avenue between East 73rd Street and East 74th Street | West | 9.0 | 739 | 1.71 | B |
|  | East 74th Street between York Avenue and First Avenue | South | 4.0 | 471 | 2.28 | B |
| 3 | York Avenue between East 73rd Street and East 74th Street | East | 9.0 | 1563 | 2.26 | B |
|  | East 73rd Street between York Avenue and the FDR | North | 2.0 | 620 | 5.54 | C |
|  | York Avenue between East 73rd Street and East 72nd Street | East | 9.0 | 1574 | 3.40 | C |
|  |  | West | 10.0 | 805 | 1.42 | B |
| 4 | York Avenue between East 72nd Street and East 71st Street | East | 14.0 | 1176 | 1.75 | B |
|  |  | West | 10.0 | 682 | 1.31 | B |
|  | East 72nd Street between York Avenue and First Avenue | South | 13.0 | 656 | 1.02 | B |
|  |  | North | 13.0 | 511 | 0.82 | B |
| Midday Peak Period |  |  |  |  |  |  |
| 2 | York Avenue between East 74th Street and East 75th Street | East | 10.0 | 525 | 1.09 | B |
|  | East 74th Street between York Avenue and the FDR | South | 6.0 | 2333 | 6.91 | D |
|  | York Avenue between East 73rd Street and East 74th Street | West | 9.0 | 679 | 1.57 | B |
|  | East 74th Street between York Avenue and First Avenue | South | 4.0 | 420 | 2.19 | B |
| 3 | York Avenue between East 73rd Street and East 74th Street | East | 9.0 | 1741 | 2.44 | B |
|  | East 73rd Street between York Avenue and the FDR | North | 2.0 | 409 | 3.93 | C |
|  | York Avenue between East 73rd Street and East 72nd Street | East | 9.0 | 1445 | 2.98 | B |
|  |  | West | 10.0 | 761 | 1.53 | B |
| 4 | York Avenue between East 72nd Street and East 71st Street | East | 14.0 | 992 | 1.23 | B |
|  |  | West | 10.0 | 721 | 1.39 | B |
|  | East 72nd Street between York Avenue and First Avenue | South | 13.0 | 796 | 1.15 | B |
|  |  | North | 13.0 | 557 | 0.89 | B |
| PM Peak Period |  |  |  |  |  |  |
| 2 | York Avenue between East 74th Street and East 75th Street | East | 10.0 | 952 | 1.82 | B |
|  | East 74th Street between York Avenue and the FDR | South | 6.0 | 2256 | 7.83 | D |
|  | York Avenue between East 73rd Street and East 74th Street | West | 9.0 | 894 | 1.77 | B |
|  | East 74th Street between York Avenue and First Avenue | South | 4.0 | 503 | 2.41 | B |
| 3 | York Avenue between East 73rd Street and East 74th Street | East | 9.0 | 1579 | 2.18 | B |
|  | East 73rd Street between York Avenue and the FDR | North | 2.0 | 725 | 7.55 | D |
|  | York Avenue between East 73rd Street and East 72nd Street | East | 9.0 | 1225 | 2.40 | B |
|  |  | West | 10.0 | 937 | 1.95 | B |
| 4 | York Avenue between East 72nd Street and East 71st Street | East | 14.0 | 831 | 1.22 | B |
|  |  | West | 10.0 | 741 | 1.31 | B |
|  | East 72nd Street between York Avenue and First Avenue | South | 13.0 | 742 | 1.14 | B |
|  |  | North | 13.0 | 672 | 1.06 | B |

| Table 9-54 2019 Build Condition Corner Analysis |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intersection No. | Location | Corner | AM Peak Period |  | Midday Peak Period |  | PM Peak Period |  |
|  |  |  | SFP | LOS | SFP | LOS | SFP | LOS |
| 1 | York Avenue and East 75th Street | Northeast | 76.4 | A | 170.6 | A | 107.5 | A |
|  |  | Southwest | 578.7 | A | 644.1 | A | 553.7 | A |
|  |  | Southeast | 83.7 | A | 133.5 | A | 103.3 | A |
| 2 | York Avenue and East 74th Street | Northwest | 411.2 | A | 507.4 | A | 494.6 | A |
|  |  | Northeast | 101.9 | A | 135.3 | A | 111.8 | A |
|  |  | Southwest | 100.4 | A | 97.5 | A | 101.9 | A |
|  |  | Southeast | 58.4 | B | 49.4 | B | 45.9 | B |
| 3 | York Avenue and East 73rd Street | Northwest | 86.8 | A | 100.7 | A | 83.5 | A |
|  |  | Northeast | 53.7 | B | 48.9 | B | 45.6 | B |
|  |  | Southwest | 141.4 | A | 148.2 | A | 119.7 | A |
|  |  | Southeast | 57.2 | B | 50.2 | B | 53.2 | B |
| 4 | York Avenue and East 72nd Street | Northwest | 132.8 | A | 147.2 | A | 129.8 | A |
|  |  | Northeast | 251.0 | A | 233.6 | A | 243.0 | A |
|  |  | Southeast | 348.5 | A | 304.9 | A | 351.7 | A |
| 5 | York Avenue and East 71st Street | Northwest | 150.4 | A | 175.5 | A | 194.5 | A |
|  |  | Northeast | 140.2 | A | 135.6 | A | 130.8 | A |
|  |  | Southwest | 170.8 | A | 182.3 | A | 216.0 | A |
|  |  | Southeast | 157.8 | A | 158.5 | A | 154.3 | A |
| 6 | York Avenue and East 68th Street | Northwest | 415.5 | A | 510.5 | A | 459.9 | A |
| 7 | First Avenue and East 72nd Street | Northwest | 488.6 | A | 567.2 | A | 527.3 | A |
|  |  | Northeast | 875.2 | A | 601.8 | A | 592.7 | A |
|  |  | Southwest | 615.8 | A | 666.1 | A | 707.5 | A |
|  |  | Southeast | 548.8 | A | 510.5 | A | 472.2 | A |
| Note: SFP = square feet per pedestrian |  |  |  |  |  |  |  |  |

| Table 9-55 2019 Build Condition Crosswalk Analysis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intersection No. | Location | Crosswalk | Street Width (feet) | Cross-walk Width (feet) | Conditions with conflicting vehicles |  |  |  |  |  |  |  |  |
|  |  |  |  |  | AM |  |  | Midday |  |  | PM |  |  |
|  |  |  |  |  | 2-way Volume | SFP | LOS | 2-way Volume | SFP | LOS | 2-way Volume | SFP | LOS |
| 1 | York Avenue and East 75th Street | East | 34.0 | 20.0 | 1203 | 58.0 | B | 667 | 108.0 | A | 835 | 85.4 | A |
| 2 | York Avenue and East 74th Street | East | 34.0 | 18.0 | 1210 | 48.5 | B | 976 | 62.9 | A | 1059 | 52.8 | B |
|  |  | South | 60.0 | 12.0 | 666 | 28.3 | C | 792 | 24.0 | D | 825 | 22.4 | D |
| 3 | York Avenue and East 73rd Street | North | 60.0 | 12.0 | 386 | 53.3 | B | 387 | 53.9 | B | 503 | 45.3 | B |
|  |  | East | 40.0 | 17.0 | 1502 | 41.7 | B | 1519 | 44.0 | B | 1387 | 40.4 | B |
|  |  | West | 30.0 | 16.0 | 891 | 90.6 | A | 768 | 110.2 | A | 932 | 84.7 | A |
| 4 | York Avenue and East 72nd Street | North | 60.0 | 15.0 | 459 | 54.0 | B | 527 | 56.4 | B | 526 | 48.7 | B |
|  |  | East | 40.0 | 20.0 | 970 | 60.2 | A | 1208 | 53.5 | B | 897 | 66.2 | A |
|  |  | West | 55.0 | 17.0 | 933 | 59.7 | B | 781 | 70.6 | A | 852 | 64.9 | A |
| 5 | York Avenue and East 71st Street | East | 34.0 | 19.0 | 864 | 80.2 | A | 922 | 85.7 | A | 854 | 75.4 | A |
|  |  | West | 34.0 | 18.0 | 839 | 75.5 | A | 759 | 85.3 | A | 711 | 94.7 | A |
| Note: SFP = square feet per pedestrian |  |  |  |  |  |  |  |  |  |  |  |  |  |



A portion of the MSK and CUNY buildings’ mechanical spaces were subtracted from the buildings’ total gross square footages for trip generation purposes.